Investigating the Impacts of Plug-in Hybrid Electric Vehicles on Distribution Networks with Real Time Pricing

Alireza Saffarian, Seyyed Ehsan Bagheri, and Mahmood Joorabian

Abstract—Despite the aforementioned advantages of PHEVs, the connection of PHEVs to the power network because of stochastic charging performance of PHEVs new challenges will bring for distribution grid such as feeder overloading, and increment of loss. In order to handling the uncertainties related to PHEV charging performances in variant charging frameworks Monte Carlo simulation is used. In this paper, in order to diminish the negative impact of PHEV charging on performance of distribution network the strategy of demand response (DR) is offered. In the offered DR methodology, all consumers can determine and control their loads. In fact, in this methodology the customer can shift controllable load from peak hours with higher prices to off-peak periods with lower prices. Also, the offered DR methodology can fulfill the function of reducing peak with regard to variant PHEV penetration while preserving consumer comfort levels. For performing DR programs, a real time pricing (RTP) model is used to investigate the impacts of PHEVs charging on distribution network operations considering a 33-node test feeder with 11.3%, 35%, and 45% PHEV penetrations in residential loads. Obtained results verify the efficacy of proposed DR program (i.e. RTP) to ameliorate the system Performance considering PHEV charging loads.

Index Terms—Plug-in hybrid electric vehicles (PHEVs), Demand Response, Distribution Losses, Peak to Average Ratio (PAR), Real Time Pricing (RTP).

Incentive of demand response programs in period.

Customer’s income at nominal value of demand for the period.

Capacity of PHEV battery (kW).

Power demand of  hour before.
implementing demand response programs.

Power demand of  hour after.
implementing demand response programs.

Self and cross elasticity of demand.

Actual energy that is drawn from utility grid.

Actual current of the  branch.

Number of branches.

Preliminary electricity price of theperiod.

Active powers injected at the bus.

Secondary electricity price of the  period.

Active power flowing over the network between the buses and

Maximum power transmitted between the buses and

Power for the  period, which should be transformed from the grid.

Power required for charging PHEV for the  period.

Total incentive of customers’ for participating in demand response programs in period.

Reactive powers injected at the 										bus.

Resistance of the  branch.

Customer’s benefit of DRPs in an hour.

Time interval (hours).

Magnitude and the angle of the voltage at the bus (V).

Minimum/ maximum allowable voltage value at bus (V).

Daily miles driven by the PHEV (miles).

Magnitude of the branch admittance between  andbuses.

Daily miles	driven with PHEVs in the electricity mode (miles).

Angle of the voltage  andbuses.

Participation level of customers in demand response programs.

Magnitude and the angle of the branch            admittance between theandbuses.

Mean and Standard deviation of lognormal distribution.

The efficiency of PHEVs charger.

# Introduction

## Motivation and Approach

I

N the near future, because of environmental and energy security benefits of alternative vehicles such as plug-in hybrid electric vehicles (PHEV) against conventional gasoline vehicles, the number of PHEVs will likely grow significantly in the world. PHEV can play an important role in controlling pollutions and related global warming on the earth, and they can be some prosperous solutions in tackling these new emerged problems [1]. PHEVs are provided with quite large batteries and are joined into the power system. These devices must be charged during the night for the convenience of their owners who use them with a fully-charged battery during the day. These vehicles have need of an immense amount of energy to be charged. These requirements of electrical energy have a negative impact upon the distribution network [2]. Despite the aforementioned benefits of PHEVs, the connection large number of PHEVs to the existing power grid owing to stochastic charging performance of PHEVs, new challenges will bring for distribution network such as overloading of feeders and transformers, decreasing the efficacy and reliability, and increment of loss of electrical grid [3]. On this basis, smart charging approaches have been used to help the distribution network to satisfy PHEVs penetration, and provide the service with unaffected peak demand. Smart charging approach is an energy managing instrument that allows customers to regulate the loads as well as concerning the consumer’s own choosing and defend their confidentiality. On this basis, participation in demand response program is one of the most significant methodologies which can affect the penetration of PHEVs in electrical grid. Also, demand response can decline the negative impacts of PHEV charging on electrical grid and ameliorate the reliability and efficiency of power systems with shifting the responsible load from peak hours with higher prices to off-peak periods with lower prices.

## Literature Review

In [2], evaluated the impacts of PHEVs on power losses and voltage deviations in the power system. In [3], large-scale distribution planning model is presented to evaluate the impacts of PHEVs on investment cost and reliability of distribution grid. Reference [4], proposed the Monte Carlo (MC) method for studying the uncontrolled charging of PHEVs by assuming in a way that each vehicle starts charging with a battery SOC that is uniformly distributed between 20% and 100% of its capacity. In [5]the impacts of PHEVs on distribution reliability are investigated for many variant charging scenarios. The total impacts of wind turbine and PHEVs on distribution systems were assessed in [6]. In [7], in order to minimize the impact of PHEVs on network losses and, voltage deviation vehicle-to-grid mode of PHEVs have been considered. Reference[8], evaluated the impacts of uncontrolled and controlled PHEV charging on the network. Reference [9], presented the impacts of a variety of penetration levels of these devices on feeder losses, and also it considered last trip end time as charging start time. In addition, in [10], premised vehicle's batteries to be discharged completely when it arrives home. But, this amount of charge must be determined considering daily miles driven or other operation modes of PHEV which is very important factor. Therefore DR programs, which hasn’t considered for improving loss and peak load in previous researches yet. Also, in this paper, the uncertainty of related to PHEV charging behavior is considered based on Monte Carlo simulation (MCS)  technique [11] to handle uncertainties.

  - Contributions

To the best of authors’ knowledge, with respect to the previous works in the area, the contributions of this study can be summarized as follows:

- Investigating the impacts of PHEVs enabled charging rate on the peak demand and shaping the demand profile.

- Stochastic model of PHEVs’ load demand with Mont Carlo simulation under variant penetration levels and charging methods.

- Investigating the impacts of DR on major attributes of residential distribution network operation.

- Offering a distributed DR technique to manage the charging tasks of PHEVs and shape the daily demand profile and, in particular, minimize the peak demand of the system and loss.

- To propose a relevant DR strategy including Real Time Pricing (RTP) to be fulfilled for the load management in a residential distribution network aimed at loss reduction and flatten load profile as well as obtaining optimal charging of PHEVs.

## Paper Organization

The rest of the paper is classified as follows: Section II describes the objective function and constraints, Section III presents the characteristic of PHEVs and assumption, Section IV provides the demand response, Section V illustrates simulation results, and finally Section VI concludes the paper.

# Mathematical model for system operation

In order to solve nonlinear relationship between the bus voltage and angles, a power flow analysis is implemented to determine power losses.

- Objective function

The main objective function can be formulated as follows:

(1).

|  | (2) |
| --- | --- |

- Constraints

Demand supply balance for both active and reactive power is depicted by load flow equations as follows:

|  | (3) |
| --- | --- |

- Bus voltage limits

The voltage magnitude at each bus, are constrained as follows:

|  | (4) |
| --- | --- |

- Distribution line limits

The substation capacity limit delineates the maximum and minimum power withdrawal capacity over the substation transformer :

(5)

# Characteristics OF PHEVS and Assumptions

In recent years, significant attempts have been intensely focused on the probing of the impacts of PHEV on distribution systems. These devices must be charged during the night for the convenience of their owners who use them with a fully-charged battery during the day. The characteristics of the PHEV are an important part of studying the effects of PHEV on distribution systems, and have two major categories: The first is based on the car manufacturer data for some abilities of the device such as the capacity of battery. The PHEV's capacity depends on the type of vehicle's battery and the driving range. It is worthwhile to note that, charging level affects the charging time, for example, the lower charging level extend the charging time of a PHEV. The second category depends on owner's way of travelling and charging such as the duration of charging time. In order to achieve an approximate calculation of PHEV charging profiles and assesses the anticipated impacts of PHEVs on distribution networks it should be estimated the variant factors such as charging strategy, number of PHEVs being charged, charger type, battery state-of-charge (SOC) and capacity, charging start time, and charging duration which are used to evaluate the impacts charging behavior of PHEVs on the demand peaks of the system. PHEVs are provided with quite large batteries and are joined into the power system for charging at home, the battery of PHEVs start getting fully-charged. These devices must be charged during the night for the convenience of their owners who use them with a fully-charged battery during the day. As the PHEV batteries are varied from one company to another, and most vehicles have their own charging profiles, it is very crucial to analyze and consider variant kinds of vehicles in the study. This amount of charge must be determined considering daily miles driven or other operation modes of PHEV which is very important factor. On this basis, daily miles driven by PHEVs are one of the significant factors to demarcate the energy which is required for charging PHEVs when arrived home after last daily trip. Also, based on NHTS, the probability density function (pdf) of daily distance driven by PHEVs has been derived by a lognormal distribution. Therefore, the pdf of the daily miles driven with PHEVs based on [12] can be expressed as follows:

(6)

The quantity of energy reminding in PHEVs after last daily trip when the PHEVs come back to home which is quantified by the factor called state of charge (SOC). The amount of the SOC of the PHEV battery when the PHEV arrives home depends on miles driven by the PHEVs. State of charge is the proportion of reminded charge of the PHEV battery after the last trip arrival time. It should be noted, it is premised when owner of PHEVs arrives at home, vehicle's batteries to be discharged completely. On this basis, the SOC of each PHEV can be formulated as [2]:

(7)

The next step is to find Energy required for charging PHEVs’ battery from the SOC value assessed previously. To determine the total energy required for charging PHEVs’ battery, two factors play significant roles: vehicle type and SOC. The actual energy required for charging PHEVs’ battery after the last trip arrival time to full-fledged charge the battery is drawn from utility grid is expressed mathematically by:

(8)

The planner of the system should perceive the amount of power which is in prospect to be delivered.

# Demand Response

DRPs was allotted to the United States by a strategy of International Energy Agency (IEA) demand side management program [13]. DRPs have two fundamental types which include TBR programs and IBPs. These types of DRPs are composed of a number of programs including TOU, RTP, and critical peak pricing. Details of demand response programs include TBR programs and IBPs are presented in [14].

## A. Responsive Load Economic Model

To assess the impact of customers’ involvement in DRPs on load profile features, expansion of responsive load economic models can be essential. A system in which customers can regulate their demand up/down based on the spot price is developed in [15].

This model can be used to program generations in a pool based electricity market [16]. In this paper, an economic model of responsive load is advanced to take into consideration flexible amount of encouragement for customers with regard to responding to load reduction.

(i) Price Elasticity of Demand: Elasticity is the demand, (i.e.,  sensitivity in regard to the price, (i.e.,  that can be written as follows [17]:

(9)

In relation to (9), the price elasticity of the period in competition with  period can be expressed as [17]:

(10)

(ii) Modeling of Single Period Elastic Loads: As soon as the DRPs are applied, the customer adjusts his demand from  to  as follows:

(11)

Assume that ISO pays  $ to the customer at period as encouragement to decrease the level of the load curve. Accordingly, the entire incentive, , for contributing in incentive-based programs will be as following equation:

(12)

The customer’s profit, , for the period can be as:

(13)

The most commonly used structural forms for benefit function, (i.e., include quadratic, potential, exponential, and logarithmic functions where a second-order Taylor series expansion of the aforementioned nonlinear functions yield the quadratic benefit function [15]. Referring to [15], the most often used customers’ benefit function is the quadratic form as follows:

(14)

One set of related assumptions required are describe as follows:

is the deterministic function of time, is the random procedure, is the deterministic function of time. The graph of  resembles to an open down parabola quadratic function. Therefore, to get the best out of the customer’s profit,  ought to be equivalent to zero as follows:

(15)

(16)

By differentiating (14) with regard to and replacing the outcome in (16) yields:

(17)

Fig.1. The electricity prices (RTP) for implementing DRPs [14]

TABLE I

Self and cross elasticity of DRPs [14]

|  | valley | off-peak | peak |
| --- | --- | --- | --- |
| valley | -0.1 | 0.016 | 0.012 |
| off-peak | 0.016 | -0.1 | 0.01 |
| peak | 0.012 | -0.01 | -0.1 |

Hence, the customer’s consumption can be formulized as:

(18)

(iii)	Modeling of Multi-period Elastic Loads: The fundamental hypothesis is that the elasticity is unchanging as follows [30]:

(19)

By considering the linear relationship between prices and demands, the customer’s consumption can be expressed as:

(20)

(iv) Load Economic Model: By aggregating (18) and (20) and making an allowance for the coefficient “” as the participation level of customers in DRPs, the responsive load economic model can be as follows:

(21)

In (21),  is the secondary electricity tariff which is different from the real price, and should be informed to the DR contributors in advance of the day of applying DRPs. Electrical energy power provided by dedicated units and demand response resources fulfill the load demand as it can be seen as follows:

(22) In this equation,  is the power required for charging PHEV for the  period, which should be moved from the network to the battery of PHEV for fully charging,  is the power for the  period, which should be transformed from the grid for charging PHEVs and responsible and non-responsible load in  hour.

# Case Study

In order to evaluate the offered methodology, the 33-node test feeder is chosen which details of data for this grid is given in [18].  The average household load profile is premised based on [2]. This figure demonstrate load profile of the home appliance that covers 24 hours which is classified into three variant periods namely valley period (00:00 A.M.–9:00 A.M.), off-peak period (9:00 A.M.–16:00 P.M.), and peak period (16:00 P.M.–24:00 P.M.). Load flow calculations of the network are accomplished based on the backward–forward sweep method during 24 hours for variant PHEV penetration levels, in order to outline the feeder power loss and total load in variant load points of the network. The uncertainty related daily driven distance with PHEVs is represented by truncated lognormal distribution functions in which the average and standard deviation of PHEV daily driven distance is premised to be 33 and 20.4 miles based on NHTS data [12], respectively. The type of these vehicles is defined randomly based on the percentage given in [19]. Based on [19], the rate of charging level for charging of each PHEV 7.68 kW per hours is premised. Also, It is premised, the PHEVs for completely charging will plug into the network at peak period. According to [14], hourly centricity prices of energy (i.e., ) for accomplishing voluntary RTP program is demonstrated in Fig. 1. Table I presents Self ) i.e., ) and cross elasticity ) i.e., ) of DRPs. The potential of accomplishing DRPs, (i.e.,  is premised to be 0.03. In order to customers participate in the DRPs; the total signed contracts are equal to 3% of the total load. In order to investigation the effectiveness of the proposed methodology, two variant scenarios are considered: 1) uncontrolled PHEV charging; and 2) Smart charging strategy.

## PHEV charging without DR

This scenario is taken as a reference case. This case emphasizes on defining the impacts of variant PHEV penetration levels on the performance of distribution network, i.e., power losses, peak load and load factor, during 24 hours, where there is no DR program. Also, it is premised that the batteries of vehicles will plug into the grid at peak load. The results of PHEV charging impacts on the distribution network are validated in Figs. 2 to 5. In addition, variant attributes of the network with various PHEV penetration levels, i.e. 11.3%, 35%, and 45% are represented in Table II. Regarding to results, the increasing of the PHEVs number which depended on penetration of PHEVs causes an increase in the loss, peak load, and PAR, because of the synchronism of PHEVs charging with daily peak load. As can be seen from the Figs. 5 to 8 and Table V, with the growth rate of PHEVs penetration levels on distribution network, the loss and peak load increases in comparison with the reference case thanks to charging time of PHEVs coincides with the households peak load. These are substantial worries about the widespread use of PHEVs in distribution networks. However, this variation can likely be declined by demand response programs in the grid.

Fig. 2. PHEV charging impacts on the distribution network load at winter

Fig. 3. PHEV charging impacts on the distribution network load at summer

Fig4. PHEV charging impacts on the distribution network total loss at winter

Fig5. PHEV charging impacts on the distribution network total loss at summer

TABLE II

variant attributes of the network with various PHEV penetration levels for Case I

|  | Season | without PHEV | 11.3% PHEV |  | 35.0% PHEV |  | 45.0% PHEV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Peak load | summer | 2171.4 | 2263.6 |  | 2517 |  | 2624 |  |
|  | winter | 2375 | 2467.2 |  | 2720.6 |  | 2828 |  |
| PAR | summer | 1.55 | 1.61 |  | 1.76 |  | 1.82 |  |
|  | winter | 1.41 | 1.46 |  | 1.58 |  | 1.64 |  |
| Loss | summer | 232.22 | Mean | std | Mean | std | Mean | std |
|  |  |  | 235 | 1.17 | 235 | 1.17 | 235 | 1.17 |
|  | winter | 283.2 | 286 | 1.33 | 286 | 1.33 | 286 | 1.33 |

TABLE III

variant attributes of the network with various PHEV penetration levels for Case II

|  | Season | without PHEV | 11.3% PHEV |  | 35.0% PHEV |  | 45.0% PHEV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Peak load | summer | 2115 | 2204.7 |  | 2451.6 |  | 2556.3 |  |
|  | winter | 2313.3 | 2403 |  | 2649.9 |  | 2754.6 |  |
| PAR | summer | 1.5306 | 1.5861 |  | 1.7349 |  | 1.7954 |  |
|  | winter | 1.386 | 1.4327 |  | 1.5585 |  | 1.61 |  |
| Loss | summer | 223.48 | Mean | std | Mean | std | Mean | std |
|  |  |  | 220 | 1.06 | 254 | 1.4 | 267 | 1.08 |
|  | winter | 272 | 268 | 1.22 | 306 | 1.52 | 320 | 1.25 |

## PHEV charging with RTP

This scenario is devoted to the evaluation of the impacts of PHEV charging with variant penetration levels on distribution network performance, i.e., power losses, PAR, and peak load with RTP program which are appraised during 24 hours. The objective of this scenario is to flatten the total load profile of the network, loss reduction with RTP strategy. Table III demonstrates variant system attributes, i.e. power losses of the system, PAR, and peak load with RTP program. Regarding the results, the significant specifications of the total load profiles can improve with demand response programs. Similarly, the improvement in the PAR is more significant when the RTP approach is applied in case II.

According to the results, the application of the proposed management technique would lead to further loss reduction. This is owing to a portion of the network losses is shifted from peak hours with higher prices to off-peak periods with lower prices.

# Conclusion

In this paper, a stochastic framework is presented, and some significant specifications of distribution network are investigated such as load curve, and feeder losses. Also, a comprehensive study of PHEV impacts has been done on distribution grid. The peak load and increase of power losses are the most significant concerns in the high penetration of PHEVs in distribution system. To this end, a DR strategy has been used to help the distribution grid to convince PHEVs penetration, and provide the service with unaffected peak demand. DR pledges many benefits such as decline network cost and loss, increasing the efficiency of network, and increasing PHEVs penetration. Application of this strategy to a test case demonstrates the efficiency of the proposed approach. The offered problem was surveyed on the 33-bus IEEE test system under variant penetration levels of PHEVs in a day. The simulation results demonstrated that the amount of loss, peak load, and peak to average ratio (PAR) increase thanks to PHEVs penetration growth. Also, the simulation results depicted that the idea of RTP can decline the amount of network losses, peak load, and PAR by shifting controllable load from peak hours with higher prices to off-peak periods with lower prices.

# References

[1]	C. Chen and S. Duan, “Optimal Integration of Plug-In Hybrid Electric Vehicles in Microgrids,” IEEE Trans. Ind. Inform., vol. 10, no. 3, pp. 1917–1926, Aug. 2014.

[2]	S. Shafiee, M. Fotuhi-Firuzabad, and M. Rastegar, “Investigating the impacts of plug-in hybrid electric vehicles on power distribution systems,” Smart Grid IEEE Trans. On, vol. 4, no. 3, pp. 1351–1360, 2013.

[3]	A. Kavousi-Fard, M. A. Rostami, and T. Niknam, “Reliability-Oriented Reconfiguration of Vehicle-to-Grid Networks.”

[4]	J. Tan and L. Wang, “Integration of Plug-in Hybrid Electric Vehicles into Residential Distribution Grid Based on Two-Layer Intelligent Optimization,” IEEE Trans. Smart Grid, vol. 5, no. 4, pp. 1774–1784, Jul. 2014.

[5]	A. Kavousi-Fard, T. Niknam, and M. Fotuhi-Firuzabad, “Stochastic Reconfiguration and Optimal Coordination of V2G Plug-in Electric Vehicles Considering Correlated Wind Power Generation.”

[6]	W. Hu, C. Su, Z. Chen, and B. Bak-Jensen, “Optimal operation of plug-in electric vehicles in power systems with high wind power penetrations,” Sustain. Energy IEEE Trans. On, vol. 4, no. 3, pp. 577–585, 2013.

[7]	B. Khorramdel, H. Khorramdel, J. Aghaei, A. Heidari, and V. G. Agelidis, “Voltage Security Considerations in Optimal Operation of BEVs/PHEVs Integrated Microgrids,” IEEE Trans. Smart Grid, pp. 1–1, 2015.

[8]	E. Sortomme, M. M. Hindi, S. J. MacPherson, and S. S. Venkata, “Coordinated charging of plug-in hybrid electric vehicles to minimize distribution system losses,” Smart Grid IEEE Trans. On, vol. 2, no. 1, pp. 198–205, 2011.

[9]	K. Clement-Nyns, E. Haesen, and J. Driesen, “The impact of charging plug-in hybrid electric vehicles on a residential distribution grid,” Power Syst. IEEE Trans. On, vol. 25, no. 1, pp. 371–380, 2010.

[10]	M. S. ElNozahy, M. M. Salama, and others, “A comprehensive study of the impacts of PHEVs on residential distribution networks,” Sustain. Energy IEEE Trans. On, vol. 5, no. 1, pp. 332–342, 2014.

[11]	R. Torquato, Q. Shi, W. Xu, and W. Freitas, “A Monte Carlo Simulation Platform for Studying Low Voltage Residential Networks,” IEEE Trans. Smart Grid, vol. 5, no. 6, pp. 2766–2776, Nov. 2014.

[12]	“NHTS Home Page.” [Online]. Available: http://nhts.ornl.gov/. [Accessed: 06-Oct-2015].

[13]	“International Energy Agency.” [Online]. Available: http://www.iea.org/. [Accessed: 30-Nov-2015].

[14]	H. A. Aalami, M. P. Moghaddam, and G. R. Yousefi, “Modeling and prioritizing demand response programs in power markets,” Electr. Power Syst. Res., vol. 80, no. 4, pp. 426–435, 2010.

[15]	F. C. Schweppe, M. C. Caramanis, R. D. Tabors, and R. E. Bohn, Spot pricing of electricity. Springer Science & Business Media, 2013.

[16]	D. S. Kirschen, G. Strbac, P. Cumperayot, and D. de Paiva Mendes, “Factoring the elasticity of demand in electricity prices,” Power Syst. IEEE Trans. On, vol. 15, no. 2, pp. 612–617, 2000.

[17]	S. Stoft, “Power system economics,” J. Energy Lit., vol. 8, pp. 94–99, 2002.

[18]	M. Narimani, A. Vahed, R. Azizipanah-Abarghooee, and M. Javidsharifi, “Enhanced gravitational search algorithm for multi-objective distribution feeder reconfiguration considering reliability, loss and operational cost,” Gener. Transm. Distrib. IET, vol. 8, no. 1, pp. 55–69, 2014.

[19]	Z. Darabi and M. Ferdowsi, “Aggregated impact of plug-in hybrid electric vehicles on electricity demand profile,” Sustain. Energy IEEE Trans. On, vol. 2, no. 4, pp. 501–508, 2011.