Abstract—In this paper, a novel mixed design of low-swing self-timed regenerator (LS-STR)is presented. The new design reduces the energy-delay product (EDP) and eliminates a fabrication constraint, so is suitable for long global on-chip interconnects. The proposed scheme is simulated in CMOS 90-nm technology at 1.0 V supply, for signal transmission along a nearly 10 mm-length line. The simulation results for different wire widths show a reduction of the propagation delay up to39.1% for iso-power mode when compared with the delay of optimal repeater insertion case. Also, up to 23.2% power reduction has been achieved in the iso-delay mode. A key advantage of the proposed LS-STR is that it employs neither multiple-threshold process technology nor an extra and different power supply voltage, while the noise margin remains in an acceptable level.