Low-Swing Self-Timed Regenerators for High-Speed and Low-Power On-Chip Global Interconnects

Hossein Rezaei

Department of Electrical Engineering and Electronics Research Center, Iran University of Science and Technology, Tehran, Iran

Email: ho_rezaei@elec.iust.ac.ir

Soodeh Aghli Moghaddam

Department of Electrical Engineering and Electronics Research Center, Iran University of Science and Technology, Tehran, Iran

Email: saghli@iust.ac.ir

Abstract—In this paper, a novel mixed design of low-swing self-timed regenerator (LS-STR) is presented. The new design reduces the energy-delay product (EDP) and eliminates a fabrication constraint, so is suitable for long global on-chip interconnects. The proposed scheme is simulated in CMOS 90-nm technology at 1.0 V supply, for signal transmission along a nearly 10 mm-length line. The simulation results for different wire widths show a reduction of the propagation delay up to39.1% for iso-power mode when compared with the delay of optimal repeater insertion case. Also, up to 23.2% power reduction has been achieved in the iso-delay mode. A key advantage of the proposed LS-STR is that it employs neither multiple-threshold process technology nor an extra and different power supply voltage, while the noise margin remains in an acceptable level.

Keywords— repeater; regenerator; interconnect; low-swing signaling; propagation delay; VLSI;

# Introduction

As chip technology and supply voltage continues to scale down, the delay of global wires remains the same or even become worse ‎[1]. Furthermore, the requirement of high clock frequencies makes the on-chip global interconnects a major bottleneck in state-of-the-art system on chips (SOC’s) and limits the performance and power dissipation of CMOS circuits. So far, in order to reduce the dependence of overall delay to the line length, repeater insertion has been the commonly used method ‎[2].However, repeaters transfer data in only one direction and for a bidirectional bus a pair of wires with repeaters is needed. Besides, as a repeater inserted in series into the line its delay will be added to the line’s delay.

To address these issues, a number of schemes without employing the repeaters have been presented in literature [3-7]. Booster design ‎[3] has a structure where extra current is injected to the line when a transition is detected, but, it limits the speed improvement due to a stack of two transistors in the path. The scheme proposed in ‎[4] is a capacitive coupling accelerator, to reduce resistance-capacitance (RC) delay of the line, but the improvement compared with repeater insertion design is not significant. The other approaches such as modulated signaling ‎[5] and pulsed current-mode signaling ‎[6] achieves near speed-of-light latency, but because of very reduced voltage swing on the line, they are very sensitive to parameter variations and have very low noise margin. Also, the complexity of these designs makes them difficult to use in the industry. Another approach is the Low-Swing (LS) signaling ‎[7] that reduces the voltage swing of the signal on the line as well as the dissipated power on it. The reduced swing leads to the noise margin will be acceptable for only short wires. However, for long wires the voltage swing reduces more due to presence of the parasitic elements (parasitic resistances and capacitances) of the line and the scheme will be more susceptible to noise. Additionally, it needs to an extra and different power supply voltage to work.

Finally, regenerator scheme [8-9] is an active circuit that has been designed to compensate for the loss inherent in the on-chip narrow lines and achieve fast signal propagation. An advantage of such a repeater-less designs is that they are attached periodically aside the line and are suitable for point to point bidirectional signaling. Once, the scheme proposed in ‎[2]- called Self-Timed Regenerator (STR)- senses a transition, it automatically amplifies the signal, and then returns to the transition monitoring mode. This is because of its self-timed design. In this mode, unlike the booster design proposed in ‎[3] and the regenerator scheme in ‎[8], it does not already drive the line but is ready to detect the next transition and to amplifies that. So, STR will prevent a so-called “fight” that occurs between the non-self-timed designs (booster and regenerator) and the next transition in the early part of the transition. Hence, makes the STR behavior more efficient. Despite all these advantages in STR design, it uses multiple threshold process technology and makes it difficult to adopt in the industry.

The mentioned drawbacks motivate us to propose a novel mixed LS-STR which benefits both advantages of the LS signaling and STR scheme; Whereas, it employs neither multiple-threshold process technology nor an extra and different power supply voltage. So, adoption to the industry will be easy and the result is a good trade-off between noise margin, speed and power consumption.

The remainder of the paper is organized as follows. In section II operation of the LS-STR circuit will be described. Section III will present the simulation results and section IV concludes this work.

# LS-STR Design

- LS Driver Structure

Fig. 1 illustrates circuit structure for the mj-driver that proposed in ‎[7] and has been employed as the LS driver throughout this paper. Mj-driver is an Up-Down LS Driver (UDLD) that the output voltage symmetrically ranges between (VTN) and (VDD-VTP), where VTN and VTP are the NMOS and PMOS transistor threshold voltages, respectively.

The driver achieves symmetrical low-swing through the use of diode connected transistor pairs (P3, P4) and (N3, N4). The upper and lower half of the circuit in Fig. 1 are responsible for low to high and high to low transitions at the output, respectively. So, the driver provides high current driving capability through the use of multi-path technique ‎[9]. The feedback path through Inv5 leads to fast switching of the output during the transitions. In the following, we will describe the operation of the mj-driver circuit.

In a general viewpoint, the gates on the left side of the circuit are employed for controlling the transitions, while the transistors on the right side, have the role of bringing the voltage swing down and driving the output load. We will describe the transistors behavior. However, more details about the control circuit is presented in ‎[7].

When the output is at low level (out = low) N1, P3 and P4 are off while P1 is on. In this case, the output is driven low through the diode connected pair N3-N4. After a low to high transition in the Inv1 output (Dout), due to delay in the feedback loop (Inv5), N1 and consequently P3 becomes on

Figure 1: Conventional LS CMOS driver

and strongly pull the output node out to high, to charge up the output load. Eventually, the feedback loop turns the nodes of the circuit to either steady state values of high and low, turning N1 off, disabling it from driving the gate of P3. However, to maintain the output voltage at VDD-|VTP|, transistor P4 which became on when out was low will remain in this state, providing a diode connected configuration ‎[7].

Conversely, when the output is at high level, the operation of the circuit will remain the same and eventually, N3 remains on, providing a diode connected configuration. In this state the output is driven high through the diode connected pair P3-P4. After a high to low transition at the Inv1 output (Dout), due to delay in the feedback loop (Inv5), P2 and consequently N4 becomes on and strongly pull the output node out to low, to discharge the output load. Eventually, as previously noted, the feedback loop turns the nodes of the circuit to either steady state values of high and low, turning the P2 off, disabling it from driving the gate of N4. However, to maintain the output voltage at VTN, transistor N3 which became on when out was low, remain in this state, providing a diode connected configuration ‎[7].

Voltage waveforms of in and out nodes for mj-driver with a capacitive load of 1pF are shown in Fig. 2. The circuit is simulated in 90nm CMOS 1.0V technology.

- Conventional STR Structure

For a sufficiently wide wire, the resistance and in result, the attenuation of the signal is insignificant. However, when the wire becomes thinner, the resistance will be increased and the signal begins to attenuate, as it propagates through the wire. In this case, the signal may not be detected at the far end of the wire, since the signal swing is degraded. Furthermore, the rise (fall) time(s) will increase. In order to compensate for this signal degradation, STR design, enhances the transitions by supplying certain amount of current from the rail. Fig. 3 shows the conventional STR circuit on both sides of interconnect. The upper part is the pull-up circuitry responsible for accelerating the rising transition and the lower part is the pull-down circuitry responsible for the falling transition. These two parts are complementary to each other. When a low-to-high transition arrives, the pull-up part is triggered and the other one remains insensitive to the signal on the line. Similarly, in a case of a high-to-low transition,

Figure 2: Waveforms of in and out nodes for LS CMOS driver with a capacitive load of 1pF and 1.0 V supply voltage.

Figure 3: Conventional STR circuit.

the pull-down part is triggered and the pull-up part will remain insensitive. STR is a repeater less design and is suitable for point to point bidirectional signaling. However, it uses multiple threshold process technology and makes it difficult to adopt in the industry. For more details about the STR you can see the ‎[2].

- Motivates

In this paper, although LS driver produces the low-swing transitions at the near end of the line for a long wire, but LS-STR compensates the voltage drop on the line; this leads to the nearly constant noise margins on the front and end of the line. Moreover, since STR uses transistors with low-threshold voltage and LS-STR reduces the voltage swing, the noise margin of these two regenerator designs will be almost the same.

Fig. 4 shows the waveform of the out node of the mj-driver with a capacitive load of 1 pF (LS-Cap). This is an ideal mode of the driver output. Also, the waveform of the intermediate node of a 1 um wide interconnect (LS) is shown.

We modeled the interconnect line as a top global layer metal with shielding wires on either side. Also, we consider the line-length about one-tenth of a die (10mm) in CMOS 90-

Figure 4: Waveforms of out node for LS driver with a capacitive load of 1pF compared with the line middle point.

nm technology and it’s fixed throughout this paper. It is evident that the signal swing decreased as it propagates in the line and with decreasing the line width, this attenuation will be more significant. The waveform of the line with employing only one LS-STR along it (LS-STR), is shown on this figure, too. It is clear that the LS-STR compensates the voltage drop of the line by injecting additional current.  So, we achieve significant improvement in terms of noise margin. This compensation, also, results in reinforcement in the transmission line effect, and consequently faster transition time at the output of the line. We will see that the answer is almost close to the ideal mode.

- Working of the Circuit

Fig. 5 presents the circuit structure for LS-STR proposed in this paper. This circuit is proposed to sense and accelerate the transition for a constant time that determined by the odd-number of inverter chains (Inv1-Inv3 and Inv6-Inv8). The size and number of inverters can be optimized for different wires and constrains, and this provides the self-timing of such circuits. In this type of regenerators, a pulse will generate at nodes B and C which would turn on the output stage for a time equal to delay of the two inverter chains. So, additional current will be injected to interconnect from the rail and expedite the signal transition. The two transistors N1 and P4 are the firing transistors which will turn on according to the signal polarity. Transistors P1 and N3 is to keep initial conditions at nodes B and C and they are weak transistors.

Considering that the line is at low voltage steady state, transistor P1 will be on and nodes B and D is derived by VDD. In this state, the pull-up circuit is ready to detect low to high transition, and the lower circuit part is insensitive to any high to low transition. If node D pulls down by any noise, P3 and P4 charges node C and after passing through the inverter chain, P9 actively drives node D to VDD and then, the state holder chain (Inv4-Inv5) will maintain the state. For a line with high voltage steady state, node C and D will set to GND.

Figure 5: Proposed LS-STR Structure.

Assuming that the initial state is low state, the next transition will be a low to high transition. The pull-down circuit would be insensitive to the transition, since, transistor P3 is off. Transistor N1 will detect the rising edge quickly and pulls down node B to GND, results in P7 turns on. After that node E will drive to VDD, transistor N6and in result P6 turns on. So, the output stage enhances the signal transition and supplies additional current to the line. After some time, N5 shorts node D to the ground and changes state of the holder. Also, transistor P2 charges node B to VDD, so the output stage turns off. After some delay, N5 turns off, but the state of node B will maintain by the holder. Now, the circuit is ready to detect opposite transition.

An advantage of self-timed designs is that transistors (P2, N1, N2) and (P3, P4, N4) are never turned on in the same time, therefore, the ‘fight’ between transitions will eliminate, results in improvement in terms of performance. So, we don’t have any short circuit current in such circuits, resulting in power reduction. In addition, the line will accelerate by a single transistor between the line and power supply, so, the result is to high drive current capability.  LS-STR uses neither multiple threshold process technology nor an extra and different power supply. Therefore, adoption to the industry will be easy. Also, the noise margin will remain in an acceptable level.

# Simulation Results and Discussion

In this section, we will compare the LS-STR and repeater schemes, both in terms of delay and energy. The CMOS 90-nm technology at 1.0 V power supply results are obtained from EXPECTRE simulation for a wide range of the interconnect widths using industrial device models. We modeled interconnect as a top global layer metal with shielding wires on either side (Fig. 6). A distributed RLC pi model has been used and every 25 um of the wire is modeled as a lumped RLC circuit. RLC values have been extracted using Predictive Technology Model (PTM) ‎[10] for a 10mm long wire, and as previously noted, the line length is fixed throughout this paper.

- Repeater and LS-STR Design

Fig. 7 shows the overall scheme for repeater and LS-STR designs. To make the comparison fair, we use identical initial drivers both in the repeater and LS-STR schemes. In repeater scheme, the first repeater is placed after the initial driver. However, in LS-STR scheme, the mj-LS driver has been used after the initial driver to produce low swing pulses. In both schemes, we use a simple and identical inverter as a receiver in the far end of the line. In the case of LS-STR, the inverter

Figure 6: Structure of global interconnect.

Figure 7: Repeaters and LS-STR implementation scheme.

exhibits better than the other low-swing receivers in terms of both power consumption and delay improvement.

Adding STR will change the load of the line. But, the difference is not significant. For instance, the impedance for a 1-um wide unloaded wire is about 5% more than same loaded wire with LS-STRs. Repeaters and LS-STRs are placed regularly throughout the interconnect and the delay is calculated from the initial driver input to the last receiver output. To achieve the best performance, both the number and size of repeaters are swept, for all wire widths. Among all the combinations, optimal number and size of repeaters which result in the best case energy-delay product is chosen.

Similarly, at each wire width, both the number and sizing of the LS-STR is optimized and the best case, in terms of, energy-delay product is chosen. It is appropriate to express that for a given wire geometry, all repeaters and LS-STRs are sized identically.

The overall power and performance comparison is summarized in Table I.

- Power and Area

In order to power comparison, the amount of power which saved in the same delay by LS-STR is reported. For the same delay, power reduction up to 23.2% is achieved in the case of 0.3-um wire width, as shown in Table I. It is evident that for all wire widths, the LS-STR design power is less than that of the repeater design. This is first due to the fact that smaller number of LS-STRs has been used than that of repeaters in the same delay. Another reason is that there is no strongly conducting direct path from VDD to GND at any time interval, therefore, the short circuit current is minimized.

From the area point of view, we observed that the LS-STR need not be oversized to produce equivalent energy or delay with repeaters. Another point is that for the wire widths greater than 1-um, as the line width increases parasitic capacitance will dominate to the parasitic resistance, as the wire width increases. The area comparison for iso-delay case is reported in Table II.

- Performance

In order to performance comparison, the delay is measured in the same power consumption of both designs for every wire widths. The delay improvement is 34.8% for the same power consumption and 39.1% for the best case. The performance improvement achieved for different wire widths with the LS-STR design has been reported in Table I. The performance improvement has direct, but the non-linear relation with the

Table I: LS-STR Power and Performance Comparison.

| Wire width (um) | Opt. # of Repeaters | Power improvement % (Iso-delay) | Opt # of LS-STRs(Iso-delay) | Delay improvement % (Iso-power) | Opt # of LS-STRs (Iso-power) | Delay improvement% (Best case) | Opt # of LS-STRs (Best case) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| .3 | 9 | 23.2 | 4 | 31 | 6 | 34.4 | 8 |
| .45 | 7 | 16.2 | 3 | 27.6 | 5 | 29.2 | 6 |
| 1 | 6 | 20 | 1 | 34 | 2 | 36.7 | 4 |
| 4 | 2 | 18.6 | 2 | 34.8 | 4 | 39.1 | 5 |

Table II: Area Comparison in iso-delay.

| Wire width | Scheme | Total device widths (um) | Improvement (%) |
| --- | --- | --- | --- |
| .3 | Repeater | 687 | 35.9 |
|  | LS-STR | 440 |  |
| .45 | Repeater | 508 | 34.4 |
|  | LS-STR | 333 |  |
| 1 | Repeater | 259 | 36.3 |
|  | LS-STR | 165 |  |
| 4 | Repeater | 371 | 30.5 |
|  | LS-STR | 258 |  |

resistance and capacitance of the line. Fig. 8 shows that LS- STR dominants to repeater scheme, in terms of performance. We can see that for all wire widths, the LS-STR delay curve is in the bottom, than that of the repeater.

It should be noted that for a few number of LS-STRs the voltage swing of interconnect will be lower than that for larger number of them, results in lower noise margin. Therefore, the optimum point for the number of regenerators is where that the voltage swing is between VTN and VDD-VTP.

Figure 8: Delay Comparison with Different Number of LS-STRs and Repeaters for the Wire Width of .45 um.

# Conclusion

Combining two methods of LS signaling and regenerator employing, we have presented a novel mixed circuit technique to improve both delay and power consumption for data transmission along the global interconnects. We have obtained up to 23.2% power reduction (iso-delay mode) and up to 34.8% delay improvement (iso-power mode) with respect to the optimal repeater insertion case. Also, up to 36.3% improvement has been achieved from the area point of view.

##### References

J. S. Clarke, C. George, C. Jezewski, A. M. Caro, D. Michalak, AND J. Torres, “Process Technology Scaling in An Increasingly Interconnect Dominated World,” In VLSI Technology (VLSI-Technology): Digest of Technical Papers, IEEE Symposium on, pp. 1-2, jun. 2014.

P. Singh, J. S. Seo, D. Blaaw, and D. Sylvester.  “Self-timed regenerators for high-speed and low-power on-chip global interconnect,” Very Large Scale Integration (VLSI) Systems, IEEE Transactions on 16, no. 6, pp. 673-677, 2008.

A. Nalamalpu, S. Srinivasan, and W.P. Burleson, “Boosters for driving long onchip interconnects—Design issues, interconnect synthesis, and comparison with repeaters,” IEEE Trans. Comput.-Aided Des. Integr. Circuits Syst., vol. 21, no. 1, pp. 50–62, Jan. 2002.

H. Huang and S. Chen, “Interconnect accelerating techniques for sub-100-nm gigascale systems,” IEEE Trans. Very Large Scale Integr. (VLSI) Syst., vol. 12, no. 11, pp. 1192–1200, Nov. 2004.

R. T. Chang, N. Talwalkar, C.P. Yue, and S.S. Wong, “Near speed-of light signaling over on-chip electrical interconnects,” IEEE J. SolidState Circuits, vol. 38, no. 5, pp. 834–838, May 2003.

M. Dave, M. Jain, M. Shojaei Baghini, and D. Sharma. “A variation tolerant current-mode signaling scheme for on-chip interconnects,” Very Large Scale Integration (VLSI) Systems, IEEE Transactions on 21.2, pp. 342-353, 2013.

J. C. Montesdeoca, G. J. Montiel-Nelson, and S. Nooshabadi. “CMOS driver-receiver pair for low-swing signaling for low energy on-chip interconnects,” Very Large Scale Integration (VLSI) Systems, IEEE Transactions on 17.2, pp. 311-316, 2009.

S. W. Lee, J. S. Kim, J. D. Cho and B. S. Komg. “Novel area-efficient regenerator for driving long on-chip interconnects,” IEICE Electronics Express 5.9, pp. 338-343.

B. Kheradmand-Boroujeni, and M. Masoumi, “A New Large Capacitive-Load Driver Circuit for Low-Voltage CMOS VLSI,” Proc. Of the IEEE International Workshop on Biomedical Circuits and Systems, pp. S1.1-5-S1.1-8, Jun. 2004, Sigapore.

Y. Cao, T. Sato, M. Orshansky, D. Sylvester, and C. Hu, “New Paradigm of Predictive MOSFET and Interconnect Modeling for Early Circuit Simulation,” In Custom Integrated Circuits Conference (CICC) Proceedings of the IEEE 2000, pp. 201-204, 2000.