Robust Adaptive Fault Tolerant Controller Design for  Active Suspension System  in the presence of physical parametric uncertainties

Hadi Azmi

Electrical Engineering Department

Sahand University of Technology

Tabriz – Sahand new Town, Iran

h_azmi@sut.ac.ir

Reza Babazadeh

Electrical Engineering Department

K.N.Toosi University of Technology

Tehran, Iran

Riza.Babazadeh@gmail.com

Abstract — This paper presents a robust adaptive state dependent fault tolerant control scheme for the class of nonlinear Lipschitz systems in the presence of bounded matched or unmatched disturbances and actuator loss of effectiveness. A constructive algorithm based on Linear Matrix Inequalities (LMIs) with creatively using Lyapunov stability theory is developed for on-line tuning of state-feedback gains to stabilize the closed-loop control system asymptotically, to compensate actuator faults, and to attenuate disturbance effects. The resulting control system has a simpler structure as compared with most existing recent methods. The merits of the proposed control scheme have been verified by the simulation on an active suspension system in the presence of physical parametric uncertainties.

Keywords — Fault Tolerant Control (FTC); Active Suspension System; Nonlinear Lipschitz Systems; Robust Adaptive Controller; Linear Matrix Inequality (LMI)

# Introduction

With the development of the automotive industry, and with the growing complexity of modern control systems and technology advances, research on vehicle suspension system design from both theoretical and practical aspects has received great attention over the past decades.[1-4] Because of the suspension system is one of important components in vehicles due to its potential to improve the ride comfort, vehicle maneuverability and safety of passengers [1].

Suspension systems can be broadly classified into three categories namely passive, semi-active and active suspensions. Fig. 1 shows suspension system classifications. A passive system comprises a damper or shock absorbers and a spring having fixed characteristics. The characteristics of the springs and shock-absorber are fixed and designed for presumed operational condition then cannot be adapted to any momentary operational condition of the vehicle. Thus the vehicle’s performance is very limited and any improvements can only be made by the optimization of springs’ and shock absorbers’ characteristics. [8] An active suspension, in addition to the already described components, is also comprised of an actuator, sensors, and a control programming unit (CPU). The actuator is incorporated that provides force without being influenced by relative velocity. A semi-active system is based on passive and active systems and has the ability to modulate the damping coefficient of damper but the direction of damping force is dependent on the relative velocity across the sprung and unsprung masses. [7]

The first design procedures of the vehicle active suspension system can be dated back to the beginning of eighties. [5] The investigators mainly used the well-known "sky-hook' principle for the design of active suspension system. The back draws of the first control strategies became soon clear. Because of the following problems the quarter-car model which had been used in the design was much more complicated than thought. First it cannot be considered as a linear system, there are several non-linearities: wheel-hoop, bump stop etc [5]. The effect of the wheel-hoop phenomenon, non-linear spring and time delay of the actuator on the behavior of the active suspension system design by means of LQG/LQR strategy is examined in [6] and demonstrated the disadvantages of the LQR control problem very well.

In the early studies of the control of wheel-suspension system, in [9] a class of proportional-integral sliding mode control for linear fixed suspension systems in the presence of only mismatched disturbances has been presented. In [10], [11] a multi-objective control method of linear vehicle active suspension systems are presented. The main objectives of these papers are to use a robust state-feedback controller to achieve multiple performance objectives for different controlled output signals. In [12] robust H infinity controller with using Linear Matrix Inequalities (LMI) optimization for the Linear active vehicle suspension under non-stationary running has been presented. In [13], [14] considered uncertain nonlinear active suspension system. The main objective of these papers is to design a novel controller for nonlinear uncertain suspension system with a nonlinear spring and piece-wise damper dynamics. Simulation results are provided to illustrate the effectiveness of the proposed methods.

The main contribution of this paper can be summarized as follows:

- A novel robust adaptive state dependent fault tolerant control strategy for the class of nonlinear systems in the presence of actuator faults and bounded matched or unmatched disturbances is presented.

- A constructive algorithm based on Linear Matrix Inequalities (LMIs) by creative using Lyapunov stability theory is developed for on-line tuning of state-feedback gains to stabilize the closed-loop control system asymptotically, to compensate loss of actuator effectiveness, and  to attenuate disturbance effects. The resulting control system has simpler structure as compared with most existing methods.

- The proposed controller design algorithm applied to active suspension system in the presence of actuator faults  and physical parametric uncertainties . In other words when structured uncertainty is present in the system (the tyre stiffness is not perfectly known).

Fig.1 . Quarter vehicle models; a) passive, b) active, and c) semi-active suspension systems

This paper is organized as follows. Active suspension system modeling in the presence of physical parametric uncertainties and disturbances in section 2. Problem formulation for control system design and the direct robust adaptive state dependent controller is developed In section 3. In section 4 a constructive algorithm based on LMI is presented for robust adaptive control system design. In Section 5, by simulation on an active suspension system which is modeled in section two, the merits of the proposed approach is verified.

We first introduce our notations R stands for the set of real numbers. For a given matrix A , AT denote its transpose. I denote unity matrix with appropriate dimension. The following lemmas are used in the paper.

Lemma 1 [15] : (Rayleigh Inequality) Consider a non-singular symmetric matrix  Q ∈ R n×n and let λmin and λmax be respectively the minimum and maximum eigenvalues of  Q. Under these condition, for any x∈ Rn,

λminQx2≤xTQ x ≤ λmaxQx2

Lemma 2 [16]: Let D, S and F be real matrices of appropriate dimensions and F satisfy FT F ≤I. Then for any scalar ε>0 and vectors x,y ∈ Rn ,we have

2xTDFSy ≤ε-1xTDDTx+εyTSTSy

# Active suspension system modeling

In the modeling of active suspension system which is shown in Fig. 2, the spring stiffness is assumed to be not perfectly known, which is quite realistic since it is a function of the internal tyre pressure, vehicle speed, temperature, etc. [5] Assume that the tyre stiffness is varying arbitrary in the following range:

kt∈[kt0- δkt  , kt0+ δ kt]

where the kt0 is the nominal value of kt and the δ kt is the range of variation. The effect of this parameter on the acceleration of the vehicle body has been considered in [5]. In the paper only the parametric uncertainty is considered, although several non-linearities are present in the system as it can be seen in [13], [14]. The aim of this study is to demonstrate the applicability of the robust adaptive design methodology in case of the active suspension system based on simple quarter-car model.

Fig. 2. The quarter car model

The sprung mass ms represents the car chassis, while the unsprung mass mus represents the wheel assembly. The spring, ks, and damper, bs, represent a passive spring and shock absorber that are placed between the car body and the wheel assembly, while the spring kt serves to model the compressibility of the pneumatic tire. The variables xs, xus, and w are the car body travel, the wheel travel, and the road disturbance, respectively. The force u (t), applied between the sprung and unsprung masses, is controlled by feedback and represents the active component of the suspension system. The dynamics of the actuator are ignored in this modeling, and it’s assumed that the control signal is the force u (t) but is affected by loss of effectiveness. Defining uF tis faulty control force because of occurring loss of effectiveness in actuators, x1= xs ,x2= xs  , x3= xus and x3= xus  the following is the state-space description of the quarter car dynamics. Let us consider the state-matrix of the linear uncertain suspension system with loss of actuator effectiveness by separating the nominal part from the uncertain one:

xt= A0 xt+ L ∆ N xt+B1 w t+B2 uF t

Where

A0 = 01-ksms-bsms  0     0ksms      bsms00ksmusbsmus01ks-kt0mus-bsmus      ,  B1 =000ktmus

L=000δktmus  ,   NT=00-δktmus0  , ∆=q1q1<1  ,  B2 =01ms1mus0

# Problem Formulation

In this section, we first present a well-known model for actuator loss of effectiveness faults. Assume uiF(t) is the output signal of i-th actuator that is faulty and ui (t) is the input signal of i -th actuator. Then, we donate a general actuator fault model as

uiFt=ρi ui t             i = 1, 2, …, m                                   (1)

where ρi is the unknown time-varying actuator efficiency factor, and ρi and ρi represent the known lower and upper bounds of ρi, respectively. From (1) it is easy to obtain

uFt=ρut                                                                                 2

Where           ρ= diagiρi                  i = 1, 2, …, m

Then, the set of operators with the above structure is donated by

∆ρ ={ ρ:ρ = diagiρi,  ρi ∈[ ρi  ρi ] }                       (3)

## Nonlinear System Modeling

Consider the following continuous-time nonlinear Lipschitz system,

xt=Axt+B2uF t+B1wt+fx , t                            (4)

where xt ∈ Rn is the state vector, uF t ∈ Rm  is the control input vector which is affected by loss of effectiveness fault, wt ∈ Rn is a continuous vector function which represents the bounded external disturbances and f(x,t) is a nonlinear function. This function can also be considered as a nonlinear perturbation affecting the system (see [15] for more details). A , B2, and B1 are known real constant matrices with appropriate dimensions.

Assumption1. The nonlinear system (4) is locally Lipschitz in a region Ω containing the origin, L>0 that is ∀x1 ,∀ x2∈Ω Rn

fx1,t-fx2,t≤Lx1-x2                                       (5)

Remark 1. It is worth mentioning that, the Lipschitz systems constitute a very important class. Many non-linearities are locally Lipschitz. A class of nonlinear systems that has received much attention in the literature is the class of Lipschitz nonlinear systems. Examples include trigonometric non-linearities occurring in robotics ,non-linearities that are square or cubic in nature etc.

## Direct Robust Adaptive State Feedback

In this sub section, the robust adaptive state feedback controller is defined and the adaptive law is developed to update the controller parameters. A method for designing direct adaptive controllers for nonlinear system to guarantee closed-loop system stability is presented in Theorem 1. Consider the following control Law for the system (4)

ut= K1  xt+ K2  (x(t))                                                    (6)

where K1 ∈ R m×n  is a constant matrix gain and K2 ∈ R m is the time-varying state dependent matrix gain that should be designed later. From (6), (4), and (2) the closed-loop system is described by

xt=A+B2ρK1xt+B2ρK2t+B1wt+fx , t

7

The following adaptive law is used to design robust adaptive state feedback controller.

K2t= -(xTPB2)T β k3 (t)xTPB2 α                                 (8)

where α, β are suitable positive constants which satisfy the inequality  0≤α≤ β  and k3(t) is updated by

dk3tdt= γ xTPB2                                                            (9)

where γ  is any positive constant. To ensure the achievement such as closed-loop stability and disturbance attenuation, the following assumptions in FTC system design are also assumed to be valid.

Assumption 2. For ∀ρ ∈ ∆ρ and for any mode of faults, the couple of (A, B2ρ) have to be controllable. It is worth mentioning that this assumption is standard and denotes the internal stability of system and completely compensating the disturbances effect.

Assumption3. External disturbance are piece-wise continuous bounded functions, that is, there exist known positive constant w such that                         || w(t) || ≤  w

Theorem 1. Under assumption 1 to 3, the control law (6) for any scalar  ε>0 and any positive matrix Q  guarantees the closed-loop system (7) is asymptotically stable if there exists a positive symmetric matrix R  and constant matrix Z for any ∀ρ∈∆ρ such that

μRR-εL2In+Q-1<0

μ=RAT+AR+ZTρTB2T+B2ρZ+ ε-1In

Furthermore if a feasible solution R ,Z exist in the above LMI, then

P= R-1    ,  K1=ZR-1

Remark 2 : Parametric LMI in Theorem 1 is dependent on ρ but for feasibly solving of it, we do not need to know ρ for any given time of simulation, because we only solve the parametric LMI for lower and upper boundaries of the ρ matrix components. For example if the nonlinear system has m actuators so LMI must be solved for 2mrepetition and at last the answer of LMI for simulation will be the convex combination of the answers.

Proof. For the adaptive closed-loop system described by (7), we first define a Lyapunov functional candidate as

Vx,k3= xTPx+γ-1k3       2                                                  (10)

where k3  is the error dynamic of controller parameter, and k3  is constant

k3t= k3- k3

Then, according to (10), the time derivative of Lyapunov functional candidate for t >0 is :

dVdt = xTA+B2ρK1TP+PA+B2ρK1x+2 xTPB2ρK2

+2 xTPB1w+2γ-1k3k3 '+2 xTP f(x,t)

with multiplying xTPB2 on numerator and de numerator in (8) it is easy to show that :

dVdt = xTA+B2ρK1TP+PA+B2ρK1x+                 (11)

-2xTPB2 ρ 2  β  xTPB2  k3(t)xTPB22  α

+2 xTPB1w+2γ-1k3k3 '+2 xTP f(x,t)

Considering the  0≤α≤ β it is easy to show that

dVdt ≤ xTA+B2ρK1TP+PA+B2ρK1x+               (12)

-2||xTPB2|| k3+2 xTPB1w+2γ-1k3k3 '+2 xTP f(x,t)

because of  xTPB1w and  xTP f(x,t) are scalar, and with considering assumptions 1 and 3 and with use of lemma 1 and 2, for any positive constant ε we can show that:

2 xTPB1w  ≤2 xTPB1w ≤  2 xTPB1 w

2 xTP fx,t ≤ xTε-1P2+εL2In x

Then with the light of inequality (9) we have:

(13)

dVdt ≤ xTA+B2ρK1TP+PA+B2ρK1+ε-1P2+εL2Inx+2 xTPB2 w-2||xTPB2|| k3+2γ-1k3k3 '

The constant k3 is assumed large enough to satisfy the followed inequality, On the other hand, since w  is a known bounded constant, there always exists a constant k3 such that

k3≥ B1B2w                                                                        (14)

Considering (13) and (14) and using of error dynamic of controller parameter we verify the adaptive law

dk3tdt= γ xTPB2

Hence the derivative of Lyapanov function will be

dVdt ≤ xTA+B2ρK1TP+PA+B2ρK1+ε-1P2+εL2Inx

With defining Positive matrix Q such that:

(15)

A+B2ρK1TP+PA+B2ρK1+ε-1P2+εL2In ≤ -Q

In other words

(16)

A+B2ρK1TP+PA+B2ρK1+ε-1P2+εL2In+Q ≤ 0

Defining R= P-1 and K1= ZP , it is easy to show that

(17)

RAT+RA+ZTB2T+B2Z+ ε-1In+RεL2In+QR<0

LMI form of (14) with use Shure Lemma is :

(18)

μRR-εL2In+Q-1<0

μ=RAT+AR+ZTρTB2T+B2ρZ+ ε-1In

after solving of feasibility LMI compute R and Z then we have:

P=R-1  ,      K1=ZR-1                                                         (19)

Considering (15) and Q definition we have

dVx,k3dt  ≤-xTQx≤0

Thus, the global adaptive compensation control problem with disturbance rejection is solvable. The closed-loop system is asymptotically stable, and the state x(t) converges asymptotically to zero.

# Constractive Algorithm for robust adaptive controller design

In this section the robust adaptive controller design algorithm for nonlinear systems with locally Lipchitz condition is presented.

Step‌‌‌1) Compute Lipschitz constant L for the function fx,t using (5). [15], [17] For easy computing the Lipschitz constant let f(x,t) : [a,b]×D → Rm be continuous for some domain D⊂Rn. Suppose that ∂f∂x exist and is continuous on [a,b]×D, then f is locally Lipschitz in x on [a,b]×D and for a convex subset W ⊂D, there exist a constant L ≥ 0 such that ∂f∂x(x,t)≤L on [a,b]×W, then for all t∈ [a, b]; x∈W, and y∈W

fx,t-fy,t≤L x-y

Step2) Check Assumptions 1~3 to be satisfied in all conditions.

Step 3) Solve feasibility LMI (18) and Compute the matrices P= R-1    and  K1=ZR-1 with assuming a positive constant  ε,γ > 0 , 0≤α≤ β  , and Q= QT>0. The ,  α , β  , γ and Q are the arbitrary positive constants and matrix that will help to  LMI being feasible and to achieve suitable time responses.

Step 4) Compute the gains K2(t) using (8).

Step 5) Compute robust adaptive control Law with (6).

# simulation results

In this section the merits of the proposed approach has been shown by simulation on an active suspension system. For simulation, we followed the steps outlined by the algorithm which is presented in section 4 and P  ,  K1 are computed by solving LMI. Active Subsection System modeling has been considered in section 2 and numerical simulation results illustrate the merit of proposed approach.

## Numerical Simulation Results

The following component values that are shown in table 1 are taken from reference [5]

Table1. Parameters of the quarter-car model

| Parameter | Symbol | Unit | Value |
| --- | --- | --- | --- |
| Tyre Mass | mus | Kg | 33 |
| Body Mass | ms | Kg | 200 |
| Suspension Stiffness | ks | N/m | 9000 |
| Nominal Tyre Stiffness | kt0 | N/m | 250000 |
| Variation | δkt | N/m | 100000 |
| Damper | bs | N/m/s | 1000 |

Now according to the section 3, the robust adaptive controller should be designed. Where A=A0 ,  fx,t=L ∆ N xt. The fault modes are as follow.

Mode 1) Actuator is in normal mode, i.e.,

ρ=1                  for                               0≤t≤3  ,   t>7

Mode 2) Actuator is in loss of effectiveness mode, i.e,

ρ=0.7               for                              3≤t≤4

Mode 3) Actuator is in loss of effectiveness mode, i.e.,

ρ=0.1              for                              4≤t≤5

Mode 4) Actuators is in loss of effectiveness mode, i.e.,

ρ=0.5               for                              5≤t≤6

As a result of solving feasible LMI with assuming the following parameters in Matlab software we have:

ε=0.1 , α=1 ,  β=2 ,  γ=10  ,  Q=eye (4,4)

K1T= -112.5-67.43109.920.7 ,  P=203       1414      8-346    2-207    1-346-2072110416-32-320.1

The simulation results of application of the robust adaptive controller for the active suspension system displacement are displayed in Fig. 3.  According to the state definition x1= xs is the vehicle vertical displacement and  x2= xs is the velocity of vehicle and x3= xus is the wheel vertical displacement and x3= xus is the wheel velocity. Because of the disturbance which accrued in 3 to 6 sec with 0.07 meter amplitude Fig 3 shows that the proposed controller stabilizes closed – loop system and attenuate disturbance effect very well. The control signal shows that the proposed approach has a good application and it is applicable in real application.

Fig. 3. Disturbance effect on active Suspension System

Fig. 4 . Suspension Travel and Acceleration of sprung mass

The goal of every control design is to achieve the desired performance specifications on the nominal model as well as other plants that are close to the nominal model. In other words, each novel algorithm wants to achieve the performance objectives in the presence of model error or uncertainty. This is called robust performance. As the mentioned matter we define the tradeoff between passenger comfort, i.e., minimizing car body travel, versus suspension travel and the acceleration of sprung mass as the performance objectives. Fig 4 shows the merit of the proposed approach.

# Conclution

In this paper, a robust adaptive state feedback controller for Lipschitz nonlinear systems in the presence of the bounded disturbance was developed. Using the Lyapunov stability theory, a constructive algorithm based on LMIs was developed for on-line tuning of adaptive control law to stabilize the closed-loop system. It has been also shown that because of being fixed one of state feedback gains, the proposed approach is more suitable from practical point of view. An active suspension system as a numerical example has shown the effectiveness of the proposed method. The open areas of research under study are the extension of the proposed method to a general class of nonlinear uncertain suspension systems in the presence of sensors and actuator faults using state and feedback and implementation issues for real applications.

# REFERENCES

[1]    Y. Huang, J. Na, X. Wu, X. Liu, Y. Guo, “Adaptive control of nonlinear    uncertain active suspension systems with prescribed performance,” ISA Transactions, 2014

[2]  Cao D, Song X, Ahmadian M. Editors's perspectives: “road vehicle suspension design, dynamics, and control”. Vehicle system dynamics 2011, 49:3–28.

[3]  Sun W, Gao H, Kaynak O. “Finite frequency H_\infinity control for vehicle active suspension systems”. IEEE Trans Control System Technology 2011, 19:416–22.

[4]   Hrovat D. “Survey of advanced suspension developments and related optimal control applications”. Automatica 1997, 33:1781–817.

L. Palkovics, P. Gaspar, J. Bokor “Design of Active Suspension System in the Precemse of Physical Parametric uncertainties”American Control Conferences, 1993.

[6] Palkovis, L., Venhovens, P.Th.3. "Investigation on stability and possible chaotic motions in the controlled wheel suspension system", Vehicle System Dynarics, vol. 21, 1992.

[7]    A. Qazi, Afzal Khan, M. Tahir Khan, Sahar Noor, “A Parametric Study  on Performance of Semi-Active Suspension System with Variable Damping Coefficient Limit”. AASRI Conference on Intelligent Systems and Control 2013, pp. 154 – 159

[8]   S. Lajqi, S. Pehan “Designs and Optimizations of Active and Semi-Active Non-linear Suspension Systems for a Terrain Vehicle”, Journal of Mechanical Engineering 58(2012)12,pp: 732-743

[9]   Y. Sam, J. Osman, M. Ruddin, A. Ghani, “A class of proportional-integral sliding mode control with application to active suspension system” , System & Control Letters 51, 2004, pp:217-223

[10]   H. Gao, J. Lam, C. Wang, “Multi-objective control of vehicle active suspension systems via load-dependent controllers”, Journal of Sound and Vibration 290, 2006, pp: 654-675

[11]   G. Koch, K. Diepold, B. Lohmann, “Multi-Objective Road Adaptive Control of an Active Suspension System”, Journal of Motion and Vibration Control, 2009, pp:189-200

[12]    L. Guo, L. Zhang, “Robust H_inf control of active vehicle suspension under non-stationary running”, Journal of Sound and Vibration Control 331, 2012, pp:5824-5837

[13]   Y. Huang, J. Na, X. Wu, X. Liu, Y. Guo, “Adaptive control of nonlinear uncertain active suspension systems with prescribed performance”, ISA Transaction, 2014

[14]  W. Sun, H. Pan, Y. Zhang, H. Gao, “Multi-objective control for uncertain nonlinear active suspension systems”, Mechatronics 24, 2014, pp: 318-327

[15]   Marquez, H. J. , "Nonlinear control systems: analysis and design": Wiley-Interscience Hoboken, NJ. 2003

[16]  Wang, Y., Xie, L., deSousa, C.E. “ Robust Control of a class of uncertain nonlinear systems” ,System & Control Letter, 1992, 19, (2).pp:139-14

[17]   Khalil K H. Nonlinear Control. 3th ed. Prentice Hall 2001.