Oversized Inductive Post 125 GHz Band-Pass Waveguide Filter

M. Tayarani1, E. Mehrshahi2, S.Oka3

- School of Electrical Engineering, Iran University of Science and Technology

- Electrical and Computer Engineering Dept., Shahid Beheshti University

- Systec Research Inc., Sagamihara, Japan

Abstract— In this paper, a low insertion loss, wide band, 125 GHz band-pass filter is proposed, fabricated, and demonstrated. The introduced filter is a 7th order double inductive post filter, implemented in an oversized waveguide, in order to facilitate the fabrication process and improve the frequency characteristics. This filter has been fabricated employing micrometer precision, high speed CNC milling machines. The measured center frequency is 125 GHz and the 3dB bandwidth is 22 GHz, resulting in a 17% relative bandwidth. The minimum insertion loss is 0.5 dB and a return loss of more than 12 dB has been achieved in the pass-band. The stop-band rejection is about 35 dB. The simulated and measured results show a very good agreement.

Index Terms—band-pass filter, double inductive post filter oversized waveguide, THz band

I

# INTRODUCTION

N recent years there have been a growing interest in the applications of the terahertz frequencies (0.1-10 THz), located between millimeter wave and infrared. Terahertz waves have the ability to carry ultra wideband signals and most materials have spectral fingerprints in the terahertz region which means they could be detected by terahertz spectroscopy.

With all the stated benefits of this fascinating electromagnetic spectrum, there’s no wonder that terahertz waves are expected to be used in scanning systems for dangerous materials, medical and non-medical imaging systems, and ultrafast wireless communication systems which are gaining an immense interest [1].

One of the most important components in the terahertz regime are band-pass filters. The application of terahertz wave filters in wireless communication systems is also ever-growing. In spite of their constant progress, as the operation frequency increases to terahertz frequencies, designing terahertz components and devices face many challenging problems. As the frequency increases, metal loss and surface roughness will increase the filter insertion loss, therefore degrading the frequency performance of the filter [2]. Realization

and fabrication of terahertz filters are still greatly constrained by problems arising by physical limitations [3].

In designing components and systems, oversized waveguides are attractive substitutes for regular rectangular

waveguides, because of their suitable sizes in these frequencies. These waveguides have lower attenuation and higher power handling capability compared to regular waveguides.

In this paper, the proposed filter is a wideband, 7th order dual inductive post, band-pass filter, realized in an oversized waveguide. The filter has been fabricated with micrometer precision high speed CNC milling machines. In section II a brief summary of the analytical design procedure has been provided. Section III explains the physical characteristics and dimensions of the oversized waveguide filter and in section IV, the fabrication considerations and measurement results have been illustrated.

# Filter design

In order to design an inductive post band pass waveguide filter, parameters like diameter of the posts, distance between opposing posts, and their offset from the waveguide side walls should be obtained. A Chebyshev filter synthesis has been used for this filter.

This filter has been realized using coupled resonators. Since the guiding media for the electromagnetic waves is a waveguide, in order to realize a resonant cavity, inductive posts with the approximate distance of half wavelength according to the central frequency have been used. Each of these resonators is coupled to their adjacent resonator by a pair of inductive posts.

In order to design a Chebyshev filter, the required filter parameters are 0.1 dB pass band ripple, 125 GHz center frequency, and the edge frequencies are 116 GHz and 134 GHz. The order of this filter has been chosen to be 7 and the design has been carried out for the dominant mode.

Designing a band pass, lumped element filter has been described in many microwave literature [4]. The model of the normalized low-pass filter prototype is shown in Fig.1.

Fig.1- a. The low-pass normalized filter equivalent circuit.b. Equivalent circuit of an impedance inverter.

According to the desired Chebyshev filter characteristics and the tables found in most microwave literature [4], the parameters, g1to gn, can be obtained. The obtained gielements should be used to design the coupling between the resonators.

If the diameters of the inductive posts are small compared to the larger side of the waveguide cross section, the equivalent circuit shown in Fig.1 could be used, where X is the inductance of the inductive posts [4]. Each X is obtained using the coupling factor between the resonators, Kj,j+1, using the following formula.

Xj,j+1Z0=Kj,j+1Z01+(Kj,j+1Z0)2                                                                         (1)

Each Kj,j+1 could be calculated using gi elements using the following equations.

K01Z0=K78Z0=π2wλg0g1ω1'            K12Z0=K67Z0=π2wλω1'1g1g2            (2)

K23Z0=K56Z0=π2wλω1'1g2g3           K34Z0=K45Z0=π2wλω1'1g3g4

Whereω1'=1; and wλ is the relative bandwidth. The electrical length between the inductive posts is related to the inductance of each of these post pairs and is obtained from the following expression.

θj=π-12tan-12Xj-1,jZ02+tan-12Xj,j+1Z02      (3)

The physical distance between the post pairs have been obtained from the electrical lengths l1=l7=θ12λg0π=1.15 mm, l2=l6=θ22λg0π=1.38 mm, l3=l5=θ32λg0π=1.44 mm and l4=θ42λg0π=1.45 mm.

With λg0 as the wavelength at 125GHz center frequency. By using this method, the distance between the inductive posts can be obtained. The next step is to realize the values of Xj,j+1 using the double inductive posts and to find the offset of the posts from the center of the waveguide and their diameters.

In order to facilitate the design and fabrication, the offset of the posts from the side walls of the waveguide has assumed to be equal to 0.69mm as a starting point. By assuming the offset to be 0.69mm; the diameters of the posts can be obtained from the following expressions [5]

Xj,j+1λg0a=-πdj2a+cscπcja2Sj+Tj                             (4)

Sj=-1-πdj2asinπcja2-14lnπdj4a+lncotπcja

Tj=1.4aλm=3 odd∞1m2πdj2a+1m1+1.5m2sin2mπcjae-mπdj2a

In Eq. 4, dj is the distance between the posts and Sj and Tj are defined below the Eq. 4.The series in the expression for Tj rapidly converges [5]. In the above equations, c=0.69 mm is the offset from the side walls,λ is the wavelength at center frequency of 125 GHz. After optimization with high frequency structure simulator HFSS13 [6], the values found for the diameters, are as follows d1=d8=0.38 mm, d2=d7=0.6 mm, d3=d6=0.66 mm and d4=d5=0.67 mm.

It should be mentioned that although an oversized waveguide has been used with dimensions of 3 mm×0.82 mm, as it is depicted in Fig. 2,  the higher order modes do not cause any problems. By considering the size of the cross section of the waveguide, it is seen that both TE10 and TE20 modes can propagate in it. Although by looking carefully at the position of the posts and the offset from the sidewalls, it could be seen that these posts are approximately located at the peak of the electric field of the TE20 mode and thusly short circuiting it. Therefore by using an oversized waveguide, not only the fabrication of the filter has been facilitated, but also by placing the posts near the peak of the electric field of the TE20 mode, its propagation has been prevented.

Fig. 2. Geometry of the 125 GHz dual post band-pass waveguide filter.

# Physical characteristics and simulation results

The structure of the 125 GHz band-pass waveguide filter is shown in Fig.2. As mentioned earlier, this filter has been designed using double inductive posts. 8 pairs of symmetric inductive posts have been placed in an oversized waveguide with internal dimensions of  3 mm×0.82 mm resulting in a 7th order band-pass filter. The distance between the centers of the opposing inductive post pairs are 1.62mm and the length of the main waveguide containing the posts is 18 mm.

This filter is going to be used in a line of sight waveguide data transmission link with waveguide cross section that is different from the main waveguide. Therefore, as shown in Fig. 2, the input and output openings of the main waveguide have been tapered in order to connect to the WR-6 which has the standard 1.64 mm×0.82 mm dimensions. The length of the taper region is 4mm. The inductive post diameters, their distance and other dimensions have been shown in Fig. 2.

Fig.3. Simulated and measured S-parameters of the fabricated dual post band pass filter.

The filter shown in Fig.2 has been simulated using finite element simulation software, high frequency structure simulator, HFSS13 [6]. The simulated S-parameters of the filter are shown in Fig.3. As shown in Fig.4 the relative bandwidth of the filter at 125 GHz center frequency is 17% which indicates that the filter is a wide band, band- pass filter. The transmission loss is lower than 0.2dB. The S11 diagram shows a very good matching and a desirable frequency characteristic.

# Fabrication and measurement

As mentioned earlier, unfortunately small dimensions make terahertz waveguide components difficult and costly to fabricate through conventional machining and assembly [7].

Using the oversized waveguide not only resulted in lower attenuation, higher power handling, and overall better frequency response; but also facilitated the machine work resulting in an easier fabrication process. The proposed 125 GHz dual inductive post band pass filter has been fabricated using micrometer precision, high speed CNC milling machines.  The basic material of the waveguide and the filter is brass and the filter has a 6 μm gold coating so the structure would not oxidize. Fig.3 shows the fabricated filter.

Fig.7. Measured group delay

The measured S-parameters have been shown in Fig. 3. The center frequency is 125 GHz, the 3dB bandwidth is 22 GHz (116GHz to 138GHz) and the relative bandwidth is 17%. The minimum insertion loss at 125 GHz is 0.5 dB, the return loss of the entire pass band is lower than -10 dB, and the good stop band rejection has been achieved. Moreover the group delay of this filter is measured and it is depicted in Fig. 4. The interesting feature of this filter which has made it superior to other filters is the very good group delay in the pass-band. This group delay shows that although the waveguide is oversized, which means that the propagation of higher order modes is probable; the higher order modes have not degraded the group delay in any way.

# Conclusion

In this paper, a novel wide band dual inductive post waveguide filter with low insertion loss and high rejection has been designed and successfully fabricated at 125 GHz center frequency. The proposed filter has been implemented in an oversized waveguide which facilitates the fabrication process. The measured results are in great agreement with the simulation which indicates the precise design procedure, fabrication and measurement of the proposed 7th order double inductive post filter.

Acknowledgement

Authors would like to thank Systec Research Inc., Japan for for full support of this work.

# References

- Hosako, I.; Sekine, N.; Patrashin, M.; Saito, S.; Fukunaga, K.; Kasai, Y.; Baron, P.; Seta, T.; Mendrok, J.; Ochiai, S.; Yasuda, H., “At the Dawn of a New Era in Terahertz Technology” , proceedings of the IEEE, pp. 1611-1623,2007.

- Wang Cheng; Lu Bin; Liu Jie; Deng Xianjin,“140GHz Waveguide H ladder Bandpass Filter”2012 International Conference on Microwave and Millimeter Wave Technology (ICMMT),Vol.2,pp1-4,2012.

- Zhurbenko, V.; Krozer, V.; Kotiranta, M.; Bouamrane, F.; Megtert, S.; Bouvet, T.; Di Carlo, A.; Dispenza,M. “Excitation of a double corrugation slow-wave structure in terahertz range” Proceedings of the 5th European Conference on Antennas and Propagation (EUCAP),: 2011 ,pp 1041 – 1043.

- Matthaie G. L., Young L., and Jones E. M. T., "Microwave Filters, Impedance Matching Networks, And Coupling Structures," Book Mart press , North

Bergen ,NJ, USA, Nov. 1985.

- H.Gruenberg, “Symmetrically Placed Inductive Posts in Rectangular Wave Guide” Canadian Journal of Physics, vol. 30, issue 3, pp. 211-217.

- HFSS 3D Full-Wave Electromagnetic Field Simulation ver13.

- Victor M. Lubecke, Koji Mizuno, and Gabriel M. Rebeiz, “Micromachining for Terahertz Applications” IEEE Transactions on Microwave Theory and Techniques.