# Optimal Sizing of Distributed Energy Storage with Consideration of Demand Response in Distribution Systems

M. Seydali Seyf Abad

PhD Student

Electrical Engineering

Sharif University of Technology

Tehran, Iran

Email: seydali@ee.sharif.edu

M.R. Karimi Gharigh

PhD Student

Electrical Engineering

Sharif University of Technology

Tehran, Iran

Email: mrkarimi@ee.sharif.edu

A. Safdarian

Assistant Professor

Electrical Engineering

Sharif University of Technology

Tehran, Iran

Email: safdarian@sharif.edu

J. Nokhbeh Zaeem

PhD Student

Electrical Engineering

Sharif University of Technology

Tehran, Iran

Email: javadn8823@yahoo.com

Abstract—In future smart grids, energy storage systems (EESs) and demand response (DR) are expected to have an important role in balancing load changes arising from life-style of consumers and varying output of some renewable energy resources. However, activating DR and installing ESSs need huge investment forcing system owner to justify their effectiveness prior to any implementation. This paper presents a model to find the optimal capacity of EESs in distribution systems with demand responding to electricity price changes. The model aims at maximizing system owner profit by lessening total electricity provision costs. The model is formulated as a mixed integer non-linear programming problem. Due to the intricacy and big size of the problem, a decouple-based procedure is proposed wherein optimal charge/discharge pattern of EESs is found by means of a quadratic programming (QP) model. The method is applied to the IEEE 33-bus network and simulation results are thoroughly discussed.

Keywords-component; Distributed energy storage systems; distribution systems; loss minimization; optimal sizing; quadratic programming

##### NUMENCLATURE

| ΩStorage | Network buses assortment which contain battery |
| --- | --- |
| ΩBus | Network buses assortment |
| ΩLine | Network lines assortment |
| Yi,j | Element magnitude of admittance matrix in row i and column j |
| θi,j | Element angle of admittance matrix in row i and column j |
| Vh,di | Voltage magnitude of bus i at hour h of day d |
| δh,di | Voltage angle of bus i at hour h of day d |
| Vmini | Minimum acceptable voltage of the network |
| Vmaxi | Maximum acceptable voltage of the network |
| Flowh,dl | Flow of line i at hour h of day d |
| Flowmaxl | Maximum allowable flow of the line i |
| Pdh,di | Real power consumption at bus i, at hour h of day d |
| Pmaxi | Real power load peak of the bus i |
| Qdh,di | Reactive power consumption at bus i, at hour h of day d |
| Qmaxi | Reactive power load peak of the bus i |
| ηh,d | Load to peak load ratio at hour h of day d |
| Pgh,di | Real power generated at bus i, at hour h of day d |
| Pghi | Real power generated at bus i, at hour h |
| Lghi | Real power generated by bus i generator at hour h |
| Pdhi | Real power consumed at bus i, at hour h |
| ki | kth battery which is placed at bus i |
| γhki | kth battery charge rate at hour h which is placed at bus i |
| βhki | kth battery discharge rate at hour h which is placed at bus i |
| CESS | Battery investment cost ($/kWh) |
| ξk | kth battery nominal capacity (kWh) |
| zk | Number of steps of kth battery |
| ξstep | Size of battery step (kWh) |
| ξmink | kth battery minimum capacity (kWh) |
| ξmaxk | kth battery maximum capacity (kWh) |
| TESS | Battery useful age |
| Prh,delec | Electricity price at hour h of day d |
| Ph,dGen | Total generated power at hour h of day d |
| ϕhk | kth battery charge status at hour h (kWh) |
| ϕh,dk | kth battery charge status at hour h of day d (kWh) |
| αCk | kth battery charge efficiency |
| αDk | kth battery discharge efficiency |
| γhk | kth battery charge rate at hour h |
| γh,dk | kth battery charge rate at hour h of day d |
| βhk | kth battery discharge rate at hour h |
| βh,dk | kth battery discharge rate at hour h of day d |
| ϕΛk | kth battery initial charge status |
| ϕinitialk | kth battery initial charge status |
| λ | Time slice duration |
| ψk | kth battery rated power |
| ψmink | kth battery minimum rated power |
| ψmaxk | kth battery maximum rated power |
| ψCk | kth battery maximum charge rate |
| ψDk | kth battery maximum discharge rate |
| Ph,dGen,after | Approximate total generated power at hour h of day d after ESSs operation |
| Ph,dGen,before | Approximate total generated power at hour h of day d before ESSs operation |
| Ph,dloss | Approximate total loss of network at hour h of day d |
| PDh,d | Vector of real power consumed at hour h of day d |
| QDh,d | Vector of reactive power consumed at hour h of day d |
| H | Imaginary resistance matrix of network |
| Γh,d | Vector of charge rate at hour h of day d |
| Βh,d | Vector of discharge rate at hour h of day d |
| fh,d | Linear coefficient matrix of ESSs operation on loss at hour h of day d |
| Di | Demand for time period i from n time periods |
| Pricej | Demand price for time period j from n time periods |
| Di0 | Initial demand for time period i from n time periods |
| Pricej | Initial demand price for time period j from n time periods |
| ηh,dDR | Load to peak load ratio at hour h of day d after consideration of demand response |

# INTRODUCTION

The power industry has been undergoing significant changes in recent years. Among the changes rapid growth of renewable energy sources (RESs) and DR are usually referred to the most conspicuous ones. RES can be seen as opportunity and threat at the same time. They are an opportunity since they produce energy without any fuel cost. However, they are taken as threat as since their output power cannot be predicted accurately. This challenge become more important as the penetration of these sources grows in power systems. As already stated in [1], RES contribute in gross electricity production of the 27 European Union members will grow from 14.3% in 2005 to 36.1% in 2030. As a matter of fact, the contribution of intermittent RESs will reach 20.7% of the total electricity production in 2030 [1]. Considering the importance of the issue and growing penetration of RES, finding ways to reduce concerns on RES intermittent behavior has attracted the attention of several area researchers. Bearing this aim in mind, ESSs play a very important part by helping the existing components to do their duties better and cheaper [2].

ESSs benefits exist along the entire value chain of the power system [3, 4]. ESSs have a great number of benefits for distribution system operators (DSO), namely voltage support, distribution losses reduction, capacity support, and system expansion deferral, to name but a handful. Reduction of transmission congestion and deferral of transmission expansion investment are also significant benefits of ESSs for transmission system operators (TSO). ESSs benefits for independent system operators (ISO) are regulation, spinning reserve, black start, and price arbitrage. It should be mentioned that ESSs have some benefits for end users, namely enhanced power quality, reliability improvement, and reduction in demand charges, to name but a few. ESSs have quite a few potential applications in power systems [5, 6]. They can be used to decrease the uncertainty risk which is imposed by RESs [7-10] as well as to provide load shaping potentials [11, 12] to shift loads from higher price periods to a lower price period.

There are two sets of questions about ESS which most of researchers try to answer. The former aims to find the optimal size, location and investment policy of ESSs. The latter determines the optimal management policy for the ESS to minimize the costs.  Answering the above questions needs a mixed integer non-linear problem to be solved. Since, distribution network operation modeling imposes non-convex power-flow constraints. Usually these kinds of problems handled via  either a linearization using DC approximation [13, 14] or a relaxation of feasible sets using semi-definite programming [15-17]. Many researchers have studied sizing of ESSs. In [18, 19], sizing of ESSs using only economic arguments without any network constraints is studied. In [20, 21], optimal size of ESSs in single bus power systems has been investigated. Kanoria et al. in [22], studied the effect of sizing of distributed storage resources on generation cost of networks. It should be noted that the optimal control policy of ESSs is another challenge which has been extensively studied. In [21, 23, 24], the control of a single storage device without any network constraint is studied. [22, 25], model the network role in the operation of ESSs. None of these researches considered effect of DR on ESSs optimal size. It should be mentioned that DR changes the load profile. By doing so, optimal charge/discharge pattern of ESSs are changed. Both ESSs and DR have remedy effect on load profile. Therefore, investigation of the sizing problem off ESSs in systems where DR is enabled, is obligatory. In this paper, sizing and control policy of ESSs are formulated. Then a new method to obtain the charge/discharge pattern of EESs with DR consideration is proposed.

The remainder of this paper is organized as follows: The ESS and DR models are presented in Sections II and III respectively. Section IV presents the problem formulation. Solution methodology is presented in Section V. A medium voltage distribution network along with demand profile and numerical results are presented in Section VI. Finally, Section VII outlines relevant conclusions.

# Storage Device Modeling

An ESS is characterized by its rated power ψ, rated energy ξ, and charge/discharge efficiencies. ESSs usually store energy at low demand and discharge energy when it is needed. The future charge status of an ESS depends on three factors -that is- the initial charge status; charging and discharging rates; and the duration of the current time slice.

| ϕh+1k=ϕhk+αCkγhk-1αDkβhkλ for ∀k∈ΩStorage,∀h | (1) |
| --- | --- |

After a cycle of time slices, the charge status ϕ of the ESS must return to initial values for serving the next cycle:

| ϕΛk=ϕinitialk for ∀k∈ΩStorage | (2) |
| --- | --- |

The followings are the limits on the ESS control parameters:

| βhkλ≤αDkϕhk for ∀k∈ΩStorage,∀h | (3) |
| --- | --- |
| αCkγhkλ≤ξk-ϕhk for ∀k∈ΩStorage,∀h | (4) |
| 0≤βhk≤ψDk for ∀k∈ΩStorage,∀h | (5) |
| 0≤γhk≤ψCk for ∀k∈ΩStorage,∀h | (6) |
| 0≤ϕhk≤ξk for ∀k∈ΩStorage,∀h | (7) |
| ψmink≤ψk≤ψmaxk for ∀k∈ΩStorage | (8) |
| ξmink≤ξk≤ξmaxk for ∀k∈ΩStorage | (9) |

Equations (3) and (4) indicate that energy to be consumed and stored is restricted to the available energy and the empty capacity of ESSs, respectively. Equations (5), (6), and (7) are constraints imposed by the properties of ESSs: the charge/discharge rates and the charge status. Constraints (8) and (9) indicate that the rated power and energy of ESSs are limited by ESS technologies. At time h, discharge rate β and charge rate γ are modeled as positive and negative power injections, respectively. Therefore, they enter into the power balance equation as below:

| Pghi=Lghi-Pdhi-γhki-βhki for ∀i∈ΩBus,∀h | (10) |
| --- | --- |

# Demand Response Modeling

It is crystal clear that the demand for most products decreases as the price increases. Figure 1 shows a typical relation for the reduction in demand due to increase in price.

Figure 1. Typical demand curve

Since quantifying this curve is difficult, if not impossible, economists often linearize it around a given point. Then, they define a new quantity referred to as price elasticity of demand which is the relative slope of this curve[26].

| ε=ΔDD0ΔPricePrice0 | (11) |
| --- | --- |

This coefficient demonstrates the relative change in demand of a commodity which is resulted from a change in the price of that commodity. In this paper, it is assumed that all prices and demands have been normalized with respect to a given point. Generally, the customer’s response to dynamic pricing is not only related to the current electricity price, but also related to the adjacent prices. The relationship between the demand of a certain period and the price of each period can be written as follows:

| Di=f(Price1,Price2,⋯,Pricej,⋯,Pricen) | (12) |
| --- | --- |

Using the first order Tylor expansion at point (Pricej0,Di0) results in:

| Di=Di0+j=1nΔDiΔPricej(Pricej-Pricej0) | (13) |
| --- | --- |

Define self and cross elasticity as[27]:

| εii=ΔDiDi0ΔPriceiPricei0 | (14) |
| --- | --- |
| εij=ΔDiDi0ΔPricejPricej0 | (15) |

So, the price-elasticity matrix is as follows:

| ΔD1D10ΔD2D20⋯ΔDnDn0=ε11ε12⋯ε1nε21ε22⋯ε2n⋮⋮⋮⋮εn1εn2⋯εnnΔPrice1Price10ΔPrice2Price20⋯ΔPricenPricen0 | (16) |
| --- | --- |

In this paper, effect of demand response on load profile is formulated as follows:

| η1,dDRη2,dDR⋮ηn,dDR=η1,dη2,d⋮ηn,d+ΔD1D10ΔD2D20⋮ΔDnDn0 | (17) |
| --- | --- |

# Problem Formulation

## Objective Function

Aim of this paper is to minimize the annual system cost using stochastic programming (i.e. the sum of operation and investment costs for a year). Annual system costs consist of electricity procurement and ESSs cost. The objective function in this paper is as follows:

| Obj=min∀k∈ΩStorageCESSξkTESS+d=1365h=124Prh,delecPh,dGen | (18) |
| --- | --- |

Where

| ξk=zkξstep for ∀k∈ΩStorage | (19) |
| --- | --- |

## Constraints

This problem has two types of constraints. The first set is related to ESSs constraints which are presented in Section II. The second is regarding the network constraints which are presented as follows:

| Ph,dGen=i∈ΩGenPgh,di | (20) |
| --- | --- |
| Pdh,di=ηh,d×Pmaxi for ∀i∈ΩBus | (21) |
| Qdh,di=ηh,d×Qmaxi for ∀i∈ΩBus | (22) |

If DR is considered in the formulation, then Pdh,di and Pdh,di are obtained from equations (23) and (24), respectively.

| Pdh,di=ηh,dDR×Pmaxi for ∀i∈ΩBus | (23) |
| --- | --- |
| Qdh,di=ηh,dDR×Qmaxi for ∀i∈ΩBus | (24) |
| Pgh,di=Pdh,di+Vh,dij∈ΩBusYi,jVh,djcosδh,di-δh,dj-θi,j for ∀i∈ΩBus-ΩStorage | (25) |
| Pgh,di=Pdh,di+γh,dki-βh,dki+Vh,dij∈ΩBusYi,jVh,djcosδh,di-δh,dj-θi,j for ∀i∈ΩStorage | (26) |
| Qgh,di=Qdh,di+Vh,dij∈ΩBusYi,jVh,djcosδh,di-δh,dj-θi,j for ∀i∈ΩBus | (27) |
| Vmini≤Vh,di≤Vmaxi for ∀i∈ΩBus | (28) |
| 0≤Flowh,dl≤Flowmaxl for ∀l∈ΩLine | (29) |
| Flowh,dl=Vh,di×Yi,j× Vh,di2+Vh,dj2-2Vh,diVh,djcosδh,di-δh,dj for ∀l∈ΩLine | (30) |

Equation (20) represents the total power generated to supply the consumers; (21) and (22) model the network load profile; (23) and (24) model the network load profile with consideration of DR; (25)-(27) are the power balance equations of the system; (28) indicate the acceptable range for voltage of all buses; (29) constraints flow over the distribution lines. All in all, the optimization problem is:

| Obj=min∀k∈ΩStorageCESSξkTESS+d=1365h=124Prh,delecPh,dGen s.t. equality constraintsinequality constraints | (31) |
| --- | --- |

Where equality constraints are (1), (2), (25), (26) and (27); and inequality constraint are (3)-(9), (28), and (29).

# Solution Methodology

The above formulation is a large-size non-linear problem. This kind of optimization problems can be solved by using heuristic and non-heuristic methods. In this paper, a decouple-based method which uses a QP model for finding the charge/discharge pattern of ESSs is proposed to solve the problem described in (31). Figure 2 shows the flowchart of the proposed method.

## Genetic Algorithm Implementation

For solving the problem, it is assumed that ESS capacities are integer multiples of a unit base. The GA individuals are characterized by the number of elements of a pre-assigned size of ESSs. The GA fitness function is as equation (18). The outputs of solving the proposed QP model for charge/discharge pattern are used to evaluate the fitness function of each individual. The algorithm reaches its target when the best fitness function value remains constant over a predetermined number of generations or the maximum number of iterations is reached.

Figure 2. Flowchart of the proposed method for solve the developed optimization problem

## QP Besed Model Implementation

The main benefit of the proposed solution method is using QP to find the charge/discharge pattern of ESSs. The proposed formulation for using QP is as follows:

| Obj=minh=124Prh,delecPh,dGen,after | (32) |
| --- | --- |

This objective function is used for every day of a year. The details of this problem are in the following:

| Ph,dloss=12PDh,dT×H×PDh,d+12QDh,dT×H×QDh,d | (33) |
| --- | --- |

In equation (33), losses power formulated as a function of active and reactive powers of demand in every bus.

| Ph,dGen,before=Ph,dloss+1,1,…,1PDh,d | (34) |
| --- | --- |

Superposition method is proposed to obtain the total needed power after operation of ESSs. Equation (35) indicates the effect of ESSs operation on total needed power.

| Ph,dGen,after=Ph,dGen,before+12Γh,d-Βh,dT×Hk×Γh,d-Βh,d+fh,dTΓh,d-Βh,d | (35) |
| --- | --- |

Where

| Hki,j=Hki,kj ∀1≤i,j≤ΩStorage,ki , kj∈ΩStorage | (36) |
| --- | --- |
| fh,di,1=1+12j∈ΩBusHki,jPdh,dj+12j∈ΩBusPdh,djHj,ki ∀1≤i≤ΩStorage,ki ∈ΩStorage | (37) |

In addition to the above constraints, the proposed QP formulation has other constraints which are (1)-(9).

# Numerical Results

## Test System

Figure 3 demonstrates the single line diagram of the IEEE 33-bus system [28]. This network is a radial distribution system with the total load of 3.715 MW and 2.300 MVAr.

Figure 3. IEEE 33-bus system single line diagram

It is assumed that load profile of this network over a year is as Figure 4. This profile is extracted from IEEE 24-bus network data [29]. For obtaining the exact load profile, Figure 4 diagram should be multiplied by the peak load of every bus.

The followings are the assumptions that are used to solve this problem. ESS investment cost of one kWh capacity is equal to 200$/kWh [30]. Useful life of batteries is 15 years; maximum charge/discharge rate of every battery is assumed to be 30% of the battery capacity; the initial energy of each battery is 10% of battery capacity; the time slice duration is one hour. It is assumed that electricity price is real time. Figure 5 shows the real time daily energy tariffs for a random day. Equation (38) shows the relation used to calculate the electricity price of every hour of a day. In this paper it is assumed that Prmin is 7 cent per kWh.

| Prh,delec=Prminηh,dmin1≤h≤24ηh,d2 | (38) |
| --- | --- |

Figure 4. Hypothetical load profile of IEEE 33-bus network

Figure 5. Real time electricity prices during a typical day (352th day of the year)

Regarding ESS efficiency, the value of 0.85 is considered in both charging and discharging states [31]. It is assumed that there are three batteries in the test network which are at buses 12, 25, and 30. It should be mentioned that total ESSs size is limited to 3500 kWh because of investment costs limitation. Self-elasticity of this network is assumed to be -0.2; it should be noted that in reality, load cannot be shifted to any hour of the day. So, in this paper, it is assumed that load can be shifted to at most 3 hours after and before its default time. Another constraint of elasticity matrix is that sum of every raw should be zero due to the customer life-style. With this in mind, cross-elasticity of elasticity matrix is either 0.26 or zero.

## Simulation Results

The problem described in (31) is solved with the data presented in the previous subsection by using the proposed method in MATLAB, and the result is presented here. TABLE I shows the optimal ESSs sizes which minimize the annual costs.

Total cost of energy procurement of one year without any remedy action in the network is 3118371.5 $, while total cost of energy supply after applying DR and placement of ESSs for the same period is 2778541.7 $. So using both DR and ESSs cause a 10.898% decrease in total annual costs. Some of this cost reduction is due to DR and the rest is thanks to ESS placement. Here, it is assumed that DR does not has any costs and demand just response to the real time electricity prices. However, DR programs usually impose some costs to DSOs in the real world. For instance, real time pricing may need initial investments on enabling technologies like two-way communication media between customers and the program administrator. In the analysis provided this paper, the required costs are discarded. Figure 6 and Figure 7 show the daily ESSs active power and the corresponding stored energy daily profile during typical day.

TABLE I.    Location and size of ESSs

| Bus number | 12 | 25 | 32 |
| --- | --- | --- | --- |
| ESS size (kWh) | 56×25 | 22×25 | 49×25 |

Figure 6. Daily ESSs active power (352th day of the year)

Figure 7. Daily ESSs stored energy (352th day of the year)

These figures show that ESSs charge and discharge during off peak and peak hours, respectively. Figure 8 indicates the effect of ESSs and DR on load profile of a typical day. As it can be seen, although DR and ESSs are effective in flattening both system load profile, the most evenly distributed profile is achieved when both of the tools are employed at the same time.

Figure 8. Effect of ESSs and DR on load profile (352th day of the year)

Figure 9 demonstrates the load profile of the test system after applying the DR effect. As it can be seen from this figure, DR on its own cannot significantly smooth the load profile. Peak to average ratio of the network without the effect of DR and ESSs; with DR; and with both DR and ESSs is 1.2117, 1.1629, and 1.0895, respectively. Decreasing the peak to average ratio is not the main goal of ESS placement. However, it is obtained as a side benefit of using ESSs. Figure 10 shows the load profile after applying the DR and ESSs effect.

Figure 9. Load profile after applying DR effect

Figure 10. Load profile after applying ESSs and DR effects

This diagram shows that ESSs adjust load during peak and off-peak hours. By doing so, system load profile becomes smoother than before. Besides, using DR and ESSs simultaneously reduces the total generations of the system at peak hours from 3.915 MW to 3.541 MW. This can be translated to about 9.55% release in system capacity.

# Conclusion

This paper focused on the description of procedure applied to the optimal sizing of ESSs with the purpose of maximizing operator profit. It should be mentioned that in this paper demand responded to time varying electricity prices. The response of demand to the prices is modeled by means of elasticity matrix. The lack of the suitable modeling makes it difficult to find the optimal charge/discharge pattern of ESSs. Therefore, a new method to model the ESSs was proposed. This paper also proposed a feasible procedure to solve the ESSs sizing problem. The use of the proposed procedure in the IEEE 33-bus network demonstrated its capability to find the optimal size of ESSs. The main result of this paper is that, not only ESSs and DR decrease the cost of the load procurement, but also they smooth the load profile of the network. It is noteworthy that using only one of these measures does not result in an effective peak-shaving. But, applying both ESSs and DR effectively reduces the peak of the load profile.

##### References

[1]	Carpinelli, G., et al., Optimal integration of distributed energy storage devices in smart grids. Smart Grid, IEEE Transactions on, 2013. 4(2): p. 985-995.

[2]	Lopes Ferreira, H., et al. Storage Devices Impact on Electricity Distribution. in CIRED 21st International Conference on Electricity Distribution, Paper. 2011.

[3]	Delille, G., et al. Energy storage systems in distribution grids: new assets to upgrade distribution networks abilities. in Proc. 20th International Conference and Exhibition on Electricity Distribution (CIRED2009). 2009.

[4]	EPRI, D., Electricity Energy Storage Technology Options-A White Paper Primer on Applications, Costs and Benefits. 2010, Report.

[5]	Chu, S. and A. Majumdar, Opportunities and challenges for a sustainable energy future. nature, 2012. 488(7411): p. 294-303.

[6]	Storage, E.E., IEC white paper. 2011, December.

[7]	Atwa, Y.M. and E. El-Saadany, Optimal allocation of ESS in distribution systems with a high penetration of wind energy. Power Systems, IEEE Transactions on, 2010. 25(4): p. 1815-1822.

[8]	Biello, D., Storing the breeze: New battery might make wind power more reliable. Scientific American. Available at http://www. scientificamerican. com/article. cfm, 2008.

[9]	Budischak, C., et al., Cost-minimized combinations of wind power, solar power and electrochemical storage, powering the grid up to 99.9% of the time. Journal of Power Sources, 2013. 225: p. 60-74.

[10]	Xu, H., et al. Load-shedding probabilities with hybrid renewable power generation and energy storage. in Communication, Control, and Computing (Allerton), 2010 48th Annual Allerton Conference on. 2010. IEEE.

[11]	Eyer, J. and G. Corey, Energy storage for the electricity grid: Benefits and market potential assessment guide. Sandia National Laboratories, 2010: p. 69-73.

[12]	Rastler, D., Electricity energy storage technology options: a white paper primer on applications, costs and benefits. 2010: Electric Power Research Institute.

[13]	Purchala, K., et al. Usefulness of DC power flow for active power flow analysis. in Power Engineering Society General Meeting, 2005. IEEE. 2005. IEEE.

[14]	Stott, B., J. Jardim, and O. Alsaç, DC power flow revisited. Power Systems, IEEE Transactions on, 2009. 24(3): p. 1290-1300.

[15]	Bai, X., et al., Semidefinite programming for optimal power flow problems. International Journal of Electrical Power & Energy Systems, 2008. 30(6): p. 383-392.

[16]	Bose, S., et al., Quadratically constrained quadratic programs on acyclic graphs with application to power flow. arXiv preprint arXiv:1203.5599, 2012.

[17]	Lavaei, J. and S.H. Low, Zero duality gap in optimal power flow problem. Power Systems, IEEE Transactions on, 2012. 27(1): p. 92-107.

[18]	Denholm, P. and R. Sioshansi, The value of compressed air energy storage with wind in transmission-constrained electric power systems. Energy Policy, 2009. 37(8): p. 3149-3158.

[19]	Kraning, M., et al. Operation and configuration of a storage portfolio via convex optimization. in Proceedings of the 18th IFAC World Congress. 2011. Citeseer.

[20]	Harsha, P. and M. Dahleh, Optimal management and sizing of energy storage under dynamic pricing for the efficient integration of renewable energy. Power Systems, IEEE Transactions on, 2015. 30(3): p. 1164-1181.

[21]	Su, H.-I. and A. El Gamal. Modeling and analysis of the role of fast-response energy storage in the smart grid. in Communication, Control, and Computing (Allerton), 2011 49th Annual Allerton Conference on. 2011. IEEE.

[22]	Kanoria, Y., et al. Distributed storage for intermittent energy sources: Control design and performance limits. in Communication, Control, and Computing (Allerton), 2011 49th Annual Allerton Conference on. 2011. IEEE.

[23]	Chandy, K.M., et al. A simple optimal power flow model with energy storage. in Decision and Control (CDC), 2010 49th IEEE Conference on. 2010. IEEE.

[24]	Koutsopoulos, I., V. Hatzi, and L. Tassiulas. Optimal energy storage control policies for the smart power grid. in Smart Grid Communications (SmartGridComm), 2011 IEEE International Conference on. 2011. IEEE.

[25]	Gayme, D. and U. Topcu, Optimal power flow with large-scale storage integration. Power Systems, IEEE Transactions on, 2013. 28(2): p. 709-717.

[26]	Kirschen, D.S., et al., Factoring the elasticity of demand in electricity prices. Power Systems, IEEE Transactions on, 2000. 15(2): p. 612-617.

[27]	Zhao, S. and Z. Ming. Modeling demand response under time-of-use pricing. in Power System Technology (POWERCON), 2014 International Conference on. 2014. IEEE.

[28]	Kashem, M., et al. A novel method for loss minimization in distribution networks. in Electric Utility Deregulation and Restructuring and Power Technologies, 2000. Proceedings. DRPT 2000. International Conference on. 2000. IEEE.

[29]	Wong, P., et al., The IEEE Reliability Test System-1996. A report prepared by the Reliability Test System Task Force of the Application of Probability Methods Subcommittee. Power Systems, IEEE Transactions on, 1999. 14(3): p. 1010-1020.

[30]	Carpinelli, G., et al. Optimal allocation of dispersed generators, capacitors and distributed energy storage systems in distribution networks. in Modern Electric Power Systems (MEPS), 2010 Proceedings of the International Symposium. 2010. IEEE.

[31]	Divya, K. and J. Østergaard, Battery energy storage technology for power systems—An overview. Electric Power Systems Research, 2009. 79(4): p. 511-520.