Application of Bat Optimization Algorithms for Optimal Power Flow

H. Delkhosh Abatari

PhD Student

Electrical Engineering

Sharif University of Technology

Tehran, Iran

Email: h.delkhosh@modares.ac.ir

M. Seydali Seyf Abad

PhD Student

Electrical Engineering

Tarbiat Modares University

Tehran, Iran

Email: seydali@ee.sharif.edu

H. Seifi

Professor

Electrical Engineering

Tarbiat Modares University

Tehran, Iran

Email: seifi_ho@modares.ac.ir

Abstract— In this paper, Bat Algorithm is proposed to solve the Optimal Power Flow (OPF) problem. The approach employs a Meta heuristic optimization algorithm called bat optimization algorithm to determine the optimal settings of control variables of OPF problem. The performance of proposed method examined on the IEEE 9, 14, 30 and 57-buses test systems with fuel cost minimization as the objective function. The numerical results are compared to those of Particle Swarm Optimization (PSO), Harmony Search (HS) and Cuckoo algorithms. Simulation results obtained from the proposed approach demonstrate that this method provide effective and robust high-quality answer for the OPF problem.

Keywords-component; Bat Algorithm (BA); Optimal Power Flow (OPF); Power System; Cost Minimization.

# Introduction

Optimal Power Flow (OPF) is one of the most important issues in power systems operation and planning. This problem is a constrained optimization problem which has an objective function and a number of equality and inequality constraints [1]. In OPF problem, the main goal is to find control variables of a given power system network in order to optimize a predetermined  objective function such as total generation cost, voltage deviation or system loss while satisfying system power flow equations, system inequality constraints and equipment operation limits [2, 3].The control variable include of the generators real power, the generator bus voltage, tap ratios of transformers and the reactive power generation of VAR sources [4, 5]. In general the OPF is a large-scale, constrained, nonlinear and non-convex optimization problem.

Many researches have been done to solve OPF problem. Plenty of traditional optimization techniques such as linear programming [6], nonlinear programming [7, 8], quadratic programming [9], Newton and interior point methods [10-12] have been used to solve OPF problem. In spite of excellent convergence characteristics of these methods, they have some drawbacks. The drawback of these methods is that it is not possible to use them in practical systems because they are developed with some theoretical assumptions and fail to deal with non-smooth, non-convex and non-differentiable objective functions and constraints. Therefore it is necessary to improve some methods that are capable of overcoming these drawbacks. In the past few years many researches have focused on this goal. Lately many population-based optimization methods such as genetic algorithm (GA) [13, 14] enhanced genetic algorithm (EGA) [15], tabu search (TS) [4], simulated annealing (SA) [16], evolutionary programming (EP) [17, 18], particle swarm optimization (PSO) [19, 20], differential evolution (DE) [21, 22] and gravitational search algorithm (GSA) [23] have been proposed to solve the OPF problem. These methods have many advantages such as implementation simplicity, lower computation load and faster convergence.

In [14], the GA with dynamical hierarchy of coding system for optimal power flow problem is employed. In [13], an enhanced GA is proposed to solve OPF problem and tested on the IEEE 30-bus, 73-bus and IEEE RTS-96 systems. In order to increase the convergence speed, Yuryevich and Wong [18], employed EP to solve OPF problem. In [24], Basu proposed DE to solve OPF problem with flexible alternating current transmission systems (FACTS) devices. In [25], Madhad employed parallel genetic algorithm to solve the OPF problem.

One of the recently improved heuristic algorithms is the bat algorithm (BA). In this paper, bat algorithm is used to solve the OPF problem considering both equality and inequality constraints in a power system. This algorithm has been developed based on the echolocation of micro-bats [26]. One of the most important advantages of BA is its lower computational load in comparison with other intelligent method. As a result, it can be implemented easier and faster which are considered as two of the most important features of an optimization method in real-time systems [27]. On the other hand, BA is a robust algorithm that can solve a vast range of optimization problems with different objective functions and a variety of linear and nonlinear constraints and equality and inequality constraints [28].

The proposed BA-based OPF optimization method is applied to the IEEE 9, 14, 30 and 57-bus test systems and the results are compared with the results obtained by PSO, HS and Cuckoo algorithms. The rest of the paper is organized as follows: Section II provides the mathematical formulation of the optimal power flow problem. The Bat Algorithm is described in the section III. Section IV, presents the proposed bat algorithm to solve the OPF problem. simulation results are presented in Section V. Finally, conclusion is drawn in section VI.

# Mathematical Formulation Of Optimal Power Flow Problem [29, 30]

The OPF is a nonlinear optimization problem which the main goal of it is to optimize the control variables in terms of one or more objective functions while satisfying equality and inequality constraints. In general, the OPF problem formulated as follows:

|  | (1) |
| --- | --- |

Subjected to

|  | (2) |
| --- | --- |
|  | (3) |

Where, f is the objective function, g is the equality constraints representing power flow equations and h is the system operating constraints.

x is the vector of dependent variables. The x vector consist of voltage magnitude of load buses, phase angle of all buses except that of the slack bus, active power of slack generator and generators reactive power. In the similar way, the control variable vector, u, includes the active output power of generators at PV buses (PG), terminal voltage magnitude at generation bus bars (VG), output of shunt VAR compensators (QC) and tap setting of the tap regulating transformers (T). Therefore, u can be modeled as in the following:

|  | (4) |
| --- | --- |

Where N, m and j denote the number of generators, shunt VAR compensators and regulating transformers, respectively.

## Objective function

In this paper, the goal is to minimize the generators’ fuel costs in the system. The total fuel cost function (Ft) for a number of thermal generating units can be represent by a quadratic function as follows:

|  | (5) |
| --- | --- |

Where, NG is the number of generating units and ai, bi and ci are the coefficient of fuel cost model.

## constraints

- equality constraints:

Equality constraints which is represented by g in (2) includes load flow equations as follows:

|  | (6) |
| --- | --- |
|  | (7) |

Where PGi and QGi are the active and reactive power of ith generator and PDi and QDi are the active and reactive demand of ith bus. Vi and Vj are the voltage magnitudes at bus i and j, respectively. Yij models the element of Ybus matrix at the row i and column j, and δi, δj and θij model the angels of Vi, Vj and Yij, respectively.

- Inequality constraints:

The inequality constraints (h) represented by (3) are the system operating limits as follows:

#### Generator constraints:

#### active and reactive power of generators and terminal voltage magnitude of generators are restricred by their lower and upper limits.

|  | (8) |
| --- | --- |
|  | (9) |
|  | (10) |

Where NG is the number of generator, PGimin and PGimax are the minimum and maximum active power of ith generator and QGimin and QGimax denote the minimum and maximum reactive power of ith generator, respectively.

- Security constraints:

These include the limits on the voltage magnitude of load buses and line flow:

|  | (11) |
| --- | --- |
|  | (12) |

Where, Sij and Sijmax are MVA flow and maximum amount of the MVA flow between two buses i and j, respectively.

# Bat Algorithm [1, 26-28]

## Bat algorithm

Bat algorithm is an optimization algorithm inspired by the echolocation of bats. This algorithm is introduced by Yang and is used for solving optimization problems [26, 27, 31, 32]. Bats perform echolocation for updating their position. Bat echolocation is a system in which a series of loud ultrasound waves are emitted to create echoes. These waves are returned with delays and different sound level which help bats to discover a particular hunt. In an echolocation, each pulse only lasts a few thousandths of second (up to about 8–10 ms). However, it has a constant frequency which is usually between 25 to 150 kHz corresponding to the wavelengths between 2 to 14 mm [30]. The typical range of frequencies for most bat species are in the range between 25 kHz and 100 kHz, though some of them can emit higher frequencies up to 150 kHz.

By idealization some of the echolocation characteristics of bats, various bat-inspired algorithm can be developed. The echolocation characteristics of bats can be idealized as the following rules:

- All bats use echolocation to sense distance and they also  know the difference  between  prey  and background barriers in some mysterious way;

- Bats randomly fly with velocity vi at position xi with a fixed frequency fmin, varying wavelength λ and loudness A0 to search for hunt. They can adjust the frequency of their emitted pulses and regulate the rate of pulse emission r[0,1], depending on the proximity of their target;

- Although the loudness can vary in many ways, it is assumed that the loudness varies from a large (positive) A0 to a minimum constant value Amin.

The basic steps of BA can be summarized as the pseudo- code shown in Figure 1.

Initialize the bat population position (xi) and velocity (vi)

Initialize frequencies fi, pulse rates ri and the loudness Ai

While (t <Max number of iterations)

Generate new solutions by adjusting frequency,

Update velocities and locations (solutions)

if (rand > ri)

Select a solution among the best solutions

Generate a local solution around the selected best solution

end if

Generate a new solution by flying randomly

if (rand < Ai & f(xi) < f(x∗))

Accept the new solutions

Increase ri and reduce Ai

end if

Rank the bats and find the current best x∗

end while

Figure 1. Pseudo code of the bat algorithm (BA)

For  each  bat i,  its  position  xi and  velocity  vi should be defined and updated during the optimization process. The new solutions  and velocities  at time step t can be calculated by following equations:

|  | (13) |
| --- | --- |
|  | (14) |
|  | (15) |

Where  is a random vector drawn from a uniform distribution in the range of [0, 1]. Here, x* is the current global best location (solution) among all the bats. As the product λifi is the velocity increment, either fi (or λi ) can be used to adjust the velocity change while fixing the other factor λi (or fi ), depending  on  the  type  of  the  problem  of  interest.  For implementation, fmin = 0 and fmax = 1 are used, depending on the domain size of the problem. Initially, a frequency that is randomly drawn uniformly from [fmin, fmax] is assigned to each bat. For the local search part, once a solution is selected among the current best solutions, a new solution for each bat is generated using a local random walk:

| xnew = xold + εAt | (16) |
| --- | --- |

Where ε is a random number drawn from [-1, 1], while At is the mean loudness of all the bats at this time step. The update of the velocities and positions of bats have some analogies to the process in the standard particle swarm optimization as fi essentially controls the pace and range of the movement of the particles. BA can be considered as a combination of the intensive local search controlled by the loudness and pulse rate and the standard particle swarm optimization [30]. Moreover the loudness (Ai) and the pulse emission rate (ri) have to be updated. Once a bat found its hunt, the loudness usually decreases while the rate of pulse emission increases. Therefore, the loudness can be chosen as any value. For simplicity, A0= 1 and Amin = 0 can be used. Assuming Amin = 0 means that a bat has just found the hunt and temporarily stop emitting any sound. Now we have:

|  | (17) |
| --- | --- |
|  | (18) |

Where  and  are constants in the range of zero to one. In fact,  is similar to the cooling factor of a cooling process in the simulated annealing [9]. For any 0<, <1:

|  | (19) |
| --- | --- |

In the simplest case,  =  can be used. For the simulations in this paper, it is assumed that  =  = 0.9.

It is worth noted that the bat algorithm is not just another meta-heuristic algorithm. Compared with existing meta-heuristics, it has two advantages: frequency tuning and dynamic control of exploration and exploitation by automatic switching to intensive exploitation if necessary. It uses the frequency-based tuning and pulse emission rate changes to mimic bats behavior, which leads to good convergence and simpler implementation compared to other algorithms. Moreover, the balance of exploration and exploitation is important; a simple fixed ratio of exploration to exploitation is not necessarily a good strategy. Bat algorithm uses a dynamic strategy for exploration and exploitation. Like the auto-homing of bats on their hunt, the variations in pulse emission rates and loudness control how exploration and exploitation are used. In fact, automatic switch from exploration to more extensive exploitation can be achieved when the optimal point is approaching; therefore, this algorithm can be very efficient in practice [27]. The following section is devoted to a discussion about handling of constraints.

- Constraint handling

From the implementation point of view, a major barrier is proper handling of nonlinear constraints for a given problem. For nonlinear equality constraints (φi) and inequality constraints (ψj), a common solution is the penalty method. The idea is to define a penalty function so that the constrained problem is transformed into an unconstrained problem. Now, the following equation can be defined:

|  | (20) |
| --- | --- |

Where  and  which should be large enough, depending on the solution quality needed. As it can be seen, when an equality constraint is met, its effect or contribution to П is zero. However, when it is violated, it significantly increases П by introducing a penalty. Similarly, it is true when inequality constraints are violated [26].

# Pproposed Method

For solving the OPF problem with the aforementioned algorithm, first a set of bats (in this application, the set of control parameters) as the initial population is defined. Then, for each of these bats, load flow is done. Then, the objective function for each of them is calculated. Later, the inequality constraints are checked and if these constraints for each of these bats are violated, its objective function is increased with a penalty. Finally, using these values, cost of all bats position are obtained. Following this process, the best bat position is obtained. The flowchart of BA for solving OPF problem is shown in Figure 2.

# Simulation Result

The proposed algorithm considers all constraints of bus voltages, transmission line flows, and limitations on active and reactive output power of generators. It has been applied to the standard IEEE 9, 14, 30 and 57-bus test systems. These systems have three, five, six and seven generators respectively.

In order to demonstrate the effectiveness of the proposed algorithm, it has been compared with PSO, HS and cuckoo algorithms. Figure 2 shows convergence curves of these algorithms for IEEE 30-bus system. It can be seen that proposed algorithm converges to a lower objective function value than other algorithms in a lower iteration number.

Figures 4, 5, 6 and 7 Shows IEEE 9, 14, 30 and 57-buses test systems, respectively. In Tables I, II, III and IV, the results of the proposed algorithm are compared with those of PSO, HS and Cuckoo algorithms. According to the Tables I, II, III and IV, the optimal cost obtained by the proposed algorithm is less than the others. Besides, it converges faster than the others and altogether, it is an appropriate alternative for the existing meta-heuristic methods for solving the OPF problem.

Figure 2. Flowchart of the bat algorithm (BA) for solving OPF

Figure 3. Convergence curves for IEEE 30-bus system

Figure 4. IEEE 9-bus test system

Results for the IEEE 9-bus system

|  | BA | PSO | HS | Cuckoo |
| --- | --- | --- | --- | --- |
| P1 | 0.8953 | 0.8987 | 0.9002 | 0.8968 |
| P2 | 1.3442 | 1.3412 | 1.3414 | 1.3445 |
| P3 | 0.9435 | 0.9432 | 0.9422 | 0.9416 |
| V1 | 1.1000 | 1.0986 | 1.0901 | 1.0987 |
| V2 | 1.0973 | 1.0972 | 1.0900 | 1.0972 |
| V3 | 1.0865 | 1.0858 | 1.0776 | 1.0862 |
| Cost | 5296.6138 | 5296.8423 | 5298.8212 | 5296.8100 |

Figure 5. IEEE 14-bus test system. [33]

Results for the IEEE 14-bus system

|  | BA | PSO | HS | Cuckoo |
| --- | --- | --- | --- | --- |
| P1 | 1.9559 | 1.9418 | 1.9209 | 1.9441 |
| P2 | 0.3709 | 0.3758 | 0.3551 | 0.3664 |
| P3 | 0.2902 | 0.1595 | 0.2085 | 0.2851 |
| P6 | 0.0011 | 0.0789 | 0.0499 | 0.0006 |
| P8 | 0.0658 | 0.1419 | 0.1491 | 0.0877 |
| V1 | 1.0600 | 1.0383 | 1.0584 | 1.0600 |
| V2 | 1.0443 | 1.0237 | 1.0420 | 1.0444 |
| V3 | 1.0184 | 1.0081 | 1.0150 | 1.0192 |
| V6 | 1.0600 | 1.0213 | 1.0549 | 1.0597 |
| V8 | 1.0600 | 0.9958 | 1.0479 | 1.0596 |
| Cost | 8081.392 | 8153.693 | 8098.315 | 8081.433 |

Results for the IEEE 30-bus system

|  | BA | PSO | HS | Cuckoo |
| --- | --- | --- | --- | --- |
| P1 | 0.4429 | 0.4422 | 0.4464 | 0.4353 |
| P2 | 0.5663 | 0.5621 | 0.5709 | 0.5878 |
| P13 | 0.2297 | 0.2419 | 0.2329 | 0.1753 |
| P22 | 0.3063 | 0.3189 | 0.3125 | 0.2306 |
| P23 | 0.1893 | 0.1754 | 0.1662 | 0.3178 |
| P27 | 0.1830 | 0.1823 | 0.1901 | 0.1707 |
| V1 | 1.0365 | 1.0078 | 1.0249 | 1.0315 |
| V2 | 1.0294 | 1.0124 | 1.0180 | 1.0281 |
| V13 | 1.0361 | 1.0173 | 1.0189 | 1.0451 |
| V22 | 1.0394 | 1.0189 | 1.0347 | 1.0189 |
| V23 | 1.0443 | 1.0176 | 1.0270 | 1.0375 |
| V27 | 1.0686 | 1.0237 | 1.0587 | 1.0287 |
| Cost | 575.164 | 578.869 | 578.201 | 575.263 |

Figure 6. IEEE 30-bus test system. [33]

Figure 7. IEEE 57-bus test system. [33]

Results for the IEEE 57-bus system

|  | BA | PSO | HS | Cuckoo |
| --- | --- | --- | --- | --- |
| P1 | 1.4583 | 1.8217 | 2.3573 | 1.4223 |
| P2 | 0.9566 | 0.4750 | 0.8513 | 0.8762 |
| P3 | 0.5108 | 0.7962 | 0.8096 | 0.4567 |
| P6 | 0.9928 | 0.7042 | 0.8084 | 0.7250 |
| P8 | 4.3667 | 4.8669 | 4.1520 | 4.5973 |
| P9 | 0.6571 | 0.5997 | 0.7945 | 0.9791 |
| P12 | 3.7226 | 3.5421 | 2.9517 | 3.6164 |
| V1 | 1.0091 | 1.0101 | 1.0213 | 1.0193 |
| V2 | 1.0136 | 0.9956 | 1.0266 | 1.0066 |
| V3 | 1.0050 | 1.0132 | 1.0120 | 1.0053 |
| V6 | 1.0196 | 1.0224 | 1.0210 | 1.0197 |
| V8 | 1.0328 | 1.0416 | 1.0263 | 1.0458 |
| V9 | 1.0062 | 0.9784 | 0.9955 | 1.0121 |
| V12 | 0.9885 | 0.9868 | 0.9780 | 0.9978 |
| Cost | 41785.70 | 43725.92 | 44108.96 | 41797.78 |

# Conclution

In this paper, one of the recently proposed heuristic algorithms which is the bat algorithm was expressed and applied to solve optimal power flow problem. The OPF problem is formulated as a nonlinear optimization problem with equality and inequality constraints in power systems. The BA approach were tested and investigated on the IEEE 9, 14, 30 and 57-bus test systems.

This approach was successfully and influentially performed to find the optimal settings of the control variables of test systems. The simulation results proved the effectiveness and superiority of the proposed approach to solve the OPF problem. Comparison of the simulation results obtained by PSO, HS and MATPOWER shows a better performance of the proposed algorithm over the other algorithms.

# References

[1]	A.J. Wood and B.F. Wollenberg, Power generation, operation, and control. 2012: John Wiley & Sons.

[2]	M. AlRashidi and M. El-Hawary, Applications of computational intelligence techniques for solving the revived optimal power flow problem. Electric Power Systems Research, 2009. 79(4): p. 694-702.

[3]	J. Zhu, Optimization of power system operation. 2015: John Wiley & Sons.

[4]	M. Abido, Optimal power flow using tabu search algorithm. Electric Power Components and Systems, 2002. 30(5): p. 469-483.

[5]	S. He, et al. An improved particle swarm optimization for optimal power flow. in Power System Technology, 2004. PowerCon 2004. 2004 International Conference on. 2004. IEEE.

[6]	R. Mota-Palomino and V. Quintana, Sparse reactive power scheduling by a penalty function-linear programming technique. Power Systems, IEEE Transactions on, 1986. 1(3): p. 31-39.

[7]	O. Alsac and B. Stott, Optimal load flow with steady-state security. Power Apparatus and Systems, IEEE Transactions on, 1974(3): p. 745-751.

[8]	R.R. Shoults and D. Sun, Optimal power flow based upon PQ decomposition. Power Apparatus and Systems, IEEE Transactions on, 1982(2): p. 397-405.

[9]	R. Burchett, H. Happ, and D. Vierath, Quadratically convergent optimal power flow. Power Apparatus and Systems, IEEE Transactions on, 1984(11): p. 3267-3275.

[10]	J.A. Momoh and J. Zhu, Improved interior point method for OPF problems. Power Systems, IEEE Transactions on, 1999. 14(3): p. 1114-1120.

[11]	H. Wei , et al., An interior point nonlinear programming for optimal power flow problems with a novel data structure. Power Systems, IEEE Transactions on, 1998. 13(3): p. 870-877.

[12]	X. Yan and V.H. Quintana, Improving an interior-point-based OPF by dynamic adjustments of step sizes and tolerances. Power Systems, IEEE Transactions on, 1999. 14(2): p. 709-717.

[13]	A.G. Bakirtzis, et al., Optimal power flow by enhanced genetic algorithm. Power Systems, IEEE Transactions on, 2002. 17(2): p. 229-236.

[14]	L.L. Lai, et al., Improved genetic algorithms for optimal power flow under both normal and contingent operation states. International Journal of Electrical Power & Energy Systems, 1997. 19(5): p. 287-292.

[15]	M.S. Kumari and S. Maheswarapu, Enhanced genetic algorithm based computation technique for multi-objective optimal power flow solution. International Journal of Electrical Power & Energy Systems, 2010. 32(6): p. 736-742.

[16]	C. Roa-Sepulveda and B. Pavez-Lazo, A solution to the optimal power flow using simulated annealing. International journal of electrical power & energy systems, 2003. 25(1): p. 47-57.

[17]	Y.R. Sood, Evolutionary programming based optimal power flow and its validation for deregulated power system analysis. International Journal of Electrical Power & Energy Systems, 2007. 29(1): p. 65-75.

[18]	J. Yuryevich and K.P. Wong, Evolutionary programming based optimal power flow algorithm. Power Systems, IEEE Transactions on, 1999. 14(4): p. 1245-1250.

[19]	M. Abido, Optimal power flow using particle swarm optimization. International Journal of Electrical Power & Energy Systems, 2002. 24(7): p. 563-571.

[20]	J.-Y. Kim, et al., Optimal power system operation using parallel processing system and PSO algorithm. International Journal of Electrical Power & Energy Systems, 2011. 33(8): p. 1457-1461.

[21]	A.A. El Ela, M. Abido, and S. Spea, Optimal power flow using differential evolution algorithm. Electric Power Systems Research, 2010. 80(7): p. 878-885.

[22]	M. Varadarajan and K. Swarup, Solving multi-objective optimal power flow using differential evolution. Generation, Transmission & Distribution, IET, 2008. 2(5): p. 720-730.

[23]	S. Duman, et al., Optimal power flow using gravitational search algorithm. Energy Conversion and Management, 2012. 59: p. 86-95.

[24]	M. Basu, Optimal power flow with FACTS devices using differential evolution. International Journal of Electrical Power & Energy Systems, 2008. 30(2): p. 150-156.

[25]	B. Mahdad, K. Srairi, and T. Bouktir, Optimal power flow for large-scale power system with shunt FACTS using efficient parallel GA. International Journal of Electrical Power & Energy Systems, 2010. 32(5): p. 507-517.

[26]	X.-S. Yang and A. Hossein Gandomi, Bat algorithm: a novel approach for global engineering optimization. Engineering Computations, 2012. 29(5): p. 464-483.

[27]	X.-S. Yang, A new metaheuristic bat-inspired algorithm, in Nature inspired cooperative strategies for optimization (NICSO 2010). 2010, Springer. p. 65-74.

[28]	X.-S. Yang and X. He, Bat algorithm: literature review and applications. International Journal of Bio-Inspired Computation, 2013. 5(3): p. 141-149.

[29]	B. Wollenberg and A. Wood, Power generation, operation and control. John Wiley&Sons, Inc, 1996: p. 264-327.

[30]	X.-S. Yang, Nature-inspired optimization algorithms. 2014: Elsevier.

[31]	X.-S. Yang, Bat algorithm for multi-objective optimisation. International Journal of Bio-Inspired Computation, 2011. 3(5): p. 267-274.

[32]	T.C. Bora, L.d.S. Coelho, and L. Lebensztajn, Bat-inspired optimization approach for the brushless DC wheel motor problem. Magnetics, IEEE Transactions on, 2012. 48(2): p. 947-950. 32.

[33]          www.ee.washington.edu/research/pstca/, 2015.