Wide-beam Spiral Antennas with Multi-Folded Arms and Compact Feed Networks for Satellite Application

M. Fartookzadeh

Department of Electrical and Electronics Engineering,

Malek Ashtar University,

P. O. Box 1774-15875, Tehran, Iran

Email: Mahdi.fartookzadeh@gmail.com

S. H. Mohseni Armaki

Department of Electrical and Electronics Engineering,

Malek Ashtar University,

P. O. Box 1774-15875, Tehran, Iran

Email: mohseni@ee.iust.ac.ir

Abstract—This paper proposes a new class of multi-fed antennas with folded arms. A compact planar antenna is proposed for wide-beam applications with circular polarization. The fabricated antenna has three multi-folded spiral arms fed by equal signals with 120° out of phase with respect to each other to produce a symmetrical radiation pattern. It is indicated that the number of arms can be changed to say, four or six arms without changing the results. In addition, compact feed networks with Wilkinson power dividers are designed to produce the required phase differences for the inputs of antennas. Finally some discussions are presented for the quadrifilar spiral antennas and a dual-mode antenna with reduced height is achieved.

Keywords-spiral antennas; multi-fed antennas; wide-beam antennas

# Introduction

Wide-beam antennas with circular polarization (CP) are the requirement of miniaturized-satellites. Miniaturization of these antennas is a well-studied topic since total size of the satellite is small and the antenna size reduction reduces the total size significantly. Resonant quadrifilar helical antennas (QHAs) are the most investigated structure for this purpose. Meanwhile many configurations are introduced for QHAs to reduce the size of antenna such as QHAs with meandered or folded arms, conical QHAs, conical QHAs with folded arms, etc. [1-4].

Quadrifilar spiral antennas are also applied for this purpose [5]. This paper employs the folded arms for spiral antennas. The spiral antennas with modified arms have been studied in previous works [6, 7]. Also the folded arms as for the QHAs are used for tri-fed spiral antennas in [8]. The spiral antenna with three folded arms is studied to provide a compact substitution for QHAs. The method of folding is selected between several structures to obtain the widest beam-with and bandwidth in a smallest place. Total diameter of the antenna is 24 cm and the height is about 5 cm. It is obvious that the size of this antenna is more proper than QHAs for small satellites since it has very smaller height, however small beam-width reduction is unavoidable.

In addition, three arms are used for the antenna to require a feed network with smallest size. The feed network consists of only a compact three-way Wilkinson power divider (PD) and the additional microstrip lines. Total size of the feed network is smaller than previous reports such as [1, 4]. The antenna and feed network are designed and studied for 435MHz and simulation results are validated by measurement.

In this paper, similar structures are introduced for quadrifilar and hexa-fed spiral antennas. In addition, the effect of changing inner radii of the spirals is studied. It is indicated that the bandwidths can be controlled by changing the inner radii of the antennas. Schematics of the feed networks for tri-fed, quadrifilar and hexa-fed spiral antennas are then presented using compact meandered lines. Moreover, some discussions are conveyed about the quadrifilar spiral antennas. It is indicated that the beam-width of antennas increase by increasing the antenna height.

# Multi-fed Spiral Antennas

The schematic of proposed antenna with three arms on a substrate with ϵr=2.55 is shown in Fig. 1 (a). In the top view we observe six left handed spirals with about 1.2 numbers of turns and a right handed spiral with about 0.3 numbers of turns. The inner ends of neighboring pairs of left handed spirals are connected to form a folded spiral. One outer end of the folded arms is connected to the right handed spiral and the other is used for the feeding. The inner radius has direct effects on bandwidth of the antenna and the outer radius has some effects on the beam-width. These sizes are selected for the highest beamwidth and bandwidth on the limited space. In should be noted that the beamwidth can be improved by increasing the height of antenna. However the small size of the antenna is the most important purpose of the design. The lengths of each three arms of the antenna are about 1.6λ0 at 435MHz. The side view indicates the probes those are isolated from the ground plane and connected to the feed network. The total structure is also comparable with the triple circular array of circular sector patches in [9]. However, the serial feed could not be employed here and the feed network is designed separately.

The schematic of proposed antennas with four and eight arms on a substrate with ϵr=2.55 is shown in Fig. 1 (b), as well. It will be observed that the number of arms does not have significant effects on the performance. The performance is dependent on the sizes of the antennas such as heights and inner radii. For example, reflection coefficients of the antennas with the sizes, [R2=116.5, Rp=101, R1=19, h=50], [R2=116.5, Rp=95, R1=8, h=50] and [R2=116.5, Rp=90, R1=14, h=50] respectively for tri-fed, quadrifilar and hexa-fed antennas are indicated in Fig. 2. It can be observed that the bandwidths are different. The bandwidth difference is due to the different inner radii of the antennas. As an example, the effects of changing inner radii on the reflection coefficients are indicated for the tri-fed spiral antenna in Fig. 3.

(a)

(b)

Schematics of proposed antennas; Top view and side view of the spiral antennas with (a) three, (b) four and six folded arms (Dimensions in mm).

Reflections and ARs of the spiral antennas with (a) three, (b) four and (c) six folded arms.

Effects of changing the inner radii on the reflections and AR of the tri-fed spiral antenna.

(a)

(b)

(c)

Radiation patterns of the spiral antennas with (a) three, (b) four and (c) six folded arms.

Radiation patterns for the three antennas in Fig. 1 are indicated in Fig. 4. It can be observed that the LHCP pattern does not have major differences since the total sizes are similar. However, some distinctions are recognizable for the backward RHCP patterns. Major distinctions are larger backward RHCP gain of the hexa-fed antenna and the rotation of the RHCP beam angle for the tri-fed antenna.

# Feed Networks and Measured Results

## Feed Networks

The feed network for tri-fed antenna contains a compact three-way Wilkinson PD. The PD has three resistors with triangular configuration as indicated in Fig. 5. It is obvious that the triangular configuration cannot be constructed on a single layer circuit. Therefore the third resistor is placed on a ground bridge [10] under the inner output line as shown in Fig. 6. The resistors and the impedance of λeff/4 lines of three-way Wilkinson PD with triangular configuration should be 150Ω for the 50Ω output lines. However we observed that 130Ω resistors produce better results for this structure. Also the widths of λeff/4 lines are tuned for the best power balance between output ports. The microstrip lines of outputs have proper lengths to provide 120° phase difference between them. The effective wavelength of a microstrip line is obtained by λeff=λ00.5ϵr+1, where λ0=69 cm at 435MHz and  the relative permittivity of the substrate is ϵr=3.9. Therefore we have λeff=44cm.

Feed network of the quadrifilar antenna should divide the input power into four equal parts with 90° phase difference between each two adjacent outputs [4]. Simple Wilkinson power dividers and transmission lines can be used for power division and 90° phase shifting respectively. Employing meandered lines can reduce the size of feed network, while compressed structures are preferred for satellite application. Consequently, the networks is designed for this antenna using three Wilkinson power dividers, a half-wave line and two quarter-wave lines to produce 90° phase difference for the antenna inputs. The schematic of this network is shown in Fig. 7, while the resistors and the impedance of thin lines of the power dividers are 100Ω and impedance of thick lines are 50Ω.

Finally the feed network for hexa-fed antennas can be constructed using combination of a three-way Wilkinson power divider and three two-way power dividers as indicated in Fig. 8. However this network could be formed using a two-way power divider and two three-way Wilkinson power dividers similarly. The lengths of lines are adjusted for the 60° phase difference between successive outputs.

Circuit model of the three-way Wilkinson power divider with triangular arrangement of the resistors.

(a)

(b)

Schematics of (a) the feed network for tri-fed antennas and (b) the triangular arrangement of the resistors with ground bridge.

Schematics of the feed network for quadrifilar antennas.

Schematics of the feed network for hexa-fed antennas.

## Measured results of the tri-fed spiral antenna

Photograph of the fabricated tri-fed antenna is shown in Fig. 9. The height of antenna is about 5 cm. Some pieces of foam are placed in the separation between antenna and ground plane to prevent movements of the antenna substrate.

Reflection coefficients of the antenna are given in Fig. 10. The bandwidth (BW) of antenna is about 4.7 % in simulation. It is reduced to 3.5 % in measurement, however if we accept the -8.8 dB reflections the measured BW will be larger than the simulated BW. Simulated and measured LHCP and RHCP gains of the antenna are indicated in Fig. 11. The successive input signals of the antenna are assumed to have 120° phase difference in simulation. This phase difference is obtained by using the fabricated feed network for measurement.  The beamwidth of higher than 3 dB Gain is about 99°, the maximum gain is 6.5 dB and the axial ratio (AR) at broadside is 0.07 dB. This antenna is suitable when we have restrictions on the height of antenna.

Photograph of the fabricated tri-fed antenna.

Simulated and measured reflections and ARs of the tri-fed spiral antennas.

# Discussions on Quadrifilar Spiral Antennas

The heights of proposed antennas are assumed about 5 cm for all situations. If we increase the heights the beam-widths will be increased. For example, the radiation patterns of similar quadrifilar spiral antennas with 10 cm, 17 cm and 21 cm heights are indicated in Fig. 12. The structures are scaled to maintain the resonant frequencies at 435 MHz. It can be observed further than increasing the LHCP beam-widths that the backward RHCP gain increases by increasing the antenna height. Now if we compare the patterns with radiation pattern of the quadrifilar monopole antenna in Fig. 13, we observe that that the spiral antennas have smaller backward gains and larger LHCP gains. The monopoles should be of 17 cm length to have a resonant frequency at 435 MHz.

An interesting advantage of the quadrifilar monopole antenna is that the shapes of antennas are similar for LHCP and RHCP forward radiations. Only the phases of inputs should be change for polarization conversion. Therefore the quadrifilar monopole antenna is a dual-mode antenna. However, the height of this antenna is 17 cm for 435 MHz, which may be too large for some applications. Therefore we introduce another dual-mode quadrifilar antenna based on the spiral shape to produce similar LHCP and RHCP radiation patterns, when the input phases are changed. Schematic of the proposed dual-mode antenna is indicated in Fig. 14 (a) and the reflection and AR are indicated in Fig. 14 (b). The height of antenna is 5 cm, which can be arranged due to size restrictions and beam-width requirement, as explained. LHCP and RHCP radiation patterns, for the first mode and second mode excitations in Fig. 14 (a), are indicated in Fig. 15 (a) and 15 (b) respectively.  It can be observed that the patterns are similar and only the polarizations are different.

(a)

(b)

Radiation patterns of the tri-fed spiral antennas; (a) simulated results and (b) measured results.

(a)

(b)

(c)

Radiation patterns of the quadrifilar spiral antennas with increased heights, (a) h=100 mm, (b) h=170 mm and (c) h=210 mm.

Radiation pattern of the quadrifilar monopole antenna with h=170 mm.

(a)

(b)

(a) Schematic of the dual-mode quadrifilar spiral antenna (Second mode fields are indicated by outlined text), (b) reflection coefficient and AR of the dual mode antenna.

(a)

(b)

Radiation patterns of the dual-mode antenna; (a) first mode radiation pattern, (b) second mode radiation pattern.

# Conclusions

A spiral antenna with three folded arms is proposed as a compact substitution for the QHAs for satellite application. In addition a very compact feed network is designed for the antenna and the required 120° phase differences with similar amplitudes are obtained successfully. The total design is fabricated and simulations are validated by measured results. In addition, it is indicated that the number of arms can be changed to say, four or six arms without changing the results. Moreover, compact feed networks with Wilkinson power dividers are also designed to produce the required phase differences for the inputs of the antennas with four or six arms. Moreover, some discussions are presented for the quadrifilar spiral antennas and the effect of changing antenna height is discussed. Finally, a dual-mode antenna with reduced height to 5 cm is achieved that has similar results with quadrifilar monopole antenna of 17 cm.

##### References

P. Xu, Z. Yan, X. Yang, and T. Zhang, “A Folded Printed Quadrifilar Helical Antenna Employing a Compact Feeding Network”, Progress In Electromagnetics Research Letters, vol. 45, pp. 57-61, 2014

M. Caillet, M. Clénet, A. Sharaiha, Y. M. Antar, “A broadband folded printed quadrifilar helical antenna employing a novel compact planar feeding circuit,” IEEE Trans. Antennas Propag., vol. 58, no. 7, pp. 2203-2209, 2010

S. Yang, S. H. Tan, Y. B. Gan, C. W. See, “Broadband conical printed quadrifilar helical antenna with integrated feed network,” Microw. Opt. Technol. Lett., 35(6), 491-493, 2002.‏

M. Fartookzadeh and S. H. Mohseni Armaki, “Multi-band Conical and Inverted Conical Printed Quadrifilar Helical Antennas with Compact Feed Networks”, AEU, articles in press.

Colburn J. S. and Y. Rahmat-Samii, ‘Quadrifilar-Curl Antenna for the “Big-LEO” mobile satellite service system’, IEEE Antennas and Propagation Society, AP-S International Symposium, Baltimore, 1996.

Newham, P. ‘Broadband spiral and sinuous antennas.’ European patent application EP1026777A3 (2000): 1-4.

Filipović, D. S. and J. L. Volakis. ‘Broadband meanderline slot spiral antenna.’ IEE Proceedings-Microwaves, Antennas and Propagation 149.2 (2002): 98-105.

M. Fartookzadeh, S. H. Mohseni Armaki, “Wide-beam spiral antenna with three folded arms fed by compact three-way Wilkinson power divider”, submitted to electronic letters.

M. Fartookzadeh, S. H. Mohseni Armaki, “Serial-Feed for a Circular Patch Antenna with Circular Polarization suitable for Arrays”, International Journal of RF and Microwave Computer‐Aided Engineering 24.5 (2014): 529-535.

M. Fartookzadeh, S. H. Mohseni Armaki, and M. Kazerooni, "A Novel 180° Hybrid Based on the Modified Gysel Power Divider." Progress In Electromagnetics Research C, vol. 27, pp. 209-222, 2012