A New Comparison Algorithm for General            4-Moduli Set RNS

Tara Ejlali

Faculty of Electrical Engineering,

Shahid Beheshti University

Tehran,Iran

taraejlali@gmail.com

Mohammad Eshghi

Faculty of Electrical Engineering,

Shahid Beheshti University

Tehran,Iran

m-eshghi@sbu.ac.ir

Abstract—Residue Number System (RNS) has many applications in computer arithmetic because of its properties such as parallelism and modularity. Comparison is one of difficult operations in RNS and improving its algorithms has an important role in the use of this system for general purpose applications. In this paper a new algorithm for comparing two residue numbers in general 4-moduli sets is proposed. First, the CRT-II reverse conversion algorithm is improved with modulo decomposition and a weighted representation is obtained. Based on this new representation, there is a unique weighted set for each number. With these weighted sets comparison is carried out similar to other weighted number systems. Comparing with previous algorithms the proposed design needs smaller modulo operations that reduces the delay. This algorithm neither assumes special conditions on the moduli set nor needs redundant modulus.

Keywords-Chinese reminder theorem (CRT), comparison, modulo decomposition, residue number system (RNS)

# I. Introduction

Residue number system (RNS) is an unconventional number system that is used in arithmetic computation and Digital Signal Processing (DSP) applications such as image processing, fast Fourier transform and digital filtering. Modularity and parallelism are the properties of RNS that limit carry propagation to computation of each modulus. This limitation reduces the delay and results in efficient VLSI implementation [1] , [2]. Although addition, subtraction and multiplication are performed very fast in this number system, some other functions such as comparison, division, overflow detection and sign test are complex and difficult. So RNS is not used in general purpose computations [7] , [8].

Because of non-position nature of the RNS, comparison of residue numbers is one of difficult operations in this system. The traditional techniques for comparing numbers use the Chinese reminder theorem (CRT) or the mixed radix conversion (MRC). Implementation of CRT is inefficient because it needs modulo M operation where M is the dynamic rage and commonly a large number. On the other hand MRC doesn’t need modulo M operation but it is a sequential process and has complex computations which increases the delay [2] , [5] , [6]. Another technique of comparison is based on CRT-II which reduces the modulo operation to M. A parity checking technique for comparison is presented in [4] that needs redundant modulus and assumes special conditions on the moduli set. Another technique, which is called diagonal method uses a function for comparing residue numbers [5]. In [6] one of the most efficient algorithms is presented that  a weighted set is calculated for each number and comparison is carried out like other weighted number systems.

In this paper for general 4-moduli set {P1,P2,P3,P4} at first residue to binary conversion (reverse conversion) based on CRT-II is improved. Then a new residue number comparison algorithm is proposed that limits the modulo operations to P3. This algorithm doesn’t assume any special conditions on the moduli set and no redundant modulus is needed.

We organize the remaining sections as follows. In section II background of RNS, reverse converters and comparison based on CRT-II, and modulo decomposition are introduced. In section III the proposed algorithm is presented. In section IV we compare our algorithm with previous designs and finally the conclusion of this paper is in section V.

# Background

In this section, the definition of RNS is in the first part. Then reverse conversion and comparison based on CRT-II are introduced in parts B and C, respectively. Modulo decomposition technique is in the last part of the section.

## RNS

For defining a residue number system first a moduli set P={P1,P2,…,Pn} is needed that the members are relatively prime (GCD(Pi,Pj)). A binary number X can be represented as X=(x1,x2,…,xn) in this system where xi=X mod pi .

M= P1P2…Pn is the dynamic range of P and defines the numbers of integers which have unique representation for this moduli set. There are many algorithms for converting residue number to its binary representation and one of them is  CRT-II which is appeared as follows.

## Reverse conversion algorithm based on CRT-II for general 4-moduli set

Based on CRT-II [2], it can be proved that in 4-moduli set P={P1,P2,P3,P4} for obtaining the binary representation of residue number N=(n1,n2,n3,n4), first the binary representations of N1=(n1,n2) for 2-moduli set {P1,P2} and N2=(n3,n4) and for 2-moduli set {P3,P4} is calculated in parallel :

| N1=n2+P2P2-1P1(n1-n2)P1 , M1 =P1P2 |  | (1) |
| --- | --- | --- |
| N2=n4+P4P4-1P3(n3-n4)P3 , M2 =P3P4 | (2) |  |

Where Pi-1Pj is the multiplicative inverse of Pi modulus Pj  (Pi-1Pj*Pi=1 mod Pj ). Now N=(N1,N2) for new moduli set (M1,M2) and the previous algorithm is applied to the new representation for calculating reverse conversion of N as equation (3):

| N=N2+P3P4(P3P4)-1P1P2(N1-N2)P1P2 | (3) |
| --- | --- |

As N2<P3P4 a weighted representation of N is shown in equation (3). In fact, the weight of N2 is 1 and P3P4 is the weight of (P3P4)-1P1P2(N1-N2)P1P2. This representatin can be used in comparison of numbers.

## Residue comparison based on CRT-II for general 4-moduli set

As mentioned in [2] for comparing two residue numbers X=(x1,x2,x3,x4) and Y=(y1,y2,y3,y4) in 4-moduli set (P1,P2,P3,P4) where P1<P2<P3<P4, first the binary representations for X1=(x1,x2) and Y1=(y1,y2) in 2-moduli set {P1,P2} and X2=(x3,x4) and Y2=(y3,y4) in 2-moduli set {P3,P4} is calculated. After that two functions SX=k'(X1-X2)P1P2 and SY=k'(Y1-Y2)P1P2 are computed where k' is multiplicative inverse of P3P4 modulus P1P2. Now comparison of X and Y  can be carried out like other weighted number system. If Sx>Sy then X>Y and if Sy>Sx then Y<X . If Sx=Sy and  X2>Y2 then  X>Y. If Sx=Sy and  Y2>X2 then  Y>X. Finally if Sx=Sy  and  X2=Y2 then X=Y.

The size of the modulo operation in this technique is bounded by P1P2 which is less than M but this modulus is still big.

## Modulo decomposition

In [3] a modulo decomposition technique was presented. If the modulo operation base is multiply of P1 and P2, then it can be written as:

| KP1P2=KP1+P1KP1P2 | (4) |
| --- | --- |

The equation (4) illustrates that modulo P1P2 operation can be calculated by two smaller modulo operations (P1 and P2). These values can be obtained in parallel and result in high speed implementation. At the right part of the equation (4) a weighted representation is shown. The weight of KP1P2 is bigger than KP1 .

# Proposed algorithm of comparing two residue numbers for general 4-moduli sets

In this section, first CRT-II reverse conversion algorithm is improved in part A; then based on the new reverse conversion, a new comparison algorithm is introduced in part B.

## New reverse conversion algorithm for general 4-moduli sets

As mentioned, for obtaining binary representation of residue number N=(n1,n2,n3,n4) in 4-moduli set P={P1,P2,P3,P4} based on CRT-II,  modulo P1P2 operation is needed. Although this modulo is less than M, it is still big and this problem can be solved with modulo decomposition of equation (4).

N=N2+P3P4k'N1-N2P1P2 =N2+P3P4k'N1-N2P1+P1k'N1-N2P1P2 =N2+P3P4k'N1-N2P1+P3P4P1k'N1-N2P1P2 = c0+ c1P3P4+c2P3P4P1 (5)

Where   is the floor function and k' is multiplicative inverse of P3P4 modulus P1P2 , then :

c0=N2

c1= k'(N1-N2)P1

c2=k'(N1-N2)P1P2

So we just need modulo P1 and P2 operations instead of P1P2 which reduces the delay. As N2<P3P4 and                    N2+P3P4k'N1-N2P1<P3P4P1, a new weighted representation of N is obtained that c2 is the most significant and c0 is the less significant digits of N. So after calculating, N1 and N2, we have a weighted set for N as W(N)=(c0,c1,c2) that the weights are (1,P3P4,P3P4P1). This new weighted set can be used for comparing numbers.

Example 1 : The moduli set is {5,7,9,11} for computing binary representation of N=123=(3,4,6,2) we have :

P2k1P1=1

P4k2P3=1

P4P3k3P2P1=1

K1=3 , k2=5 , k3=29

X1=4+ 7×3×(3-4)5=18

X2=2+ 11×5×(6-2)9= 24

c0=24

c1=29×(18-24)5=1 ,

c2=29×(18-24)57=0

N=24+99×1+495×0=123

In this reverse conversion method the biggest modulo operation is 9 which is smaller than 35 in CRT-II and can result in reducing the delay.

## New comparison alghorithm for general 4-moduli sets

Comparison of two residue numbers X=(x1,x2,x3,x4) and Y=(y1,y2,y3,y4) for general 4-moduli set (P1,P2,P3,P4)  where   P1<P2<P3<P4 is carried out by algorithm 1.

- )  Find k1 , k2 and k3 form  k1P2p1= 1,

k2P4p3=1 and k3P3P4P1P2=1 equations, respectively,

- )  Calculate X1 and X2 :

X1 = x2 + P2 k1(x1-x2)P1

X2 = x4 + P4 k2(x3-x4)P3 ,

- )  Calculate Y1 and Y2 :

Y1 = x2 + P2 k1(y1-y2)P1 ,

Y2 = x4 + P4 k2(y3-y4)P3 ,

- )  Calculate W(X)=(a0,a1,a2) such that

a2=[k3(X1-X2)P1P2  ,

a1=k3(X1-X2)P1   ,

a0 = X2

- )   Calculate W(Y)=(b0,b1,b2) such that

b2=[k3(Y1-Y2)P1P2 ,

b1=k3(Y1-Y2)P1 ,

b0 = Y2

- ) Compare W(X) and W(Y) like two weighted                                                                                                                                                                              numbers

If a2 > b2            then   X > Y

else if a2 < b2     then   X < Y ,    
              else if a2= b2      then
               if a1 > b1           then   X > Y ,

else if a1 < b1    then   X < Y ,

else  if a1=b1     then

If  a0>b0            then   X>Y ,

else if  a0<b0     then   X<Y ,

Else X=Y.

Algorithm I. Comparison of two residue numbers in general 4-moduli sets

After calculating binary representations of X1,X2,Y1 and Y2 in parallel, a 3-digits weighted set for each number (W(X)=(a0,a1,a2) and W(Y)=(b0,b1,b2) ) is computed. Then like weighted number system we compare the digits of the new sets. As a2 and b2 are the most significant digits of W(X) and W(Y) we start comparison form them. If a2=b2 we need to compare a1 and b1 and if a2=b2 and a1=b1 the less significant digits (a0 and b0) should be compared. It can be seen that the computations are bounded by P3 and there is no need for modulo P1P2 operations. Following example illustrates the use of algorithm1.

Example 2 : The moduli set is {5,7,9,11} for comparing X=123=(3,4,6,2) and Y=40={0,5,4,7} which are in the dynamic range we acts as follows :

- P2k1P1=1

P4k2P3=1

P4P3k3P2P1=1

K1=3 , k2=5 , k3=29

- X1=4+ 7×3×(3-4)5=18

X2=2+ 11×5×(6-2)9= 24

- Y1=5+ 7×3×(0-5)5=5

Y2=7+ 11×5×(4-7)9= 40

- Calculate W(X)=(a0,a1,a2) such  that              a2=29×(18-24)57=0

a1=29×(18-24)5=1

a0=24

- Calculate W(Y)=(b0,b1,b2) such  that

b2=29×(5-40)57=0

b1=29×(5-40)5=0

b0=40

- Compare W(X)=(24,1,0) and W(Y)=(40,0,0) like two weighted numbers

a2 = b2 & a1 > b1  then

X > Y

# comparing  proposed algorithm with previous comparison methods

Table I summarized the evaluation of the proposed residue comparison algorithm with previous designs. Complexity refers to the largest modulo operations or the biggest integer that is existed in the operations. In the third column of table I the complexity of previous algorithms for example2 is computed. In the proposed comparison algorithm the biggest modulo operation is P3 which is much smaller than dynamic

Table I .  Evaluation of proposed residue comparison algorithm and previous designs

| Redundunt moduli | Special conditions on moduli set | Complexity for example2 | Complexity | Comparison algorithms |
| --- | --- | --- | --- | --- |
| No | No | Modulo(35) | Modulo( MAX(P3 , P1P2) ) | [2] |
| Yes | Yes | 14-bit | Log(nm)-bit | [4] |
| No | No | Modulo(1888) | Modulo(S (M/pi)) | [5] |
| No | No | Modulo(11) | Modulo( P4 ) | [6] |
| No | No | Modulo(9) | Modulo( P3 ) | proposed |

range M in CRT. In comparison algorithm based on CRT-II the biggest modulo operation is MAX(P3 , P1P2) which  commonly equals to P1P2. On the other hand in CRT-II, calculating Sx and Sy needs modulo P1P2 operations which are reduced to P1 and P2 in our algorithm and reduces the delay.

Authors in [6] presented one of the best algorithms for comparison that its weighted set has 4 digits and its biggest modulo operation is modulo P4. Our algorithm reduces both the biggest modulo operation and the number of digits in weighted set. As shown in table I, the proposed algorithm is the best in complexity among other algorithms. Further, all the modulus of the moduli set for comparison algorithm in [4] should be odd and redundant moduli is needed for parity checking. Our comparison design does not need any redundant modulus and special conditions for the moduli sets. It is clear that our algorithm is superior to the other comparison algorithms for general 4-moduli sets based on the criteria’s in table I.

# Conclusion

Residue number system is an unconventional number system that addition, multiplication and subtraction are carried out in parallel and modularly. But some operations such as comparison and division are difficult and complex. In this paper a new algorithm for comparing two residue numbers in general 4-moduli sets was proposed. First, the CRT-II reverse conversion algorithm for general 4-moduli sets was improved using modulo decomposition. There was a new weighted representation for each number and the modulo operations were limited to P3. Base on that new representation, a weighted set was obtained for each number. By using those new weighted sets, comparing two residue numbers was carried out similar to comparison in other weighted number systems. As it was shown in table I, smaller modulo operations were needed for our comparison algorithm comparing to previous designs. This property reduced the delay. Our algorithm neither assumed special conditions on the moduli sets nor needed redundant modulus.

# references

Navi, Keivan, Amir Sabbagh Molahosseini, and Mohammad Esmaeildoust. "How to teach residue number system to computer scientists and engineers." Education, IEEE Transactions on 54.1 , pp. 156-163, 2011.

Y.Wang, X. Song, M. Aboulhamid, and H. Shen, “A new algorithm for RNS   magnitude comparison based on new Chinese remainder theorem II,” in Proc. 9th Great Lakes Symp. VLSI, pp. 362–365, 1999.

Shaoqiang Bi, Wei Wang, and Asim Al-Khalil "New modulo decomposed residue-to-binary algorithm for general moduli sets." Acoustics, Speech, and Signal Processing, 2004. Proceedings .(ICASSP'04). IEEE International Conference on. Vol. 5. IEEE, 2004.‏

Lu, Mi, and Jen-Shiun Chiang. "A novel division algorithm for the residue number system." Computers, IEEE Transactions on 41.8 , pp.1026-1032,1992.‏

Dimauro, Giovanni, Sebastiano Impedovo, and Giuseppe Pirlo. "A new technique for fast number comparison in the residue number system."Computers, IEEE Transactions on 42.5, pp.608-612, 1993.

Shaoqiang Bi, and Warren J. Gross. "The mixed-radix Chinese remainder theorem and its applications to residue comparison." Computers, IEEE Transactions on 57.12 , pp.1624-1632, 2008.

Labafniya, Mansoureh, and Mohammad Eshghi. "Non-iterative RNS Division Algorithm." Proceedings of the International MultiConference of Engineers and Computer Scientists. Vol. 1. 2012‏.

Labafniya, Mansoureh, and Mohammad Eshghi. "RNS division algorithm for 2 n-1 and 2 n dividers." Electrical Engineering (ICEE), 2014 22nd Iranian Conference on. IEEE, 2014.‏