Ebrahim Abbaspour Sani

Associate professor

Microelectronics Research Laboratory Urmia

Urmia, Iran

Email: e.abbaspour@urmia.ac.ir

Javad Shahavand

Postgraduate

Microelectronics Research Laboratory Urmia

Urmia, Iran

Email: javad_shahavand@yahoo.com

Farshad Farahi Bolamiri

Postgraduate

Microelectronics Research Laboratory Urmia

Urmia, Iran

Email: farshadblr@yahoo.com

Design and Simulation of a 3-axis Capacitive Accelerometer with Novel Electrode Structure in MEMS Technology

Abstract— This paper reports the design and simulation of a three axis capacitive accelerometer with novel electrodes structure in surface micromachine technology. The proposed accelerometer utilizes three individual capacitance groups to measure acceleration along three axis. Also the accelerometer utilizes improved interdigitated in-plane comb electrodes to detect in-plane acceleration. The electrodes are differential and both variations in overlapping area and air gap between fixed and movable electrodes are used simultaneously to increase output sensitivity. The simulation results show that the sensitivity (nonlinear) along X-axis is 23.2fF/G (2.9%), along Y-axis is the same and along Z-axis is 13.22fF/G (10%). The proposed accelerometer utilizes only a single proof mass to sense acceleration in each three axis which caused small occupied area. The total occupied area of the accelerometer is 1.2mm×1.2mm. Considering the limitations of Urmia Micromachine Research Center, a verified fabrication process has been proposed.

Keywords— MEMS accelerometer; surface micromachine technology; 3-axis accelerometer

# Introduction

Compared to piezoelectric and thermal accelerometers, capacitive accelerometers are more popular for applications on automotive and consumer electronics due to their high sensitivity, good temperature performance, low cost and easy integration with CMOS [1]. However capacitive sensors have some disadvantages such as parasitic capacitances and low resolution. The use of surface micromachining technology in implementing capacitive accelerometers has various advantages such as easily achieved good mechanical stability and simple fabrication processes.

Interdigitated in-plane comb electrodes are mostly used to detect in-plane acceleration. For out-of-plane accelerometer, it remains a challenge to realize Z-axis differential capacitive electrodes for monolithic integration of three-axis accelerometer [1]. Matsumoto et al. demonstrated an SOI three-axis capacitive accelerometer, in which the whole handle layer was used as the bottom electrode of the Z accelerometer, but the handle layer also formed an unwanted capacitor with the in-plane comb fingers, which caused large parasitic capacitance.  Furthermore, the differential capacitive sensing was not available in their Z-axis accelerometer [2]. Hsu et al. fabricated a silicon-on glass (SOG) three-axis accelerometer, in which the authors integrated in-plane and out-of-plane accelerometer into a single sensing structure to achieve small size, and made the unbalanced proof mass as the outer frame of the in-plane accelerometers to enhance the output of Z-axis sensing [3]. Vertical comb electrodes, which are composed of high and low comb fingers,

were utilized to detect Z-axis acceleration on SOI wafer with differential capacitive scheme by Tsuchiya and Funabashi and Hamaguchi et al. [4] [5]. But in their three-axis accelerometers, both the out-of-plane and in-plane motions were detected by the same group of vertical comb electrodes, so the sensitivity must be decoupled for each of the three axes, and consequently the capacitive sensitivities for all the three axes were reduced. Xie et al. fabricated a three-axis accelerometer on SOI wafer that utilize both in-plane and vertical comb electrodes [1]. XY acceleration is detected by in-plane comb electrodes, while Z acceleration is detected by the vertical comb electrodes with differential capacitive sensing scheme and thus high sensitivity.

In this paper, we present a three-axis accelerometer with improved in-plan electrodes in surface micromachining. Mostly, one of the variations in overlapping area and air gap between fixed and movable electrodes are used to measure acceleration in capacitive accelerometer. While in proposed in-plane electrodes structure both variations in overlapping area and air gap between fixed and movable electrodes are used simultaneously to measure XY acceleration which cause an increment in output sensitivity. By using differential electrodes structure in capacitive accelerometer, output linearity and sensitivity will be increased and also the effects of noise will be decreased.  Also in the proposed accelerometer, the proof mass is used as upper electrode to detect acceleration along z direction.

# Accelerometer Structure

The structure of proposed accelerometer is shown in Fig. 1. As shown in this figure, the accelerometer structure consists of a simple movable cross shaped proof mass which is surrounded by three capacitance groups in four quadrants. In Fig. 1-(b), fixed electrodes and movable electrodes connected to the movable mass are shown with red and blue colors, respectively. Capacitances belong to first and third quarters only take part in acceleration measurement along x-direction. While capacitances belong to second and fourth quarters only take part in acceleration measurement along y-direction. As well upper and bottom cross shaped electrodes are used together as fifth capacitance group to measure acceleration along z-direction.

As shown in Fig. 2, capacitance variations in each quarters are done by simultaneous variations in overlapping area and air gap of electrodes, whereas variations in fifth capacitance group is done by only air gap variation. Differential electrodes improve sensitivity and output linearity of the accelerometer. In the proposed accelerometer only acceleration measurement along x and y directions are differential.

(a)

(b)

Figure 1. General structure of proposed 3-axis accelerometer with (a) 3D view with structure of electrodes and springs (b) 2D view for more clarity

As shown in Fig. 1, Accelerometer structure consists of some grooves that increase number of electrodes and sensitivity and also decrease damping effect. As well make it easy to etch sacrificial layer to release movable proof mass. Movable proof mass suspended by four folded springs to achieve verified in-plane and out-of-plane displacement.

# Theoty

As shown in Fig. 2, there are fixed electrodes of 1 and 2 with movable electrodes in first and third quarter that form differential capacitance pair of C1 and C2 that participate in acceleration measurement along x direction. Also there are fixed electrodes of 3 and 4 with movable electrodes in second and fourth quarter that form differential capacitance pair of C3 and C4 that participate in acceleration measurement along y direction. Fifth capacitance group participated in out-of-plane acceleration detection, consists of fixed bottom cross shaped with movable upper cross shaped electrode that forms capacitance of C5. Fixed electrodes in each of three capacitance groups formed by same material and there is no electrical connection between them.

By attention to Fig. 2 (a) and (b), when acceleration is applied along x-direction (y-direction), movable electrode comes close to fixed electrode of 1 (3) and simultaneously keeps out from fixed electrode of 2 (4) . In this case, both variations between air gap and overlapping area of fixed and movable electrodes will be occurred.

It must be said that capacitance groups in each quarters has the role of in-plane acceleration measurement and by attention to Fig. 2, due to differential structure of these electrodes, they are not able to detect out-of-plane acceleration and only fifth capacitance group participates in out-of-plane acceleration measurement.  When proof mass has the in-plane movement, because the fixed bottom cross shaped electrode is considered wider than movable proof mass (see Fig. 2(b)), there is no capacitance variation in fifth capacitance group.

## In-plane acceleration measurement

For a capacitor plate, the capacitance and its change against small displacement are written as:

| C=ε0lwd, ∆C=ε0wd∆l+ε0ld∆w-ε0lwd2∆d | (1) |
| --- | --- |

Where, ε0ε0 is vacuum permittivity, and l, w and d are length, width and air gap of electrodes, respectively.

When acceleration is applied along x-direction, only capacitances of C1 and C2 belong to first and third quarters will be varied. The capacitance variations of C1 and C2 based on the assumption that acceleration is applied to ±x-direction are derived as:

| ∆C1=-∆C2=±kε0ld∆x±kε0lhd2(∆x) | (2) |
| --- | --- |

Where, k is the number of movable electrodes in first and third quarters. Because of differential structure of electrodes, output capacitance is derived by subtracting C2 from C1.

∆CX=∆C1-∆C2=±2kε0l(1d+hd2)∆x                             (3)

Similarly, when acceleration is applied along  y-direction, only differential capacitance pairs of C3 and C4 belong to second and fourth quarters will be varied and output capacitance is derived in same way as x-direction.

∆CY=∆C3-∆C4=±2kε0l(1d+hd2)∆y                             (4)

Where, k is the number of movable electrodes in second and fourth quarters.

## Out-of-plane acceleration measurement

When acceleration is applied along z-direction, only capacitance of C5 will varied. In this case, only air gap between fixed bottom cross shaped and movable upper cross shaped electrode will be varied.

∆Cz=∆C5=±ε0A5d52∆z                                                      (5)

Figure 2. cross view of electrodes structure of first up to fourth capacitance quadrant when (a) acceleration is not applied, (b) acceleration is applied to x(y)-direction, with (c) top view of  cross shaped fifth capacitance group in which fixed and movable electrodes are indicated with blue and red colors respectively

As indicated in the Equations of (3), (4) and (5), 
and only rely on the displacements of ∆x, ∆y and ∆z, so there is ideally no cross-sensitivity between each two axes.

By attention to above equations, overlapping area variation is done only between movable and caps of fixed electrodes (see Fig.2). So, to remove cross-access sensitivity between capacitance groups, overlapping area variation between movable and side walls of fixed electrode must be zero. Thus, the length of fixed electrodes must be considered smaller than movable electrodes (see Fig. 1(b)).

# Fabrication Process

The fabrication process of the MEMS accelerometer is proposed with considering limitations and equipment of the Urmia Micromachine Research Center. The fabrication process is proposed by surface micromachining technology because of its enough sensitivity and accuracy to implement the size of the structure dimensions. The reason of utilizing Nickel for movable parts is that it has a high Young's modulus (160Gpa) and low hysteresis comparing to other metals. The fabrication process is proposed as following:

- Firstly wafer is oxidized and 100/2500 Å-thick Cr/Au is evaporated as the seed layer for Nickel electroplating. 3000 Å-thick SixNy is deposited as the isolation layer between bottom and upper cross shaped electrode by PECVD (plasma enhanced chemical vapor deposition). The process is followed by patterning the layers with RIE (reactive ion etching) (Fig.3 (a)).

Figure 3. Fabrication process of proposed accelerometer (a) Cr-Au sputtering and patterning, (b) Spin photoresist and Nickel electroplating, (c) Photo resist deposition and patterning as sacrificial layer, (d) Nickel is electroplated to create movable and part of fixed electrodes, (e) Nickel electroplating is continued to complete fixed electrodes, (f) finally, Removing sacrificial layer and releasing proof mass

- In the next step, depositing and patterning of photoresist is followed by Nickel electroplating as a part of fixed electrodes (Fig.3 (b)).

- Photoresist is removed by acetone and then Al as sacrificial layer is deposited by evaporating. Lithography is utilized to prepare Al surface for Nickel electroplating. Then deposition and lithography of photoresist is used to create sacrificial layer (Fig.3 (c) (

- Nickel is electroplated to create movable electrodes and a part of the fixed electrodes and then Spin photoresist is used as sacrificial layer to make difference in height between fixed and movable electrodes which is followed by lithography process (Fig.3 (d)).

- Again Nickel is electroplated to complete fixed electrodes. Then Au is deposited and patterned to prepare Nickel electroplating process (Fig. 3 (e)).

- Nickel is electroplated to create upper caps of fixed electrodes and complete the structure of fixed electrodes.

- The last step is to remove the sacrificial photoresist and Aluminum and releasing the structure. To prevent surface tension and sticking, plasma ashing procedure is used to remove sacrificial photoresist. Al dry etching is then used to remove sacrificial Al. Some holes are created in cross shaped electrode to release the movable mass easily (Fig. 3 (f)).

# Simulation Results

Simulations are done by using COMSOL Multiphisycs software by finite element approach. Fig. 4 shows output sensitivity of simulated three-axis accelerometer. The simulation results show that the sensitivity along X-axis is 23.2fF/G, along Y-axis is the same and along Z-axis is 13.22fF/G up to ±2g acceleration. The nonlinearity for each axis as percentage of full scale are 2.9% F.S and 10% F.S respectively.

Table I shows the value of cross-axis sensitivity in different directions while acceleration is applied to a given direction. Also the design parameters and simulated results are summarized in Table II. The mechanical design consists of four folded suspension of serpentine format attached to rectangular plates that provide in-plane and out-of-plane movement. The entire structure is anchored to the substrate at the ends of the folded suspensions and the end section of each spring was constrained in all degrees of freedom (FOD). In this table, maximum stress refers to stress applied to springs of accelerometer. However, applied stresses are very smaller than Young's modulus (160Gpa) of Nickel so accelerometer can endures several g acceleration easily.

Table I. Cross-axis sensitivity for each sensing axis

| Sensing Axis | Cross-axis (%) |  |  |
| --- | --- | --- | --- |
|  | X | Y | Z |
| X | - | 0.51 | 0.33 |
| Y | 0.51 | - | 0.35 |
| Z | 0.56 | 0.56 | - |

(a)

Figure 4. Output sensitivity of simulated three-axis accelerometer, for (a) x-axis (y-axis is the same) and (b) z-axis.

(b)

Table II.  Design parameters and simulated results of the accelerometer

| Parameter | X and Y axes | Z axis |
| --- | --- | --- |
| Spring constant (N/m) | 0.95 | 1.9 |
| Resonant frequency (KHz) | 0.75 | 1 |
| Movable electrode height (µm) | 5 |  |
| Movable Electrode length (µm) | 500 |  |
| Electrode gap (µm) | 2 |  |
| No. of movable electrodes | 44 | 1 |
| Capacitance sensitivity (fF/g) | 23.2 | 13.22 |
| Max. stress during ±2g acceleration (MPa) | 6.13 | 9.46 |

# Concluson

We have reported a 3-axis MEMS accelerometer with novel electrodes structure. The fabrication process of accelerometer is presented by surface micromachining technology. The proposed accelerometer utilizes three individual capacitance groups to measure acceleration along three axis. Also the accelerometer utilizes improved interdigitated in-plane comb electrodes to detect in-plane acceleration. The electrodes are differential and both variations in overlapping area and air gap between fixed and movable electrodes are used simultaneously to increase output sensitivity. Simulation results show large enough sensitivity across three axes while sensing area is small enough. The structure of accelerometer is symmetric with respect to the center that results in same measurement in the value along X and Y axis.

# REFRENCES

[1]  Xie J, Agarwal R, Liu Y, Tsai JM (2012) A three-axis accelerometer sensing with bot in-plane and vertical comb electrodes. Microsyst Technol 18:325–332 DOI 10.1007/s00542-011-1393-9

[2]   Matsumoto Y, Nishimura M, Matsuura M, Ishida M (1999) Three axis SOI capacitive accelerometer with PLL C–V converter. Sens Actuators A 75:77–85

[3]   Hsu YW, Chen JY, Chien HT, Chen S, Lin ST, Liao LP (2010) New capacitive low-g triaxial accelerometer with low cross-axis sensitivity. J Micromech Microeng 20:055019

[4]   Tsuchiya T, Funabashi H (2004) A z-axis differential capacitive SOI accelerometer with vertical comb electrodes. Sens Actuators A 116:378–383

[5]  Hamaguchi H, Sugano K, Tsuchiya T, Tabata O (2007) A differential capacitive three-axis SOI accelerometer using vertical comb electrodes. In: Digest of Technical Papers of 14th International Conference on Solid State Sensors, Actuators and Microsystems (TRANSDUCERS), Lyon, pp 1483–1486