Learning Automaton based  Adaptive Access Barring for M2M Communications in LTE Network

Faezeh Morvari and Abdorasoul Ghasemi

Faculty of Computer Engineering

K.N. Toosi University of Technology

Tehran, Iran

morvari@ee.kntu.ac.ir, arghasemi@kntu.ac.ir

Abstract— Massive access of machine-to-machine (M2M) devices over LTE network as infrastructure is a challenging problem due to the possible overload in the radio access network (RAN). The access class barring (ACB) scheme is an efficient and simple scheme which is proposed in the 3GPP documents to relieve the massive access by barring some M2M devices in each contention cycle. In this paper, we propose a learning automaton (LA) based algorithm to adaptively and dynamically adjust the ACB factor at evolved Node B (eNB) taking into account the number of collided preambles in the previous contention cycle. We show that by using an appropriate LA at the eNB, the system performance asymptotically converges to the optimal performance in which the eNB knows the number of access-attempting devices a priori.  Simulation results are provided to show the performance of the proposed approach in adjusting the barring factor properly.

Keywords- Machine-to-machine communication; Access class barring; Learning automaton; RAN overload;

# Introduction

Machine-to-machine (M2M) or machine type communication (MTC) can be defined as ubiquitous and fully automated communication among M2M devices, and is expected to be a suitable infrastructure for emerging new communication paradigms, such as smart cities and internet of things (IoTs). This type of communication consists of a large number of M2M devices and enables a wide range of applications such as smart grids, city automation, and infrastructure management [1], [2].

Currently, cellular system and in particular, the third generation partnership project (3GPP) long term evolution (LTE) network is considered as a suitable candidate to support of M2M communication, because of their capillary geographical coverage, lower service cost, and high speed data rate. However, the cellular network is basically designed and optimized for human-to-human (H2H) communication without considering the requirements and in particular, the traffic characteristics of M2M communication [1], [3]. M2M communication has distinct characteristics of H2H communication where a large number of devices, each with only a small amount of data for transmission, attempt to access the network simultaneously. Therefore, supporting MTC in the LTE infrastructure raises a number of challenges [2].

Simultaneous trying by a huge amount of MTC devices for acquiring uplink (UL) channel access in LTE, may cause the radio access network (RAN) overloaded. In this case, RAN is collapsed and almost no device being granted access. So far, several methods have been proposed in order to reduce the RAN overload problem [4].

In 3GPP LTE release 11, i.e., LTE-A system, several approaches to relieve the RAN overload such as separate resource allocation for M2M and H2H communication, access class barring (ACB) scheme, and MTC-specific back off scheme have been proposed [2]. Among the proposed solution the ACB scheme has attracted more attention and noted as a key solution [5]. In this scheme the evolved Node B (eNB) broadcasts a barring factor and each device attempts to transmit its access request with this probability. In addition to the ACB, 3GPP also proposed extended access barring (EAB) scheme. In EAB, eNB considers 16 access classes and in the case of RAN overload only specific classes which belong to high priority applications are allowed to participate in random access (RA) procedure while other classes become barred [2]. Authors in [5] proposed a heuristic algorithm for adaptive adjustment of the ACB factor and derived an analytical model for determining the total required time for serving all M2M devices. In [6], the authors proposed a scheme which jointly utilizes from timing advance information and ACB to prevent the RAN overload problem caused by M2M devices. Authors in [7] presented a prioritized random access mechanism that pre-allocates resources to different MTC classes according to their priorities. Furthermore, this mechanism prevents from a large number of concurrent random accesses by dynamic access barring (DAB). In [9] the authors proposed a random access protocol which makes use of learning automaton (LA) in WDM passive star networks. The transmission probability for each wavelength is determined dynamically using LA in the proposed protocol. Furthermore, the application of LA for Ad-Hoc wireless networks is discussed in [10].

In this paper, we propose a LA based RA scheme for dynamically adjusting the transmission probability of M2M devices in the ACB scheme and also, we investigate the asymptotic behaviour of the system for the proposed approach.

The rest of this paper is organized as follows. In section II         we explain an overview of the RA procedure and the functionalities of LA. The proposed learning automaton random access (LARA) scheme is presented in section III. Simulation results are presented in section IV and the paper is concluded in section V.

# Back Grounds and preliminaries

## Random Access Procedure in LTE Network

When user equipment (UE) needs radio resources to transmit its access request to the eNB, it initiates the RA procedure. The UE performs the RA procedure by sending a randomly selected preamble among specific orthogonal preambles to the eNB. UEs are only allowed to transmit the selected preamble in the specific time-frequency radio resources of physical random access channel (PRACH), which are called RA slots. At the LTE system, the RA procedure can be categorized to contention based and contention free procedure. The contention based RA procedure is completed by the following steps:

1) The UE randomly selects a RA preamble among a set of RA preambles allocated to the contention based RA procedure and sends it to the eNB, through the next available RA slots of the PRACH.

2) In response to each detected preamble, the eNB sends a random access response (RAR) which includes a random access preamble identifier (ID), a temporary cell identifier (C-RNTI), a time alignment (TA) command, and an UL grant (for transmitting the third message of RA procedure). The RAR is transmitted through physical downlink shared channel (PDSCH).

3) If the UE receives a RAR corresponding to the transmitted preamble in step 1, it sends the connection setup request message in the initial UL grant that specified in the received RAR. In the case of undetected preamble collision, all of M2M devices which send the same preamble will receive the same RAR.

4) If eNB successfully decodes the connection setup request message, it will send the contention resolution message to the UE. The RA procedure is completed when the UE receives contention resolution message. Otherwise, the UE assumes that failed and attempts for a new RA procedure from step 1.

The collision occurs in the case of one preamble is selected by more than UEs in the same RA slot. The reason is that in this situation more than one UE transmit their third messages through the same UL grant and the eNB cannot decode the received data in this UL grant. UE that do not receive the contention resolution message should retry in the next RA slots [3, 8].

## Learning Automata

LA is a self-operating mechanism which has been found to be useful in the unknown and random environments. The LA model consists of three main components including the automaton, the environment and the feedback structure. a continuously performs actions in the random

Fig. 1. Uplink channel structure

The automaton continuously performs actions in the random environment and receives feedbacks from it. The received feedbacks show how much the selected action is favourable or unfavourable. The automaton updates the selection probability of possible actions in the environment, according to the received feedbacks. In other words, the automaton utilizes from these feedbacks in order to improve its performance. The goal of a LA is to learn the specifications of the environment and to find the optimal actions which can be performed in the dynamic environment [10], [11].

# System model and LARA scheme

## System Model

We consider a system where N M2M devices exist in the coverage area of an eNB. All these devices are activated and each of them has only one access request within the interval [0, Ttc]. In order to maintain the burst feature of M2M traffic appropriately, we consider that the activation rate of these devices follows a beta distribution according to (1).

gt=tα-1(Ttc-t)β-1Ttcα+β-1β(α,β),                           (1)

where βα,β is a beta function and also, α and β determine the symmetry of the distribution which has been suggested to be set as 3 and 4, respectively [2]. As illustrated in Fig. 1, the activation interval is divided into x time slots. The first part of each time slot is allocated to the PRACH that is used by devices for transmitting preamble sequences and the second part is used for transmitting another uplink messages. Each activated device tries to transmit its request at the beginning of each time slot and remains active until the transmission of its access request completed successfully. The total time required to serve all devices, i.e., the time in which all devices in a traffic burst send their access requests to the eNB successfully, is called total service time (TST).

Let M be the number of preambles from available preambles for contention based RA procedure, which is reserved for M2M devices. The eNB calculates the power delay profile of each preamble and if this value is greater than a threshold, the preamble is considered as a selected preamble. Since the selection of preambles by devices is uniformly random, we can divide the set of preambles into three groups: 1) Successful preambles: preambles which are selected by only one device. 2) Idle preambles: preambles which are not selected by any device. 3) Collided preambles: preambles which are selected by more than one device. We assume that for a collided preamble the eNB cannot decode any of the transmitted third messages and all of devices corresponding to the collided preamble must retry in the next time slots [5].

When a large number of devices participate in the RA procedure, most of the preambles are selected by more than one device and collision occurs. Requests from the failed devices in the current slot are added to the new arrivals in the next slot and lead to a higher amount of requests. As a result, the PRACH is overloaded and almost no device being granted access. As previously mentioned, in the ACB scheme to control this overload the eNB broadcasts a probability p before the start of each time slot which is called ACB factor. The active device which attempts to send its access request uniformly generates a random number between 0 and 1 and compares it by ACB factor. If the generated random number is less than the ACB factor, the device is allowed to choose preamble and otherwise the device repeats the ACB check in the next time slot. In fact, this mechanism aims to relive the RAN congestion by reducing the number of devices participating in the RA procedure. Since the number of devices that have an access request varies in each time slot, the best situation is achieved when the ACB factor adaptively changed in each time slot according to the number of active or access-attempting devices. According to [5] the optimal ACB factor in kth time slot is obtained from (2).

pk*=min1 , MNk,                              (2)

where Nk denotes the number of active M2M devices at the start of the kth time slot. As is clear from (2), to determine the optimal ACB factor, eNB needs to know the number of active devices in each time slot which is not typically available and should be inferred. In the following we use a LA-based scheme to adjust the ACB factor according to the number of collided preambles.

## LARA scheme

The available information in the eNB consists of the status of the preambles in each time slot. At the end of each time slot, eNB determines the number of successful, collided, and idle preambles. Recall that the number of active M2M devices in the kth time slot is Nk.The probability that preamble m is selected by only one device, i.e., to be a successful preamble obtained from (3).

pksucc=NkpkM(1-pkM)Nk-1,                     (3)

where pk is the broadcasted ACB factor by eNB in the start of kth  time slot.The probability that preamble m remains idle is

pkidle=(1-pkM)Nk.                           (4)

Since each preamble is in one of the mentioned three states, the probability that preamble m is a collided preamble is given by (5).

pkcoll=1-NkpkM1-pkMNk-1-1-pkMNk.           (5)

As mentioned before, the optimal ACB factor for the kth time slot is given by (2). In the case that the ACB factor is adjusted by the optimal value, the probability that a preamble is successful, idle, and collided preamble is given by e-1, e-1 and 1-2e-1, respectively. If the ACB factor is adjusted by a value greater than the optimal value, the number of collisions would be greater than the optimal case since the number of contending devices in the RA procedure is increased. On the other hand, if the ACB factor is adjusted with a value less than the optimal one, the number of idle preambles would be greater than the optimal case. In both cases, the probability of successful transmission is less than the expected optimal value. Therefore, it is important that we properly adjust the ACB factor with optimal value with the aim of maximizing the successful transmission probability. In this way, we propose a LARA scheme to determine the ACB factor dynamically. We consider a LA which is deployed in the eNB. The LA determines the ACB factor in each time slot according to the available information in the eNB. In other words, LA uses from available information in the eNB as feedback for updating the ACB factor. If the percentage of collisions in the last time slot is less or equal than the expected optimal case, the ACB factor should be increased for the next slot. On the other hand, if the percentage of collisions is greater than the expected optimal case, the ACB factor should be decreased. Therefore, by considering v=1-2e-1, the ACB factor updating procedure should be as in (6).

pk+1=pk+∆1                      if pkcoll≤v pk-∆2                       if  pkcoll>v               (6)

where0<∆1<1- pkand 0<∆2< pk. Now, we should choose the suitable value for ∆1 and ∆2 such that the ACB factor asymptotically converges to the optimal value. By taking derivative from (5) with respect to pk, we find that pkcollis a monotonically decreasing function ofpkand the optimal value of pkcollwill be achieved by increasing or decreasing the value of pk, i.e., the ACB factor. Also, as mentioned before if pk is adjusted by optimal value, we should have pkcoll=v. It is clear that the unknown optimal value of pk corresponds to a known value of v. Therefore, it is possible that we determine the values of ∆1 and ∆2 such that the ACB factor converges to optimal value. According to (6) we have

δpk=Epk+1-pk=(1-pkcoll)∆1-(pkcoll)∆2=          ∆1-pkcoll∆1-pkcoll∆2=∆1-pkcoll∆2+∆1=

∆2+∆1(-pkcoll+∆1∆2+∆1).                                                   (7)

In fact, the probability of increasing the ACB factor is 1-pkcoll and the probability of decreasing the ACB factor is equal to pkcoll. In order to convergence to the optimal case, i.e., pkcoll=v, the ACB factor updating procedure must satisfy the following conditions.

- If pkcoll<v then δpk>0 and therefore δpkcoll>0.

- If pkcoll>v then δpk<0 and therefore δpkcoll<0.

- If pkcoll=v then δpk=0 and therefore δpkcoll=0.

According to (7) all these conditions are satisfied when  ∆1∆2+∆1=v. Therefore, we have

∆1=v1-v∆2=d∆2 ,

d=v1-v=0.359.                              (8)

By considering ∆2=∆  the updating procedure becomes

pk+1=pk+d∆             if pkcoll≤vpk-∆               if pkcoll>v                   (9)

where 0<∆<1-pkd  and 0<∆<pk. We adjust  ∆ as follows

∆=Lpk1-pk,         where       L∈(0,1).           (10)

L is a parameter which determines the convergence speed and also the estimation accuracy of the automaton. The lower the value of L leads to more accurate estimation made by the automaton but with slower convergence speed. In sum the ACB updating procedure is given by (11).

pk+1=pk+dLpk1-pk                    if pkcoll≤v  pk-Lpk1-pk                     if pkcoll>v      (11)

In this procedure, the step size of updating is very small for values of pk near to 0 or 1. Therefore, there is a problem in the speed of convergence and causes degradation in the performance of the network. To eliminate this inefficiency, the ACB updating procedure should be modified as follows [9].

pk+1=pk+dLpk1-pk+aL21-pk2    if pkcoll≤v  pk-Lpk1-pkt-bL2pk2       if pkcoll>v                   (                                                                                           (12)

where L∈0,1 and a,b∈0,1L.

## Asymptotic Analysis

As mentioned earlier, we consider pkcoll which is a function of pk, as feedback in ACB updating procedure. Also, we discuss that the value of pkcoll is equal to v if pk is adjusted with optimal value. In this section, we prove that the transmission probability of M2M devices i.e., pk asymptotically converges to take its optimal value by using the ACB updating procedure in (12). Proof of this is done by using the following theorem from [9].

Let x(t)n≥0 be a stationary Markov process dependent on a constant parameter θϵ [0,1]. Each x(t) ϵI, where I is a subset of the real line. Let δxt=xt+1-x(t). The following are assumed to hold:

- I is compact.

- Eδxtxt=y=θωy+O(θ2).

- E|δxt|2xt=y=θ2by+Oθ2.

Where         supO(θk)θk<∞ and supO(θ2)θ2→0  as θ→0.

- ωy has a Lipschitz derivative in I.

- b(y) is continuous in I.

Theorem1: If assumptions (i)–(v) hold, ωy has a unique root y* in I and dω/dy|y=y*<0.

To apply the above theorem in our problem, identify x(t) with pk, θ with L and I with (0,1). According to (12) we have

Eδpkpk=L1+dpk1-pk(1-pkcoll)-(1-v)+La(1-pkcoll)1-pk2-bpkcollpk2=Lω(pk).

and

E|δpk|2pk=L2pk1-pk2(1+(1-pkcoll)d2-1)+OL2=L2bpk+OL2.

According to [9] assumptions (i)–(v) are satisfied for small values of L and also ωpk has a unique root near the optimal point. In the case of one preamble and Nk active devices, the transmission probability asymptotically converges to pk'=1Nk which is equal to the optimal value (pk'≈pk*). Therefore, when there are M preambles, for small values of L the transmission probability, i.e., the ACB factor converges to the optimal value (pk'≈pk*) where pk*=MNk.

# Performance evaluation

In this section, we show the performance of the proposed LARA scheme for adjusting the ACB factor by comparing it with the situation that eNB knows the number of active M2M devices in each time slot and is able to adjust the ACB factor with optimal value. Also, we present the performance of the LARA scheme in terms of required TST to serve all MTC devices for different number of preambles against the fixed number of devices. In our simulation, we set Xtc=100 and the duration of each time slot to 5ms. Also, based on the simulation results we set the LA parameters including L, a, and b by 0.3, 0.1 and 0.99.

In Fig. 2 the variations of the ACB factor in different time slots is shown for LARA scheme in comparison with the optimal case in which the eNB knows the number of active devices in each time slot. In this simulation the number of active devices and the number of preambles are set by 5000

Fig. 2. The value of ACB factor vs time slots when N=5000 and M=40.

Fig. 3. The number of successful access requests vs time slots in TST interval when N=5000 and M=40.

and 40, respectively. The result shows that the proposed learning scheme can successfully follow the optimal decision.

Fig. 3 shows the number of successful access requests in the TST interval for the proposed LARA scheme and for the optimal case. The number of active M2M devices is 5000 and the number of preambles is equal to 40. The presented results are the average of 30 runs in the simulation.  As can be seen the number of successful access requests in LARA scheme is close to the optimal value.

In Fig. 4 the TST for different number of preambles varying from 10 to 45, when the number of devices is 1000 are depicted for both schemes. The results show that the performance of LARA scheme is very close to optimal case. In sum, according to these simulations we find that by designing appropriate LA at the eNB of each cell in the LTE network we are able to efficiently avoid the overload by proper adjustment of the ACB factor.

# Conclusion

In this paper we study the RAN overload problem in the RAN of the LTE networks due to massive access and proposed a LA based approach for adjusting the ACB factor

Fig. 4. TST vs the number of preambles with N=1000 and M=10-45.

dynamically according to the network condition and received feedbacks. We investigate the asymptotic behavior of the system which converges to the optimal case in which the eNB knows the number of access-attempting devices in each time slot. Simulation results show that the proposed LA based scheme can follow the optimal decision successfully.

# References

L. Ferdouse, A. Anpalagan, and S. Misra. “Congestion and overload control techniques in massive M2M systems: a survey,” Trans. Emerging Tel. Tech. (2015), doi: 10.1002/ett.2936.

3GPPTS 37.868, “Study on RAN Improvements for Machine-type Communications,” v11.0.0, Sep. 2011.

F. Hussain, A. Anpalagan, and R. Vannithamby. “Medium access control techniques in M2M communication: survey and critical review,” Trans. Emerging Tel. Tech.  (2014), doi: 10.1002/ett.2869.

G. C. Madueno, N. K. Pratas, C. Stefanovic, P. Popovski, “Massive M2M Access with Reliability Guarantees in LTE Systems,”  arXiv preprint arXiv:1502.02850 (2015).‏

S. Duan, V. Shah-Mansouri, and V.W.S. Wong, “Dynamic access class barring for M2M communications in LTE networks,” in Proc. of IEEE GLOBECOM, Altlanta, GA, Dec. 2013.

Z. Wang and V. W. S. Wong,  “Joint access class barring and timing advance model for machine-type communications,”  Proc. IEEE ICC,  pp.2357 -2362 2014.

L. Tzu-Ming, L. Chia-Han, C. Jen-Po, and C. Wen-Tsuen, “PRADA: prioritized random access with dynamic access barring for MTC in 3GPP LTE-A networks,” Vehicular Technology, IEEE Transactions on, vol. 63,  no. 5, pp. 2467-2472 2014.

A. Laya, L. Alonso and J. Alonso-Zarate  “Is the random access channel of LTE and LTE-A suitable for M2M communications? A survey of alternatives,”  IEEE Commun. Surveys Tuts.,  vol. 16,  no. 1,  pp.4 -16 2014.

G. I. Papadimitriou and D. G. Maritsas,  “Self-adaptive random-access protocols for WDM passive star networks,”  IEE Proc.-Comput. Digit. Tech.,  vol. 142,  no. 4,  pp.306 -312 1995.

P. Nicopolitidis, G. I. Papadimitriou and A. S. Pomportsis,  “Distributed protocols for Ad-Hoc wireless LANs: A learning-automata-based approach,”  Ad Hoc Netw.,  vol. 2,  no. 4,  pp.419 -431 2004.

K. S. Narendra, and M. A. L. Thathachar, Learning automata: An introduction. Englewood Cliffs NJ:Prentice Hall, (1989).