Filter-and-Forward Relay Design for OFDM-based Multiple Access Relay Channel

Fatemeh Bahadori, Bahareh Akhbari

Faculty of Electrical Engineering

K.N.Toosi University of Technology

Email: f.bahadori@ee.kntu.ac.ir, akhbari@eetd.kntu.ac.ir

Abstract— In this paper, we study the filter-and-forward (FF) relaying strategy for a multiple access relay channel (MARC) based on orthogonal frequency division multiplexing (OFDM) transmission. To reduce OFDM demodulation and redemodulation complexity at the relay, the FF-relay directly filters the incoming signal with a finite impulse response (FIR) filter and forwards it to the destination. We intend to design the relay filter subject to ensuring quality of service (QoS) at the destination. We propose an optimization criterion that is the joint sources power allocation and relay filter design, to maximize the worst subcarrier channel signal-to-noise ratio (SNR) subject to the sources and the relay transmit power constraints. We employ an alternating optimization algorithm to find the relay filter and the allocated power to sources, for our problem that is non-convex. Simulation results show that the proposed FF approach has a better performance for OFDM based MARC in frequency selective channels in comparison to the amplify and forward (AF) strategy. Also we investigate the effect of the relay filter length on the system performance.

Keywords-component; filter and forward (FF); multiple access relay channel (MARC); OFDM; amplify and forward (AF); frequency selective channel.

# Introduction

Cooperative communication is an important technique that can extend the coverage of wireless networks and also can exploit cooperative spatial diversity of different users in the network by means of providing several copies of a signal which have independent channel gains [1]. To obtain cooperative diversity, different relaying strategies have been proposed. Amplify and forward (AF), where the relay transmits an amplified version of its received signal including noise to the destination, decode and forward (DF) where the relay decodes its received signal, encodes a new one and transmits it to the destination, compress and forward (CF) and other strategies have been widely used in relay networks [2], [3]. Among the relaying schemes, the AF strategy due to its simplicity is suitable for cheap relay deployment and is more popular. Recently, in the case of frequency selective channels there have been some efforts to extend the AF relaying strategy to another scheme called filter and forward (FF) scheme [4]-[6], since AF is not efficient in suppressing the significant amount of inter-symbol interference (ISI) [1]. FF strategy is a linear filtering relaying scheme in which the incoming signal to the relay is filtered with a finite impulse response (FIR) filter [5].

On the other hand, multiple access relay channel (MARC) is used to well model a cooperation in multi-user networks, where multiple sources (generally considers two sources) communicate to a destination with the help of a relay node [7]. Since there are different relaying strategies, different scenarios of MARC such as using AF, DF and network coding schemes have been studied [8] - [10].

In this paper, we consider using FF relaying strategy in the MARC with frequency selective channels. In frequency selective channels another scheme that is used to compensate the effects of frequency selectivity of the channel is orthogonal frequency division multiplexing (OFDM) transmission [11]. OFDM is a type of multi-carrier modulation in which the subcarriers of the corresponding subchannels are mutually orthogonal [11]. So by using OFDM transmission in frequency selective channels, the signals encounter flat fading subchannels. Thus, in this paper we investigate OFDM transmission for MARC system model (with two sources). Our considered model can be regarded as the generalization of the investigated model in [5] that is a three-node relay channel. We use direct FF relaying for OFDM transmission that the received signal at the relay passes through a FIR filter at the chip rate of OFDM modulation and is then directly forwarded to the destination. Therefore there is not any need for OFDM demodulation/remodulation at the relay.

In this paper, we consider a criterion to design the FF relay for OFDM systems. We formulate the problem of joint relay filter design and sources power allocation for the worst subcarrier signal-to-noise ratio (SNR) maximization. The constraints are on the sources and the relay transmit powers. The resulted optimization problem is non-convex that we solve it with an efficient iterative algorithm which includes two sub-problems. The first problem is designing the relay filter to maximize the worst subcarrier SNR for a given allocated power, and the second is the power allocation optimization at the sources to maximize the worst subcarrier SNR for a given relay filter. Providing simulation results we investigate the effect of the relay filter length on the performance of the system. Also we show that the FF approach has a better performance for OFDM based MARC in frequency selective channels in comparison with AF scheme.

Notations: In this paper, vectors and matrices are written in bold letters and capital letters denote matrices. All vectors are vertical. Superscripts,  and  denote transpose, conjugate and Hermitian transpose, respectively.  signifies the matrix vectorization operator and ⊗ denotes the Kronecker operator. and stand for the trace and the rank of matrix, respectively. The symbol  represents the expectation operation.  means that  is

complex circularly symmetric Gaussian distributed with mean

vector and covariance matrix. and imply the  identity matrix and the  zero matrix, respectively. The notation Toeplitz indicates a  Toeplitz matrix with rows and [] as its first row vector, where is a row vector of size. Moreover, denotes a diagonal matrix with diagonal elements .

The rest of the paper is organized as follows. Section II defines the system model. The FF relay design problem is formulated in Section III and the proposed algorithm for solving this problem is also described in Section III. Finally, simulation results and conclusion are provided in Sections VI and V, respectively.

# system model

We consider a full duplex MARC, with two source nodes as and , the relay node and the destination node . All channels are frequency selective and we assume that the channel state information (CSI) of the sources to relay (SR) channels are known to the relay, and the relay also knows the relay to destination (RD) channel distribution. The sources and the destination employ OFDM modulation and demodulation respectively with subcarriers. The OFDM symbol vector of size at each source for  is given by , where each data symbol is assumed to be as  for . Then the –point normalized inverse discrete Fourier transform (IDFT) yields the time domain signal vector at each source for  that are given by

|  | (1) |
| --- | --- |

where is a scale factor. An alternative method that avoids ISI is to attach a cyclic prefix here with length  to each block of signal samples as

|  | (2) |
| --- | --- |

Then  is transmitted from the source to the relay, and the received signal at the relay is given by

- The multiple access relay channel with OFDM transmission.

|  | (3) |
| --- | --- |

whereis the additive white Gaussian noise at the relay with,  is the vector of channel impulse response between the source  () and the relay. According to the FF strategy to compensate the effect of the frequency selectivity of communication channels, the received signal at the relay node

is passed through a FIR filter and then is transmitted to the destination. Thus, the transmitted signal by the relay at time  is

|  | (4) |
| --- | --- |

where are the taps of FIR ﬁlter with length  . Finally, the transmitted signal by the relay passes through RD channel with order of and channel tap coefficient vector . So, the received signal at the destination  can be written as

|  | (5) |
| --- | --- |

where  is an additive Gaussian noise with variance  at the destination and , , are independent and identically distributed (i.i.d.) random variable according to

. So, each tap is independently Rayleigh faded and instead of the realization of each tap, its distribution is just known to the relay. We can rewrite (4) and (5) in the form of matrix as

|  | (6) |
| --- | --- |

by defining the following vectors and matrices for :

We assume that  for transmission without interference. Thus the discrete Fourier transform (DFT) of the received vector of size at the destination is given by

|  | (7) |
| --- | --- |

where is the normalized DFT matrix of size , for  is a circulant matrix that is generated from the Toeplitz filtering matrices , and:

|  | (8) |
| --- | --- |

To do relay filter optimization in Section III we must first drive the expression of the received signal in terms of relay tap coefficients. We need to know only the first row of , to compute. Since is generated from the Toeplitz filtering matrix , so only (the first row of ) is sufficient, where is the first row of . Then we have where

Now the diagonal elements of can be obtained as

|  | (9) |
| --- | --- |

whereis the DFT matrix of size . Finally, the th subcarrier of the received signal at the destination is given by

|  | (10) |
| --- | --- |

with signal component as

|  | (11) |
| --- | --- |

where is the signal component with respect to source  and  is the th row of  . The noise component is

|  | (12) |
| --- | --- |

where .

# Maximzation of The Worst Subcarrier SNR

We consider the FF relay design problem for the MARC described in Section II. We consider the problem of maximizing the worst subcarrier SNR at the destination subject to sources and relay power constraints. The problem of optimizing the relay filter and the sources power allocation to maximize the worst subcarrier SNR is formulated as follows:

|  | (13) |
| --- | --- |

where  is the maximum available total transmit power of sources and is the maximum available relay transmit power. To achieve the SNR of the th subcarrier channel, first we express the received signal power at the destination in terms of the relay filter :

|  | (14) |
| --- | --- |

First we compute the received signal power at the destination corresponding to source as

|  | (15) |
| --- | --- |

where is the transmit power of source , and

Using similar techniques as above, we can derive the received signal power with respect to source B. We also can derive the received noise power as

|  | (16) |
| --- | --- |

where

and  is the th row of .

Based on (15) an (16), the SNR of the th subcarrier channel corresponding to sources and are expressed as

|  | (17) |
| --- | --- |

Based on (6), we can obtain the relay transmit power as

|  | (18) |
| --- | --- |

where .

Now, based on (17) and (18), the optimization problem (13) is non-convex. So in order to avoid high computational complexity for solving a non-convex problem, we use a suboptimal alternating optimization algorithm [5]. This technique splits the main problem in two sub-problems. That is, first, it is assumed that the allocated power of sources is initialized and the problem (19) is considered to optimize the relay filter for a fixed power allocation at the sources. Then, using the relay filter obtained by solving problem (19), the sources power allocation is optimized. These two problems are

- Description of Algorithm 1.

Algorithm 1: Choose some suitable interval subject to Step 1: Set . Step 2: Solve problem (22). If it is feasible, then set , otherwise, . Step 3: Repeat the process until the criterion is satisfied.

solved in an alternating algorithm until the convergence is

established. Let the first problem for can be written as follows

|  | (19) |
| --- | --- |

By introducing the slack variable, the given max–min problem can be rewritten as

|  | (20) |
| --- | --- |

The above optimization problem is non convex and we convert it to a convex problem by a semi-definite relaxation (SDR) [12]. It is relaxed by dropping the rank-one constraint, which leads to the convex semi-definite programming problem (SDP) [12], as (21). By defining  and

we have

|  | (21) |
| --- | --- |

Due to variation of , the relaxed optimization problem is quasi-convex [12] and can be obtained by solving its corresponding feasibility problem as

|  | (22) |
| --- | --- |

For the optimal value of problem (21) (that we call it), problem (22) is feasible for, and infeasible for . Thus, we solve it by a bisection search method [12] with the procedure described in Algorithm 1.

In Algorithm 1, is the allowed error tolerance. After applying bisection method, if the rank of matrix  that is obtained by solving the relaxed optimization problem (22) is not one, we apply randomization techniques for generating the rank-one solution [13].

In the second step of the alternating algorithm, we need to optimize the allocated sources powers  and , for the given relay filter vector , which is obtained from the previous step. Thus, this sub-problem can be written as

|  | (23) |
| --- | --- |

Now, we first rewrite the relay power constraint in (18) as a linear form of , as follows:

whereand denotes the th row of the normalized IDFT matrix. Considering the relay power constraint as a linear form of   (that is denoted in (24)), the optimization problem in (23) can be written as a linear programming (LP) problem [12] formulated as (25) by defining the following terms:

| - Description of Algorithm 2. Algorithm 2: Step 1: Initialize and for . Step 2: Solve problem (19) with Algorithm 1. Step 3: Set the allowed minimum for the worst subcarrier SNR in problem (23) as the maximum value obtained from Step 2. Step 4: For the given and from Steps 2 and 3, solve problem (25) to obtain new and . Step 5: Go to Step 2. Here, set of problem (19) as the solution to problem (25) in Step 4. Step 6: Repeat Steps 2 to 5 until the termination criterion is satisfied.. | (25) |
| --- | --- |

This LP problem can be solved by a standard convex optimization solver, like CVX [12]. Now we combine problems (19) and (23) in Algorithm 2 for the joint sources power allocation and relay filter design to maximize the worst subcarrier SNR.

# Simulation Results

We have assumed a MARC with OFDM transmitters which the number of OFDM subcarriers are . A relay uses FF strategy and the quasi-static frequency selective sources-to-relay and relay-to-destination channels are with the lengths. Each plotted value in each figure is obtained by taking the average over the 500 channel realizations. The channel impulse response coefficients are modeled as zero-mean complex Gaussian random variables with an exponential power delay profile [4] that is:

|  | (26) |
| --- | --- |

where  is the symbol duration and  represents the delay spread. The noise variances at the relay and the destination are 1 dB.

- The worst subcarrier SNR versus maximum allowed relay transmit power,based on Algorithm 1

- The worst subcarrier SNR versus maximum allowed relay transmit power,based on Algorithm1 , and Algorithm 2.

We evaluate the performance of FF relay design to maximize the worst subcarrier SNR subject to some constraints on transmit powers of nodes, provided in problem (13). First, we consider the relay filter design only for a given equal sources power allocation, i.e. , based on Algorithm 1 that the results are shown in Fig 2. It can be seen that the obtained gain in terms of achievable SNR by using the FF relay is significantly more than that obtained by using the AF relay. Note that fore the length () the FF relay is equivalent to the AF relay. The performance is also improved by increasing the number of the relay filter taps.

We next (as shown in Fig. 3) evaluate the performance of the joint FF relay filter design and sources power allocation based on Algorithm 2 and compare the results with that obtained in the previous case. It can be seen that the joint optimization significantly outperforms the case that only design the relay filter. The sufficient length of FF filter to compensate the effects of frequency selective channels completely, is  which with exponential power delay profile the ISI term can be cancelled even with  [4]. According to Fig. 2, in our relay filter design, the filter length is sufficient.

# Conclusion

‌   In this paper, we have studied a multiple access relay channel system based on OFDM transmission and with frequency selective channels. The relay uses FF strategy to reduce the complexity of OFDM processes at the relay. With the aim of obtaining the FF relay filter and sources power allocation, we have investigated an optimization criterion which is maximizing the worst subcarrier SNR with transmit power constraints at sources and the relay. An alternating algorithm has been proposed to achieve the optimal solution, which includes two sub-problems. We have shown that joint optimization has much more gain. Simulation results have shown that the FF strategy outperforms AF relay significantly and is more practical than AF scheme for OFDM transmission.

REFERENCES

- H. Chen, A. B. Gershman, and S. Shahbazpanahi, “Filter-and-forward distributed beamforming for two-way relay networks with frequency selective channels,” IEEE Transactions on Signal Processing, vol. 60, no. 4, pp. 1927-1941, 2012.

- J. N. Laneman, D. N. C. Tse, and G. W. Wornell, “Cooperative diversity in wireless networks: efficient protocols and outage behavior,” IEEE Transactions on Information Theory, vol. 50, no. 12, pp. 3062 – 3080, Dec. 2004.

- A. Sedonaris, E. Erkip, and B. Aazhang, “User cooperation diversity —

- Part I. System description,” IEEE Transactions on Communication, vol. 51, pp. 1927-1938, Nov. 2003.

- H. Chen, A. B. Gershman, and S. Shahbazpanahi, “Filter-and-forward distributed beamforming in relay networks with frequency selective fading,” IEEE Transactions on Signal Processing, vol. 58, pp. 1251-1262, Mar. , 2010.

- D. Kim, J. Seo and Y. Sung, “Filter-and-forward transparent relay design for OFDM systems,” IEEE Transactions on Vehicular Technology , vol. 62, no. 9, pp. 1-16, Nov. 2013.

- M. Maleki and V. Tabataba Vakili, “Filter-and-forward transceiver design for cognitive two-way relay networks,” IET Communications, 2015.

- L. Sankaranarayanan, G. Kramer and N. B. Mandayam, “Capacity theorems for the multiple-access relay channel,” Proc. 42nd Annu. Allerton Conference. Communications, Control, and Computing. 2004.

- M. El Soussi, A. Zaidi, and L. Vandendorpe, “Amplify-and-forward on a multiaccess relay channel with computation at the receiver,” Proc. IEEE International Conference on Network Games, Control and Optimization (NetGCooP), pp. 74-79, 2012.

- M. El Soussi, A. Zaidi, and L. Vandendorpe, “DF-based sum-rate optimization for multicarrier multiple access relay channel,” EURASIP Journal on Wireless Communications and Networking 2015.

- B. Han, Z. Zhao, M Peng, Y. Li, W. Wang, “Resource allocation for OFDM multiple-access relay channels with network coding,” Proc. Global Communications Conference (GLOBECOM), 2012 IEEE, pp. 4701-4706, Des. 2012.

- J. G. Proakis, Digital Communications,  New York: McGraw-Hill, Inc., Fifth ed. 2008.

- S. Boyd and L. Vandenberghe, Convex Optimization, Cambridge,  U.K.: Cambridge Univ. Press, 2004.

- N. D. Sidiropoulos, T. Davidson, and Z.-Q. Luo, “Transmit beamforming for physical-layer multicasting,” IEEE Transactions on Signal Processing, vol. 54, no. 6, pp. 2239–2251, Jun. 2006.