Current Ripple Reduction in Single Input, Multiple Output Converters Using Phase-Shift and Coupled Inductors

S. A. Jafarian M.

M. Sc. student

Dept. of Elec. and Computer Eng.

Isfahan University of Tech.

Esfahan, Iran, 8415683111

Email: a.jafarian@ec.iut.ac.ir

Hamid R. Karshenas

Faculty member

Dept. of Elec. and Computer Eng.

Isfahan University of Tech.

Esfahan, Iran, 8415683111

Email: karshen@cc.iut.ac.ir

Abstract—This paper investigates current ripple reduction in single input multiple output converters based on input current phase-shift and output inductor coupling. Design considerations in power electronic converters such as weight, volume, price and reliability are important factors. Electrolytic capacitors which are usually used for smoothing voltage ripple are one of the weakest component based from the reliability point of view. Besides, inductors are generally bulkiest and heaviest component in converters. Current ripple causes losses and as a result, increases the size of both capacitors and inductors. In this paper, phase-shift of input currents employed for decreasing the input current ripple of converter, which consequently leads to lower losses and smaller input capacitors. Also, coupled inductors are used for decreasing current ripple and the volume of output inductors. In this paper, the expressions for obtaining the optimum value of coupling coefficient to achieve the minimum ripple current for a given duty-cycle are derived. Simulation results are used to validate the analysis. An experimental setup with single input multiple output two buck converters was implemented and used for the verification of theoretical analysis.

Keywords-ripple reduction; multiple output; phase-shift; coupling inductors; capacitor reduction

# Introduction

Some of the most important design considerations for power electronic converters are weight, volume, price and reliability. In Most cases, the bulkiest and heaviest component in converters is inductor. On the other hand, electrolytic capacitors which are usually used for dc voltage smoothing are one of the weakest points due to their high failure rate. An important parameter that directly affects the size of both inductors and capacitors is current ripple. Current ripple causes increased losses and consequently influences the size of components. In other words, reducing current ripple results in smaller size and higher efficiency and reliability of system [1].

Electrolytic capacitors which are often used in power electronic converters typically have high Equivalent Series Resistance (ESR). As a result, high current ripple causes increased losses and temperature. As shown in [2], when temperature of an electrolytic capacitor goes up by 10 degrees Celsius, its lifetime becomes half. Therefore, larger capacitor should be used to handle this issue. Generally, there are two solutions for this issue:

### Using film capacitors: film capacitors have smaller ESR and thus can withstand more current ripple and have longer lifetime. These features, of course, come with higher price as compared with electrolytic capacitors.

### Using methods to reduce current ripple in electrolytic capacitors. Generally, there are two methods for performing this task [3]:

#### Using appropriate control method

#### Using auxillary circuits

The key point in designing inductors in power electronic converters is the optimization of inductor value for both fast transient response and optimum current ripple. High value of inductor yields low current ripple and subsequently increased efficiency, but at the same time it leads in larger inductor size, slower transient response. Coupled inductors used for interleaved multiphase dc-dc converters have both fast transient response and high efficiency. Yet, coupling coefficient have dependence on duty-cycle of converter.

Single input multiple output dc-dc converters are used in computer power supply and Electric Vehicle (EV) battery charging stations. Fig. 1 shows a simple structure of this systems. In this configuration, Iin consists of all dc-dc converters input current and capacitor Cdc should be designed such that it tolerates the current ripple of all converters.

The inverse coupling of output inductors of two dc-dc converters can be used for decreasing the size of inductor [4] [5]. Because dc-dc converters works in various type of condition with different duty-cycle ratios, coupling coefficient should be calculated precisely for each application.

Figure 1.	Structure of single input multiple output dc-dc converters

In this paper, phase-shift of input current of two single input multiple output buck converters employed to reduce current ripple of input capacitor. Also, inverse coupling of inductors is used to decrease current ripple and size of each converter’s inductor. Fig. 2 shows circuit diagram of proposed converters.

# Phase-Shift of Input Currents

One of the control methods used for reducing the current ripple of capacitors in multi converter systems is applying phase-shift among different input currents. Fig. 2 shows the circuit diagram of two single input multiple output buck converters. It is assumed that both converters work with same constant duty-cycle and 180˚ phase-shift of input currents. Fig. 3 shows input currents of each buck converter and the summation of them when there is no phase-shift in compared with the case of 180˚ phase-shift. As can be seen, Iin which is equal to the summation of two input currents, in the case of 180˚ phase-shift does not have zero level. When there is no phase-shift between input currents or the phase-shift is either less than 1-D∙360˚  or more than D∙360˚ (for D ≥ 50 %), Iin have zero level. The perunit current ripple rms values have also been shown in these figures to illustrate the effect of phase-shift on current ripple.

As shown in [6], the optimum phase-shift which yields minimum input current ripple for N parallel converters working with the same duty-cycle is given by

phase-shiftopt=360˚N	

Fig. 4 shows the gating waveforms of two buck converters which is phase-shifted by 180˚ [6] [7].

Figure 2.	Two buck converters with coupled inductors

a) 180˚ Phase-shift condition

b) No phase-shift condition

Figure 3.	Waveforms of input current of two buck converters and summation of them with a) 180˚ Phase-shift b) No phase-shift

As shown in [8], for asymmetric conditions where the duty-cycle of converters are different, Fast-Furrier Transform (FFT) can be employed to analyze each converter’s input currents. Subsequently, optimum phase-shift between input currents can be calculated by minimizing the phasor of Iin which is equal to the phasor summation of input currents. Although using this method results in minimum input current ripple, it needs several steps of calculation and at the other hand does not cause significant improvement as compared with phase-shift as in (1) [9].

# Steady State Analysis of Coupled Inductors

As stated in the introduction, coupling of inductors can be exploited for both decreasing their size and current ripple in dc-dc converters. Based on the winding direction in each winding, the coupling of two inductors can be done in two ways:

### Direct-coupling: In this method, the flow of currents in two windings produces flux in the same direction.

### Inverse-coupling: In this method, the flow of currents in two windings produces flux in opposite  direction.

In an inductor carrying dc current, the type of coupling can be determined by winding direction, which is recognized by dot convention. For example, in the circuit of Fig. 2, inverse coupling of inductors is employed for two paralleled buck converters. Assuming M is the mutual inductance between the inductors of two converters, the voltage and current relationships in inductors can be written as [4]

vL1=L11diL1dt+M12diL2dt	

vL2=M21diL1dt+L22diL2dt	

Figure 4.	Gating waveforms of two buck converters phase-shifted by 180˚

In this paper, it is assumed

L11=L22=L	

M12=M21=M=k∙L	

where k is coupling coefficient and is between -1 (inverse coupling) to 1 (direct coupling).

By substituting diL2dt from (3) in (2) and using (4) and (5), (2) can be written as

vL1=1-kL∙diL1dt+kL∙vL2	

The gating pattern of two buck converters (Fig. 2) with 180˚ phase-shift can be divided into three modes as shown in Fig. 4. In the following, an average equivalent inductance is obtained for each mode and consequently an average equivalent inductance will be calculated for one cycle. The average equivalent inductances are calculated for converter 1, although the final average equivalent inductance is the same for both converters. For mode 1, both MOSFET 1 and MOSFET 2 are on. Assuming that both converters are working with the same duty-cycle, the inductor voltages in Fig. 2 are given by

vL1=vL2=vin-vout	

Substituting (7) in (6) yields

vL1=1+kL∙diL1dt	

The average equivalent inductance for mode 1 is defined as

Leq1=1+kL	

Similar approach can be used for mode 2 and 3 and consequently it can be shown that the average equivalent inductance corresponding to each mode is obtained as

Leq2=L1-k21+k∙D1-D	

Leq3=L1-k21+k∙1-DD	

For duty-cycles more than 50% and phase-shift either less than 1-D∙360˚ or more than D∙360˚, there is another mode which both MOSFET 1 and 2 are off. It can be shown that the average equivalent inductance for this mode is equal to Leq1.

The current ripple of an inductor in one cycle is given by

∆iL=v∙TLeq	

where v, T and Leq are respectively inductance average voltage, switching period and average equivalent inductance in one period.

The current ripple of each mode can be calculated by using the time interval of each mode based on Fig. 4 and using (9) to (11). By summing current ripple of each mode, the current ripple of L1 in one cycle is given by

∆iL1=v1∙TLeq=v1∙(2D-1)TLeq1+v1∙(1-D)TLeq2+v1∙(1-D)TLeq3	

Solving Leq in (13) for duty-cycles more than 50%yeilds

Leq=L1-k21+k∙1-DD	

Comparison of (14) and (11) shows that the equivalent inductance in a system with two buck converters with duty-cycles more than 50% is equal to Leq3 [10].

For comparing current ripple in coupled and uncoupled (k=0) situations due to (12), one may write

∆iL,coupled∆iL,uncoupled=Leq,uncoupledLeq,coupled=1+k∙1-DD1-k2	

# Proposed Method

In this paper, both phase-shift of input currents and inductors coupling are employed in order to reduce both input and output current ripple. As no significant improvement can be achieved by the approach addressed in Sec. II, (1) is used in this paper for calculating optimum k.

Inverse coupling of inductors increases core losses due to increased ac flux [11]. On the other hand, it decreases core cross-section and size due to dc flux reduction. In this paper, reverse coupling is employed as the effect of core size reduction due to dc flux reduction is more prominent. Theoretically, all inductors in an N-converter system can be coupled in one core. However, because of difficulties in building multi winding inductor and complication in calculating optimum coupling coefficients, only two inductors are coupled.

The method discussed in Sec. III is used to calculate average equivalent inductance for one cycle. For minimizing the current ripple of inductor, Leqshould be maximized based on (12). By derivating (14) with respect to k for a given duty-cycle, one may find optimum k for the specific duty-cycle. It can be shown that variation of duty-cycle does not have significant influence on k.

# Simulation Verification

For evaluating the foregoing analysis, the multi converter system in Fig. 2 was simulated in PSIM software with parameters as shown in Table I.  Setting D=0.66 in (14) gives

Leq=L1-k21+k2	

Derivating (16) with respect to k yields

dLeqdk=L∙(-12)k2+4k+11+k22	

Solving (17) gives k=-0.268. Fig. 5 shows the variation of normalized current ripple with respect to duty-cycle with k=-0.268. This graph shows that current ripple reduces by about 7% with inductor coupling (point A) as compared with no coupling where the normalized current ripple is 1 (Point B). Furthermore, it can be seen from Fig. 5 that the variation of duty-cycle has no significant effect on current ripple

Fig. 6 shows Iin for both phase-shift and no phase-shift conditions. It can be seen that about 50% reduction in input current ripple has been achieved using phase-shift.

Fig. 7 shows IL for coupled and uncoupled output inductors in phase-shift or no-phase-shift conditions. Waveform of IL2 shifted down for showing both IL1 and IL2 in one figure. By comparing figure (a) and (c), coupling of inductors decreased inductor current ripple with approximately 8.5%. As seen in Fig. 7 (b), inductor coupling with no phase-shift not only does not improve output current ripple but also increases it.

# Experimental Results

In order to validate the proposed method and simulation results, a laboratory type experimental setup similar to Fig. 2 was implemented with parameters listed in Table I. The output inductor was constructed by placing two windings on the two

Simulation and Experimental Setup Parameters

| Parameter | Description | Value |
| --- | --- | --- |
| Vin | Input dc Voltage | 18 V |
| D | Duty-cycle | 0.66 |
| fsw | Switching Frequency | 30 kHz |
| Cin | dc Link Input Capacitor | 200 μF |
| L | Self-Inductance | 1 mH |
| k | Coupling Coefficient | -0.268 |
| Cout | Output Capacitor | 5 μF |
| Rload | Output Load Resistor | 12 Ω |

Figure 5.	∆iL,coupled/∆iL,uncoupled vs duty-cycle

side legs of an EE core, and incorporating an airgap in the central leg. In practice and due to error in calculations, the coupling coefficient of inductors was measured equal to -0.29 while their self-inductance was measured equal to 920 μH.

Fig. 8 shows input current ripple for phase-shift and no phase-shift conditions. The results are in agreement with simulation results shown in Fig. 5. The gain of current sensors used was 165 mV/A.

Fig. 9 shows IL ripple for coupled and uncoupled inductors in phase-shift or no-phase-shift conditions. A good agreement can be seen between experimental and theoretical results. A close inspection of Fig. 9 shows that the reduction of current ripple with coupled inductors is about 3%. This can be contributed to the fact that the coupling coefficient in experimental system is slightly different from the calculated value. The gain of current sensor was 1 V/A.

a) No phase-shift condition

b) Phase-shift condition

Figure 6.	Simulation results of Iin with: a) No phase-shift, b) Phase-shift

a) Uncoupled inductors

b) Coupled inductors-No phase-shift

c) Coupled Inductors-phase-shift

Figure 7.	Simulation results of IL with: a) Uncoupled inductors, b) Coupled inductors andNo phase-shift, c) Coupled inductors with phase-shift

# Conclusions

In this paper, phase-shift of input currents along with coupled inductors was employed in order to decrease input/output current ripple in single input multiple output converters. Analytical derivation of optimal coupling for minimum current ripple was investigated.  It was shown that variation of duty-cycle does not have significant impact on the reduction of current ripple. The analysis was validated by simulation of a system consisting of two buck converters. An experimental setup was implemented for the verification of theoretical analysis. Acceptable agreement between the simulation and experimental results shows the validity of analysis.

a) No phase-shift condition

b) Phase-shift condition

Figure 8.	Experimental results of input current ripple with: a) No phase-shift, b) Phase-shift

a) Uncoupled inductors

b) Coupled inductors-No phase-shift

c) Coupled Inductors-phase-shift

Figure 9.	Experimental results of IL ripple with: a) Uncoupled inductors,     b) Coupled inductors and No phase-shift, c) Coupled Inductors with phase-shift

##### References

M. H. Rashid, Power electronics handbook: devices, circuits and applications: Academic press, 2010.

M. L. Gasperi, "Life prediction model for aluminum electrolytic capacitors," in Industry Applications Conference, 1996. Thirty-First IAS Annual Meeting, IAS'96., Conference Record of the 1996 IEEE, 1996, pp. 1347-1351.

H. Wang and H. Chung, "A novel concept to reduce the DC-link capacitor in PFC front-end power conversion systems," in Applied Power Electronics Conference and Exposition (APEC), 2012 Twenty-Seventh Annual IEEE, 2012, pp. 1192-1197.

V. C. Valchev and A. Van den Bossche, Inductors and transformers for power electronics: CRC press, 2005.

H. Kosai, S. McNeal, A. Page, B. Jordan, J. Scofield, and B. Ray, "Characterizing the effects of inductor coupling on the performance of an interleaved boost converter," Proc. 29th Annual Passive Components and Exhibition (CARTS USA-2009), pp. 237-251, 2009.

T. Kohama and T. Ninomiya, "Automatic interleaving control for paralleled converter system and its ripple estimation with simplified circuit model," in Power Electronics, 2007. ICPE'07. 7th Internatonal Conference on, 2007, pp. 238-242.

C. Chang, "Current ripple bounds in interleaved DC-DC power converters," in Power Electronics and Drive Systems, 1995., Proceedings of 1995 International Conference on, 1995, pp. 738-743.

M. Schuck and R. C. N. Pilawa-Podgurski, "Input current ripple reduction through interleaving in single input multiple output dc-dc converters," in Control and Modeling for Power Electronics (COMPEL), 2013 IEEE 14th Workshop on, 2013, pp. 1-5.

X. Lu, W. Qian, D. Cao, F. Z. Peng, and J. Liu, "A carrier modulation method for minimizing the dc link capacitor current ripple of the HEV DC-DC converter and inverter systems," in Applied Power Electronics Conference and Exposition (APEC), 2011 Twenty-Sixth Annual IEEE, 2011, pp. 800-807.

S. A. Wibowo, T. Zhang, K. Masashi, T. Taura, Y. Kobori, K.-I. Onda, et al., "Analysis of coupled inductors for low-ripple fast-response buck converter," IEICE transactions on fundamentals of electronics, communications and computer sciences, vol. 92, pp. 451-455, 2009.

P.-L. Wong, "Performance improvements of multi-channel interleaving voltage regulator modules with integrated coupling inductors," Virginia Polytechnic Institute and State University, 2001.

P. S. D. Seminar, "Practical Magnetic Design: Inductors and Coupled Inductors," 2012.b