Temperature Control of Heating Procedure in Blade Production Process

| Niloufar Minouchehr Department of Electrical Engineering Ferdowsi University of Mashhad, FUM Mashhad, Iran niloofar_minoochehr@yahoo.com | Nima Vaezi Department of Avionic Engineering Mashhad Aviation Training Center, MATC Mashhad, Iran nimavaezi368@gmail.com |
| --- | --- |
| Parisa Tavakoli Department of Electrical Engineering Ferdowsi University of Mashhad, FUM Mashhad, Iran parisatavakoli@yahoo.co.uk | Seyed Kamal Hosseini Sani Associate Professor of Electrical Engineering Ferdowsi University of Mashhad, FUM Mashhad, Iran kamal_hosseynii@um.ac.ir |

Abstract— Wind turbine blade manufacturing is one of the most important projects executed in Sun and Air Research Institute (SARI). The Vacuum Infusion Process (VIP) is used in the blade manufacturing. Each blade concluded from layers of composite combined with resin. This combined material is heated based on the predefined temperature profile. In the heating process, the most important task is tracking the temperature profile and regulating the blade temperature with the reference signal. Therefore the temperature control should be executed through accurate control systems. In this paper the heating element voltage is controlled by using adaptive GPC and EDMC controllers. Controlling this voltage leads to blade temperature control. Simulation and control design is done in MATLAB Simulink. There are some limitations in the heating system that obligates the designer to utilize the adaptive control system. These limitations are listed as delay in heating system, different transfer function parameters in various blade sections and lack of the cooling system. In fact there is not a cooling system and therefore overshoot in the blade temperature is not acceptable and controllable during the heating process. It is necessary to utilize the adaptive control to overcome overshoot and over-heating as well as other limitations in heating system. The simulation results illustrate that the temperature of heating process alters smoothly moreover it tracks the desired temperature profile without any overshoot.

Keywords: Blade, EDMC, GPC, Heating Process, Wind Turbine.

# Introduction

Wind turbine blade production is one of the most significant sub-projects in the wind industries. The Vacuum Infusion Process (VIP) is used in the blade manufacturing. Each blade concluded from layers of composite combined with resin. Vacuum Infusion Process (VIP) is one of the most well-known method in the blade manufacturing. This method leads to lighter blades which have less loading effect on the generator and mechanical components in comparison to other methods like Bill of Material (BOM) and using of Glass Fibers [1].

One of the most significant items in blade design and manufacturing is the blade length. When the blade length increases, its weight becomes more that 2.5 order greater. The main procedure in resin injection process is done by the vacuum. The vacuum reduces one side pressure of the blade body and helps the resin injection procedure. After resin injection, heating process is performed by using the heater element. The heating process needs a long time to be fulfilled but it can be shortened by increasing the temperature if the material has the ability to resist against high temperature (Fast Heating). The heating process increases the blade strength [1].

The temperature profile should be predefined and applied to the control system. As an example the temperature profile can be selected similar to Fig. 1.

In the recent years and probably in future, model predictive controller (MPC) is widely used in industries in different applications. In addition, adequate performance and simplicity stems from the specific strategy of this control method. The main advantages of the model predictive controllers are listed below.

(1)  Predicting future behavior of the process,

(2)  Optimizing future behavior by determining proper inputs,

(3) Applying certain number of inputs and repeating the procedure in the next sampling interval [2].

Fig. 1. An example of the temperature profile

Another advantage of the MPC controllers is the ability to solve the problem with any type of the model. Both linear and nonlinear models are able to dealing with the MPC controllers with the same complexity. To predict the process output and solve the optimization problem in a nonlinear MPC, high computational capability is required. This makes the approach impractical in some applications [2].

The most part of the problem is performing the optimization algorithm. Different MPC algorithm are known as DMC, GPC, EMDC and etc.

EDMC represents extended dynamic matrix control, which is used to control nonlinear process. In this method, control inputs are determined based on a linear approximation model of the process and will be updated during each sampling interval. Due to the nonlinear relation between the prediction error and the control input, numerical methods are used to solve the optimization problem defined in the EDMC method. Some these nonlinear processes do not converge to an acceptable and adequate solution for some desired outputs or external disturbances. Therefore, the first step is process stabilizing by using output feedback or proper state [3].

Generalized Predictive Control )GPC) has the ability to stabilize all linear processes even process with time delays. According to optimal control theorem, great prediction horizon is significant in GPC design. Lyapunov stabilization theory shows that GPC with great prediction horizon is stable and its objective function has been assigned as the Lyapunov function. The stability conditions will be achieved when control horizon has been chosen greater than number of the unstable poles. If there are any limitations in the system, the control horizon should be greater than number of limitations and effective limitations [3-4].

GPC can be either with little previous knowledge or a more complex system such as non-minimum phase, open loop used unstable and variable dead time to a "simple" plant control system (open-loop stable). In particular, GPC seems to be untouched (unlike poles placement strategies) when the plant model is over parameterized. Furthermore, as offsets are eliminated from the result of the adoption of a CARIMA plant model, GPC is a contender for the overall self-optimization applications [3-4].

In this paper after modeling the heating process and obtaining a discrete transfer function, the GPC and EDMC control methods are described and the control design will be done based on these algorithms. At the end the simulations results are presented and compared to show the similarities and difference between various MPC methods.

# Blade Heating System Modeling

The initial model is obtained from the experimental data which are obtained from the heating process of 100 KW wind turbine blades will be manufactured in Sun and Air Research Institute (SARI). These data set contains the reference temperature signal and the real blade temperature. The result from the MATLAB identification toolbox is the continous trannfer function which is presented in (1).

| GpS=40.811+9957Se-30S | (1) |
| --- | --- |

The discrete transfer function which is appropriate to design the MPC controllers is shown in (2).

| Gpz=β£Tsz-e-Ts£z-TdTs | (2) |
| --- | --- |

In this equation the parameters are defined as: a=£-1≈104,Ts=1s, β≈41,Td=30 sec. By considering  the parameters the discrete transfer function is presented in (3).

| Gpz=4×10-3z-0.9999×z-30 | (3) |
| --- | --- |

In this paper the heating element voltage is controlled by using adaptive GPC and EDMC controllers. Controlling this voltage leads to blade temperature control. Simulation and control design is done in MATLAB Simulink. There are some limitations in the heating system that obligates the designer to utilize the adaptive control system. These limitations are listed as delay in heating system, different transfer function parameters in various blade sections and the cooling system. In fact there is not a cooling system and therefore overshoot in the blade temperature is not acceptable and controllable during the heating process. To avoid overshoot and over- heating and to overcome the other limitations in heating system, it is necessary to utilize the adaptive control systems.

# GPC Algorithm & Control Design

GPC is one of the most applicable MPC methods based on the process transfer function and the control signal will be calculated through minimizing the objective functions. Most of the SISO systems can be defined as (1) after linearization through operating point.

| Az-1yt=z-dBz-1ut-1+Cz-1et∆ ∆=1-z-1 | (4) |
| --- | --- |

Equation (4) is famous as CRIMA. In this equation, yt is output, ut is input and et is the white Gaussian noise with zero mean. The polynomials A, B and C are defined as (5).

| Az-1=1+a1z-1+a2z-2+⋯+anaz-na Bz-1=b0+b1z-1+b2z-2+⋯+bnbz-nb Cz-1=1+c1z-1+cz-2+⋯+cncz-nc | (5) |
| --- | --- |

The aim is to calculate the control input ut, u(t+1) and…. The aim will be achieved by minimizing the (6) cost function.

| JN1,N2,Nu=j=N1N2δj[yt+jt-wt+j]2+j=1Nuλj[∆u(t+j-1)]2 | (6) |
| --- | --- |

The first step is to predict the yt+j  for j≥N1 and j≤N2. Therefore this is necessary to solve the Diophantine equation (7).

| 1=Ejz-1Az-1+z-jFjz-1 , Az-1=∆A(z-1) | (7) |
| --- | --- |

The polynomials Ej and Fj degrees are j-1 and na respecively. Remind that C=1, the result of applying ∆Ej(z-1)zj to equation (4) is:

| Az-1Ejz-1yt+j=Ejz-1Bz-1∆ut+j-d-1+Ejz-1et+j | (8) |
| --- | --- |

According to equation (7), (8) will be the result.

| yt+j=Fjz-1yt+Ejz-1Bz-1∆ut+j-d-1+Ejz-1et+j | (9) |
| --- | --- |

Since noise will be occurs in future, yt+j can be predicted by (10).

| yt+j¦t=Gjz-1∆ut+j-d-1+Fjz-1yt Gjz-1=Ejz-1Bz-1 | (10) |
| --- | --- |

The polynomials Fj and Ej are calculated by the recurrent methods. There are many different methods to do the GPC method calculations which are not based on the recurrent methods to solve the Diophantine equation. Here the recurrent method is described. According to the recurrent method the polynomials Fj and  Ej are defined through (11) and (12).

| Fjz-1=fj,0+fj,1z-1+⋯+fj,naz-na | (11) |
| --- | --- |
| Ejz-1=ej,0+ej,0z-1+⋯+ej,j-1z-(j-1) | (12) |

After calculating Fj and Ej, Gj could be calculated by using Gjz-1=Ejz-1Bz-1. The future output prediction would be done by using (12).

| yt+d+1t=Gd+1∆ut+Fd+1yt yt+d+2t=Gd+2∆ut+1+Fd+2yt yt+d+Nt=Gd+N∆ut+N-1+Fd+Nyt | (13) |
| --- | --- |

The equation (13) can be rewritten as (14).

| y=Gu+Fz-1yt+G'z-1∆ut-1 | (14) |
| --- | --- |

Matrix in (14) would be defined as following.

y=yt+d+1|tyt+d+2|t⋮yt+d+N|t        u=∆ut∆ut+1⋮∆ut+N-1

G=g0    0g1     g0⋯0⋯0⋮⋮gN-1gN-2⋮⋮⋯g0     Fz-1=Fd+1z-1Fd+2z-1⋮Fd+Nz-1

G'z-1=Gd+1z-1-g0zGd+2z-1-g0-g1z-1z2⋮Gd+Nz-1-g0-g1z-1-…-gN-1z-N-1zN

The polynomials related to the past will be classified in f.

| y=Gu+f | (15) |
| --- | --- |

To solve the GPC problem the control signal will be calculated through (16). It is noticeable that the d samples delay causes the response appearance in time sample, d+1. Therefore the prediction horizon will be defined as N1=d+1 and N2=d+N and Nu=N. The control signal u will be the result of minimizing  J without any conditions.

| J=(Gu+f-w)T(Gu+f-w)+λuTu u=GTG+λI-1GT(w-f) | (16) |
| --- | --- |

The first sample of control signal u is calculated by (17).

| ∆u(t)=K(w-f) | (17) |
| --- | --- |

In (17), K is the first row of (GTG+λI)-1 GT. The block diagram of the GPC controller is shown in Fig. 2.

Fig. 2. The block diagram of the GPC controller

If the future error, (w-f=0) will be zero, the control signal will be zero too. The control signal will be applied with considering the future error [5-6].

# Extended Dynamic Matrix Control (EDMC)

If the Dynamic Matrix Control (DMC) method utilized for a linear problem and the modeling done completely, the D vector defined in (18), contains external disturbances. If a similar control law has been applied to the nonlinear problem, the D vector contains two components, one of them originated from nonlinear process, dnl, and another originated from the external disturbance, dext.

| D=dext+dnl=yprocess-ymodel=y(t)process-ytmodel11⋮1 Dt+1⋮Dt+P=dextt+1⋮dextt+P+dnlt+1⋮dnlt+P dnlt+i=ynlt+i-ylit+i | (18) |
| --- | --- |

In the DMC method the D vector is constant while in the EDMC method, the D vector will be calculated through nonlinear dynamic model and there is no necessity that the D vector will be constant. Therefore the dnl elements will be various and dext elements will be constant. Therefore the optimal control input will be calculated through (19).

| ∆u=GTQG+λI-1GTQyd-ypast-D ∆u=GTQG+λITGTQyd-ypast-dext-dnl | (19) |
| --- | --- |

The element dnlt+i is a nonlinear function of ∆u and is calculated through the following procedure. In second t:

- l=0

- Dlnl=0

- ∆u is calculated from (16) then yl+1nl and yl+1li will be calculated.

old  Dl+1nl=yl+1nl-yl+1li

Therefore

new Dl+1nlmodificationDl+1nl old

- old Dl+1nl=1-βDlnl+βnew Dl+1nl

And β is a very small number.

- Dl+1nl= new Dl+1nl

- l=l+1 and return to step 3.

The above algorithm will be repeated until Dl+1nl-Dlnl becomes very small. After second t the algorithm will be repeated for second t+1. Fig. 3 shows the calculation procedure of, the D vector.

In Fig. 4 the EDMC control algorithm is shown. In this figure in any sampling interval in the recurrent calculations, the control input key will be closed and when the convergence achieved the key will be closed to the process and this algorithm will be repeated in the next sampling interval [6-7].

Fig. 3. The procedure of calculating the D vector in EDMC algorithm

Fig. 4. EDMC control algorithm [8]

# Simulation Result and Comparison

The system model and designed controllers are simulated in MATLAB. The simulation is done based on the discrete model of the blade heating system. The GPC and EDMC controllers are compared through this simulation. For this analysis the reference input which is the reference or command in temperature is chosen the same for both the controllers. The reference temperature signal is shown in Fig. 5. The control signal and real temperature of the blade is shown in Fig. 6. and Fig. 7, respectively. As it is shown in Fig. 6, the control signal of GPC controller has small variations in comparison to the control signal of EDMC.

Fig.5. Reference signal for blade temperature

Fig.6. Control signals for EDMC and GPC controllers

Fig.7. Blade temperature for EDMC and GPC controllers

As it is shown in Fig. 7, the blade temperature has soft changes and there is no overshoot by using EDMC controller. As it is mentioned before, because there is not any cooling system and the temperature control is very vital in the process, overshoot is not allowed. The temperature response is slow because of the heating system nature. Therefore the response rate is not as important as the overshoot and temperature oscillations. The numerical comparison is done in Table I. This table shows overshoot, rise time and settling time of the heating system with two adaptive controllers.

Table 1: Comparison of GPC and EDMC controllers

|  | Overshoot | Rise Time (Second) | Settling Time (Second) |
| --- | --- | --- | --- |
| GPC | 3.45% | 51 | 180 |
| EDMC | 0 | 141 | 240 |

# Conclusion

In this paper, the final step of wind turbine blade manufacturing process titled heating procedure has been described. Model of the process is extracted based on the the experimental data which are obtained from the heating process of 100 KW wind turbine blades will be manufactured in Sun and Air Research Institute (SARI). Two different controllers have been designed by the use of MPC strategies. The performance of the controllers has been evaluated in MATLAB Simulink environment. The results of simulation indicate the appropriate performance and behavior of the designed adaptive controllers. Due to the limitations caused by the heating system, adaptive control is essential to be applied. The comparison between two controllers shows that the EDMC controller provides a smooth response without no overshoot whilst the GPC controller tracks the desired temperature profile rapidly with a noticeable overshoot.

##### Acknowledgment

The authors would like to thank Sun and Air Research Institute (SARI) for technical and financial support.

##### References

Anthony Broad,”Development of Vacuum Assisted Composites Manufacturing Technology for Wind Turbine Blade Manufacture”, University of Central Lancashire, MSc Thesis, 2013.

Alain Yetendje, Maria M. Seron and José A. De Doná,” Robust multiactuator fault-tolerant MPC design for constrained systems”, Int. J. Robust. Nonlinear Control, pp1828–184, 2013.

V. Ghaffari, S. Vahid Naghavi, A.A. Safavi,” Robust model predictive control of a class ofuncertain nonlinear systems with application to typical CSTR problems”, Elesevier, Journal of Process Control 23, pp 493– 499, 2013.

N. Vaezi, A. Karimpour, “Predictive Controller Using Modal Series for DC/AC Convertors”, 23rd Iranian Conference on Electrical Engineering (ICEE),2015.

E. F. Camacho and C. Bordons,” Model Predictive Control in the Process Industry”, Springer, 1995.

Saeed Shamaghdari, S.K.Y. Nikravesh and Mohammad Haeri, “Integrated guidance and control of elastic flight vehicle based on robust MPC”, Int. J. Robust Nonlinear Control 2014.

Samaneh Sadat Sajjadi, Naser Pariz, Ali Karimpour & Amin Jajarmi,” An off-line NMPC strategy for continuous-time nonlinear systems using an extended modal series method”, Springer, Nonlinear Dynamics, 2014.

M. Haeri, “Improved EDMC for the processes with high variations and/or sign changes in steady-state gain”, COMPEL, 2004, 23, (2), pp. 361–380.

Integrated Auto-Regressive Moving-Average