Finite Control Set Model Predictive Control for dc Voltage Balancing in Vienna Rectifier

S. Majid Hosseini Khorasgani

M.Sc. Student

Department of Electrical and Computer Engineering

Isfahan University of Technology

Isfahan, Iran

majid.hosseini@ec.iut.ac.ir

Alireza Izadinia

M.Sc. Student

Department of Electrical and Computer Engineering

Isfahan University of Technology

Isfahan, Iran

a.izadinia@ec.iut.ac.ir

Hamid R. Karshenas

Faculty Member

Department of Electrical and Computer Engineering

Isfahan University of Technology

Isfahan, Iran

karshen@cc.iut.ac.ir

Abstract— This paper is concerned with the investigation of Finite Control Set Model Predictive Control (FCS-MPC) for dc link voltage balancing in a Vienna rectifier. Large EV charging stations with common dc bus utilize a high-power rectifier as a front end conversion stage. In this regards, Vienna rectifier, as a central ac-dc stage, with bipolar dc bus structure is an attractive choice. Bipolar dc bus is potentially prone to voltage imbalance which is undesirable. dc link voltage imbalance also lead to distorted input current and more power quality issues. This paper proposes a balancing method based on the FCS-MPC method. In order to apply the FCS-MPC method, the discrete time model of the Vienna rectifier with regard to unbalanced voltage condition is derived. By applying this model, the proper cost function is proposed to balance dc bus voltage. Simulation and experimental results are presented to validate the proposed control strategy in terms of effectiveness of the balancing method and low distortion operation.

Keywords- Finite Control Set- Model Predictive Control (FCS-MPC); Vienna rectifier.

# Introduction

The limited autonomy of commercially available and economically viable batteries and their rather long charging times along with the need for additional infrastructures are some of the main challenges with respect to the widespread use of EVs [1], [2]. Fast and ultra-fast EV charging concepts have been proposed as a feasible solution for reducing the charging time. These schemes, however, call for high power charging stations. Off-board chargers, which are usually not installed in residential areas, are advantageous when the power rating of chargers have been increased [2].

The structure of high power charging stations are classified into two main groups: In one method different chargers are directly connected to ac grid bus. In the other method,  however,  chargers are connected to a common dc bus fed by a central rectifier [3], [4]. The second one has the benefit of less conversion stages  which leads to higher efficiency and lower cost [3], [4]. Furthermore, this architecture offers easier integration of the storage system and renewable energy sources [3], [4]. The central rectifier plays an important role in the latter charging station architecture. Specifically, the impact of central rectifier on grid should be carefully investigated in order to avoid any power quality issue such as distorted input current or poor power factor associated with conventional phase-controlled rectifiers. In other words, central rectifiers should satisfy some features such as controllable power factor and low distortion input current [3].

The configuration of the dc bus in high power charging stations can either be unipolar or bipolar. The bipolar structure is preferable since it offers more power transfer capability while keeping the step-down ratios of dc-dc converters low. In [3], an advantageous high power charging station structure utilizing NPC converter as a rectifier and bipolar dc bus configuration is proposed. [3] Also presents a method for balancing the dc bus mid-point voltage. In [4], dc power balance management (PBM) techniques for this charging station is proposed.

In this paper, a three-phase/switch/level boost rectifier known as the Vienna rectifier, introduced in [5], is investigated as the central charging station. High input power factor, low total harmonic distortion (THD), reduced number of switching device, high power density, low switch voltage stress, low switching frequency and high efficiency are important advantages of the Vienna rectifier [6], [7]. Additionally, this topology is compatible with bipolar dc bus structure.

Considering bipolar dc bus, unbalanced loading condition can occur in a Vienna rectifier which will lead to dc bus voltage imbalance. This issue also leads to distorted input ac current. Therefore, the neutral point balancing methods have to be considered in the control system. In order to mitigate the unbalanced operation of the Vienna rectifier, several approaches have been proposed. These methods are implemented by controlling the switching states of active switches. [8], [9] present that an offset in the current reference can balance the neutral point voltage.

Finite Control Set Model Predictive Control (FCS-MPC) is a predictive control method that is introduced in [10] for power electronic application. In this control method, the discrete nature of power converters will be considered to optimize the number of calculations needed to achieve the optimum state. In [11] the application of FCS-MPC method for 4-leg NPC converter is proposed.

Figure 1. Structure of high power charging station with Vienna rectifier and bipolar dc bus.

In this paper, FCS-MPC method is proposed for the Vienna rectifier. This method is a promising solution to achieve many advantages in terms of simplicity and effectiveness for nonlinear control application [10].

In this paper, the discrete time model of a Vienna rectifier and unbalance capacitor voltages is derived. According to this model, the suitable cost function is proposed. The proposed cost function guarantees sinusoidal input current while keeping capacitor voltages balanced.

The rest of the paper is organized as follows: in Section II, Vienna rectifier is introduced and fundamental equations for this rectifer is explained. Furthermore, unbalanced voltage issue for a Vienna rectifier is described.  In section III, the FCS-MPC method is proposed for solving the unbalanced voltage problem. Section IV and V verify the validation of the proposed control method by simulation and experimental results. Finally, conclusions are expressed in section VI.

# Vienna Rectifier in Charging Stations

The structure of a high power charging station is shown in Fig. 1. As explained in the introduction, Vienna rectifier is a suitable structure for such applications with the ability of providing low distortion input current and bipolar dc bus. In this section, the topology of a Vienna rectifier is introduced and basic equations are derived. Also, the space vector modulation which is a well suited modulation technique for this structure is briefly explained.

## Vienna Rectifier

The topology of a Vienna rectifier is shown in Fig. 2. It consists of three active switches and eighteen diodes. Low switching frequency is one of the main advantage of this converter. However, this converter requires complex modulation strategies which is usually realized using space vector modulation (SVM) technique. In space vector

Figure 2. Vienna rectifier topology.

modulation, the reference vector is realized by properly adjusting the dwell times of vectors [12].

In a conventional three-level SVM technique, there are 27 switching states. By elimination of 8 repetitive states, 19 remaining switching states leads to 19 fundamental vectors in 6 sectors. In each sector, Vienna rectifier switching states can be selected between 7 different space vectors and states [13], [14].

Considering Fig. 2 and applying KVL to rectifier poles, the basic equations for a Vienna rectifier can be obtained as

vsabc=Lsdiabcdt+Rsiabc+vrabcN-vnN       

where different parameters are shown in Fig. 2. Writing (1) in space-vector form yields

Vs=LsddtIs+Rs(Is)+VrN-VnN

where Is, VrN and VnN are the space vectors associated with input line current, input phase voltage to dc center point voltage and dc center point to neutral voltage respectively. The rectifier input voltage in (2) can be defined by switching states and dc link voltage as

VrN=Sr× Vdc            

where Sr is the switching state vector of the rectifier and Vdc is the dc link voltage.

## Unbalanced voltage model for Vienna rectifier

Since bipolar dc bus configuration allows the connection of unequal dc loads, the dc link voltage imbalance and consequently neutral voltage shift can occur. Furthermore, unbalanced loading can lead to distorted input current. All these conditions are undesirable.

Assuming equal capacitors in the dc link, i.e. C1= C2=C, the voltage imbalance which is the difference between the instantaneous voltages of each capacitor is defined as

∆vC=vC2- vC1

where vC1  and vC2 are the capacitor (or pole) voltages. The current through output capacitors are given by

iC1=C1dvC1dt                                    (5)

iC2=C2dvC2dt                                    (6)

Subtracting (6) and (5) in (4) yields

d( ∆vC) dt=1CiC2-iC1 

The neutral point current, shown in Fig. 2, is given by

i0= iC2-iC1  

Equation (7) and (8) show the effect of neutral point current on capacitors voltage imbalance. In order to balance the capacitor voltages, the control system should adjust the neutral point current properly. The relation between neutral point current and switching states, which can be obtained by analyzing a Vienna rectifier operating principal, can be written as

i0=s1ia+s2ib+s3ic 

where s1 =1 and s1 =0 represent on and off states respectively. Equation (9) shows how the controllable parameter i0 can be adjusted using switching states with the final objective of controlling voltage imbalance.

# Finite Control Set Model Predictive Control (FCS-MPC) Method

In order to apply FCS-MPC algorithm to power converters, the following major steps should be considered as follows [10].

- Deriving the discreet time Model of the converter topology with regard to all the feasible switching states.

- Evaluating the proper cost function based on control objectives.

- Applying this cost function for all the feasible switching states and choosing the switching state minimizing the cost function.

The basis of FCS-MPC methods is on the discretization of system model. After model discretization with appropriate sampling frequency, the prediction procedure should be done online in each sampling interval. In this section, the implementation of FCS-MPC method for Vienna rectifier is explained.

## Rectifier model discretization

In order to implement the FCS-MPC, the discrete time model of converter should be derived. A block diagram of predictive current control for a Vienna rectifier is shown in Fig. 3. The objective here is to obtain the optimum current vector such that a cost function is minimized. For the calculation of seven feasible predicted current vectorsis(k+1), the measured current is(k) and the main voltage vs(k) are required. After prediction of seven current vectors, the reference current is*(k+1) is obtained from outer control loop, and finally, the predicted current are compared with their references. Writing (2) in discrete form yields

Isk+1=TsLs[Vsk-VrN]+(1-RsLsTs)Is(k)

where Ts represents the sampling time.

Applying Clarke’s transformation to (10) yields

isαk+1=TsLs[vsαk-Srαk×Vdc2]+(1-RsLsTs)isα(k))  

isβk+1=TsLs[vsβk-Srβk×Vdc2]+(1-RsLsTs)isβ(k)) 

where α and β subscripts represent quantities in α and β axes respectively. Equations (11) and (12) give expressions for predicted currents in an orthogonal coordinate.

Figure 3. Basic principle of predictive current control.

## Imbalance voltage model discretization

Equation (5), (6), (7) and (8) can be written in discrete form as

vC1 k+1= vC1 k+ TsC iC1(k)                 

vC2 k+1= vC2 k+ TsC iC2(k)                

∆vCk+1= TsC i0k+ vC2k- vC1(k)

Equation (13) and (14) can be used to obtain the discrete time model of capacitors voltages under unbalanced condition. Equation (15) gives the imbalance voltage model to be used in FCS-MPC method. Substituting the discretized version of (9) in (15) yields

∆vCk+1= TsC s1ia+ s2ib+s3ic

+ vC2k- vC1k                          

Equation (16) expresses the relation between unbalanced voltage and switching states.

## Cost function

As mentioned in the introduction, different control objectives can be included in a single cost function in FCS-MPC methods. Each objective in the cost function can be included with a specific weighting factor. The weighing factors must be properly tuned based on the importance of corresponding objective [15]. Therefore, determining a proper cost function is a crucial part for applying MPC method.

For the case of a Vienna rectifier with bipolar dc bus, there are two important objectives that need to be fulfilled by MPC controller. The first one is reference current tracking to achieve sinusoidal, unity power factor input current, and the second one is the balancing of neutral point voltage. Therefore, the cost function for this application can be expressed as

f=isα*-isα+isβ*-isβ+ λ ΔvCk+1-∆vC*(k + 1)2          

where isα* and isβ* are reference current components in α - β coordinates,  isαand isβ are predicted current components in α - β coordinates  and λ represents the weighting factor. The first two terms in (17) are meant for reducing error in sinusoidal reference current tracking, while the third term attempts to reduce capacitor voltage imbalance. Since the desired imbalance on capacitor voltages is zero, it is assumed that

∆vC*(k + 1) = 0                             

Substituting (9) in (17) yields

f=isα*-isα+isβ*-isβ+

λ TsC s1ia+ s2ib+s3ic+ vC2k- vC1(k)2      

Equation (19) expresses the proposed cost function for a Vienna rectifier with unbalanced voltage. The proposed cost function can satisfy sinusoidal input current while maintaining bipolar dc bus voltages balanced provided that weighing factor is properly selected.

# Simulation  Results

In this section, simulation results for the proposed voltage balancing method are illustrated. The system parameters corresponding to the simulated system are listed in Table I. The simulated system has been chosen to be similar to the experimental system such that the agreement between simulation and experimental results can be better comprehended. For this purpose, a three phase transformer with the turn ratio of 6:1 is used at the front-end of Vienna rectifier in order to reduce the input ac voltage to safer level.  For the implementation of the FCS-MPC method, the proposed cost function given by (19) is calculated in each sampling interval to select the optimum switching state.

Table I. Simulated and experimental system parameters

| Rectifier Parameters | Value |
| --- | --- |
| Line to line ac voltage | 400 V |
| Reference dc voltage | 150 V |
| dc side capacitors | 2 mF |
| dc load | 50 Ω |
| Input transformer ratio | 6:1 |
| Input filter inductance | 5 mH |
| Input resistance | 0.1 Ω |
| Sampling frequency | 20 kHz |

## dc link voltage balancing

Fig 4 shows the capacitor voltages before and after applying voltage balancing technique. As can be seen, there is approximately 30 V (20%) imbalance between capacitors voltages before t=0.6 s. At t=0.6 s, the balancing term in the cost function given by (19) is applied by setting weighing factor λ equal to 0.5. The effectiveness of this term can be observed from waveforms.

## Input current

As stated in the introduction, voltage imbalance lead to distorted input current and power quality problems. Fig. 5(a) and Fig. 6(a) show the input current waveform of a Vienna rectifier in unbalanced and balanced conditions respectively. Also shown in Fig. 5(b) and Fig. 6(b) is the corresponding harmonic spectra of these waveforms. Comparison between these two waveforms demonstrates how unbalanced condition will lead to distorted input current which is undesirable from the power quality point of view.

# Experimental Results

In this section, experimental results are presented in order to validate the performance of the proposed control algorithm. For the verification of the proposed control method, a laboratory type 500 W Vienna rectifier setup was implemented. The MPC control procedure was implemented via a DSP control board connected to a host PC by serial port.

Figure 4. Simulation result for the bipolar dc bus balancing method.

(a)

(b)

Figure 5.  Simulation results for the input current of phase-a for unbalanced bipolar dc bus voltages. (a) Simulated current. (b) Harmonic spectra.

i

Figure 7. Experimental result for the bipolar dc bus balancing method.

(a)

(b)

Figure 6.  Simulation results for the input current of phase-a after applying balancing method. (a) Simulated current. (b) Harmonic spectra.

(a)

(b)

Figure 8. Experimental results for the input current of phase-a for unbalanced bipolar dc bus voltages. (a) Experimental current. (b) Harmonic spectra.

## dc link voltage blancing

Fig. 7 shows the experimental results for output dc voltages. Comparison with Fig. 4 confirms the validity of theoretical analysis and shows how the FCS-MPC algorithm can be used to balance capacitors voltages in a Vienna rectifier.

## Input Current

The experimental results of the input current corresponding to phase “a” with unbalanced dc link voltage is shown in Fig. 8(a). Fig. 9(a) shows the input current and also input voltage when the proposed balancing algorithm has been applied.   The harmonic spectra corresponding to each waveform is also shown in Fig. 8(b) and Fig. 9(b). As can be seen, current distortion has been reduced.

(a)

(b)

Figure 9. Experimental results for the input current of phase-a after applying balancing method. (a) Experimental current. (b) Harmonic spectra.

# Concludion

In this paper, the implementation of FCS-MPC method for dc link voltage balancing in a Vienna rectifier was investigated.  Vienna rectifier is a practical choice as the high-power front-end rectifier in large EV charging stations with common dc bus. The dc link voltage imbalance which may happen in this structure is undesirable. In this paper, the relationship between dc-link voltage imbalance and dc neutral current was derived. To control the voltage imbalance, FCS-MPC method was used. Therefore, the system model was discretized. By defining a proper cost function, both sinusoidal current tracking and voltage balancing were achieved. Simulation of a test system showed the validity of analysis. A laboratory type experimental system was used to verify theoretical analysis. Close agreement between results confirms the validity of analysis.

##### References

[1]	L. Dickerman and J. Harrison, "A new car, a new grid," IEEE Power and Energy Magazine, vol. 8, p. 55, 2010.

[2]	M. Vasiladiotis and A. Rufer, "A Modular Multiport Power Electronic Transformer With Integrated Split Battery Energy Storage for Versatile Ultrafast EV Charging Stations," Industrial Electronics, IEEE Transactions on, vol. 62, pp. 3213-3222, 2015.

[3]	S. Rivera, B. Wu, S. Kouro, V. Yaramasu, and J. Wang, "Electric vehicle charging station using a neutral point clamped converter with bipolar DC bus," Industrial Electronics, IEEE Transactions on, vol. 62, pp. 1999-2009, 2015.

[4]	L. Tan, B. Wu, S. Rivera, and V. Yaramasu, "Comprehensive DC Power Balance Management in High-Power Three-Level DC-DC Converter for Electric Vehicle Fast Charging."

[5]	J. W. Kolar and F. C. Zach, "A novel three-phase utility interface minimizing line current harmonics of high-power telecommunications rectifier modules," Industrial Electronics, IEEE Transactions on, vol. 44, pp. 456-467, 1997.

[6]	J. Miniboeck and J. W. Kolar, "Comparative theoretical and experimental evaluation of bridge leg topologies of a three-phase three-level unity power factor rectifier," in IEEE Power Electronics Specialists Conference, 2001, pp. 1641-1646.

[7]	D. Mukherjee and D. Kastha, "Voltage sensorless control of the three-level three-switch Vienna rectifier with programmable input power factor," IET Power Electronics, 2015.

[8]	L. Dalessandro, S. D. Round, and J. W. Kolar, "Center-point voltage balancing of hysteresis current controlled three-level PWM rectifiers," Power Electronics, IEEE Transactions on, vol. 23, pp. 2477-2488, 2008.

[9]	T. Friedli, M. Hartmann, and J. W. Kolar, "The essence of three-phase PFC rectifier systems—Part II," Power Electronics, IEEE Transactions on, vol. 29, pp. 543-560, 2014.

[10]	J. Rodriguez, M. P. Kazmierkowski, J. R. Espinoza, P. Zanchetta, H. Abu-Rub, H. Young, et al., "State of the art of finite control set model predictive control in power electronics," Industrial Informatics, IEEE Transactions on, vol. 9, pp. 1003-1016, 2013.

[11]	F. Rojas-Lobos, R. Kennel, and R. Cardenas-Dobson, "Current control and capacitor balancing for 4-leg NPC converters using finite set model predictive control," in Industrial Electronics Society, IECON 2013-39th Annual Conference of the IEEE, 2013, pp. 590-595.

[12]	L. Hang, B. Li, M. Zhang, Y. Wang, and L. M. Tolbert, "Equivalence of SVM and carrier-based PWM in three-phase/wire/level Vienna rectifier and capability of unbalanced-load control," Industrial Electronics, IEEE Transactions on, vol. 61, pp. 20-28, 2014.

[13]	J. W. Kolar and U. Drofenik, "A new switching loss reduced discontinuous PWM scheme for a unidirectional three-phase/switch/level boost-type PWM (VIENNA) rectifier," in Telecommunication Energy Conference, 1999. INTELEC'99. The 21st International, 1999.

[14]	T. Viitanen and H. Tuusa, "Experimental results of vector controlled and vector modulated VIENNA I rectifier," in Power Electronics Specialists Conference, 2004. PESC 04. 2004 IEEE 35th Annual, 2004, pp. 4637-4643.

[15]	J. Rodriguez and P. Cortes, Predictive control of power converters and electrical drives vol. 40: John Wiley & Sons, 2012.