Improvement of Hyperspectral Images Compression using Compress Sensing and Spectral Correlation

Nasrin Shayestehnia

Young Researchers Club and Elites

Bushehr Branch, Islamic Azad University,

Bushehr, Iran.

Email: shayesteh343@yahoo.com

Ahmad Keshavarz

Department of Electrical Engineering

Persian Gulf University,

Bushehr, Iran.P.o.box:75169

Email: a.keshavarz@pgu.ac.ir

Abstract—In this paper, a compression algorithm for hyperspectral images based on the compressed sensing is proposed. First tests for correlation analysis and sparsity of hyperspectral images are performed, and then, based on these results, a scheme for compressed sensing these images is presented.  Thus, considering one of the two adjacent bands as a reference and the other one as a non-reference, first the reference bands are reconstructed, and then using those reconstructed reference bands their adjacent bands are reconstructed. Measurement matrix used in the encoder is a type of structured random matrices that take random measurements of each band.  Simulation results show that the proposed algorithm is faster compared to conventional hyperspectral compressed sensing, which reconstructs the all the bands simultaneously, and has higher compression performance.

Keywords-component; compressed sensing; compression; hyperspectral  images ; spectral dependency;

# introduction

The main theme of this article is in the general field of remote sensing. We know that, remote sensing is the science and technology of obtaining information about an object without physical contact with it. One type of sensors that is used in remote sensing is hyperspectral sensor or Imaging spectrometer. The images obtained by these sensors, which are known hyperspectral images, in principle, are images taken from an area in the number of narrow and continuous spectral bands, as well as hyperspectral because of this reason is used for this type of data, which have valuable spectral information on the number of very high spectral bands. Result of simultaneous imaging in multiple bands by this sensor along with the aerial or space platform movement of its carrier is a three-dimensional data set, consisting of hundreds hyperspectral image, in which each image corresponding to a particular spectral band. Since a single digital image can typically have a size of 12 megabytes or more, the size of hyperspectral data cubes could easily move to the gigabyte level.

Therefore, due to the large volume of these data and limitations of onboard resources, transmission bandwidth and storage space we are always facing big challenges to process, store, archive, and especially their transmission from the satellite to ground station. For this reason, hyperspectral images compression, from the beginning of the project topics of hyperspectral systems, as one of the most significant issues in facing these data is known.

So far, various methods have been proposed for compressing these images [1]. These methods can be divided into the following three groups: prediction, transform coding and vector quantization. In hyperspectral images, as well as traditional two-dimensional images, prediction-based methods are used for lossless compression. In these methods, first data by a predictor be uncorrelated and then the prediction error by an entropy coder, be coded. The predictor takes advantage of the high correlation between spectral bands and also is based on the correlation with the neighboring pixels values. Best methods in the field of compression based on prediction is lookup tables (LUT)] [2] or methods from CALIC [3]. Compression of vector quantization (VQ) of the hyperspectral images, which also based on the classification, has two steps: codebook generation and coding [4]. Generally, methods to generate these codebooks, especially in hyperspectral images, due to their high volume, have high computational cost, and major works done have tried to reduce the computational complexity of this step. Transform coding also has two steps: first step is to transform the data in a domain where the representation of the data is more compact and less correlated, and the second step, is to encode this information in the best way possible(as efficiently as possible). Generally, in this last step, due to use of quantization, the information loss occurs. It should be noted that most of the developed algorithms for hyperspectral image compression, according to the same method of transform-based compression [5]. However, each of the above methods has disadvantages, including unsuitable for onboard implementation due to the complexity of used encoder,  waste of  the energy and  imaging resources (compression based on transform), and therefore, wasting costs (wastage of costs) and error-resilient ability.

Studying traditional methods of compression, it is possible that these questions arise in the mind that: why go to so much effort to acquire all the data when most of what we get will be thrown away? Can’t we just directly measure the part that won’t end up being thrown away? Fortunately, compressed sampling theory answer these questions. This theory made it possible to reconstruct signals from fewer samples than required by the Shannon-Nyquist theorem; under the assumption any signal is sparse in some domain. In addition, CS is robust to the noise in the measurements, Due to the facility of the measurement acquisition at the encoder, the CS-based technique is a natural fit for onboard compression.

# Compressed sensing theory

Compressed sensing which was originally introduced in [6] and [7] is the result of utilizing sparsity condition in the sampling problem. Compressed sensing is now a hot research topic in literature. The interesting point in this discussion is that, before the establishment of sampling methods, reconstruction method is widely known. In fact, the phenomenal success of l1-norm minimization technique, led to design of sampling methods compatible with this method.

Suppose that we are going to take a signal vector x. For this purpose, it is enough that we compute its Inner product with some waveforms Φ. The state that is of consideration in the CS is that the number of measurements taken is less than the size of the original signal x. This situation in many practical contexts (a lot of applications) is considered; for example, in some cases, production of sensors is expensive (e.g., infrared cameras), so we want to use less sensors than the number of pixels. Alternatively, it is possible that the process of measurement is very costly (e.g., imaging or neutron scattering), and finally, it is possible that measurement process is time consuming (i.e. MRI), and the only way to accelerate it, is taking less measurements. Thus, the measurement process is as follows:

								

If the signal x in a transform base, such as Ψ, has a sparse representation, means that a large number of its elements in the  Ψ domain are zero, we can write:



Where, θ is a vector with length N with the maximum K nonzero elements. Therefore, the CS measurement process becomes as the following:

		

As mentioned xN×1 is the vector of the original signal, ΦM×N is the measurement basis, i.e. is a system that we want to measure signal in it, and essentially, the dimensions of this matrix Φ, determine the measurement rate(MR), and yM×1 is vector of measurement (observations) obtained.

Clearly, the above equation with the condition M <N, represents underdetermined system of linear equations, and generally, has an infinite number of solutions, if any. . A question that arises is, which one of the answers is the original solution of the equation, or in other words, how can do accurate reconstruction with M <N measurement? Compressed sensing topic that was introduced in 2006 is the answer to this question. In fact, the major issue in CS, is that, if x is sparse in a basis Ψ, under these sparseness conditions, the solution of equations is unique, and is a solution that a number of its non zero elements is less, Or in other words has smaller l0 norm. Therefore, the reconstruction problem leads to an optimization problemP0. But because of difficulties which existed in practice for solving P0, and also high sensitivity of l0norm to noise, l0norm was replaced by the l1norm, and P1reconstruction problem for solving CS problems was considered [8], [9]:

In general, many algorithms for finding sparse solution of underdetermined system of equations are used, including Matching pursuit algorithms (MP) that are fast algorithms but inaccurate, Basis pursuit family (BP) (equation 4), which are algorithms that , obtain sparse solution of system of equations via minimizing the l1 norm. This family has more accuracy than the Matching pursuit algorithms, but has a relatively high computational complexity, and the family of convex optimization algorithms, which requires few measurements to reconstruct despite the complex calculations, among it can be pointed to the Gradient projection for sparse reconstruction (GPSR) that for large-scale problems has very good performance and trying to solve the following unconstrained convex optimization problem [10]:



In this paper, GPSR-BB algorithm is used, in which the step length, αk, according to the Barzilai-Borwein gradient method, (BB) [11] is set, and the step size λk, by minimize linear search is calculated. This algorithm requires less computation than The GPSR-Basic, and therefore reaches the optimal solution, in less time. Also (moreover), some other issues about compressed sensing, has been demonstrated [12], such as, how to make sure that the l1 solution will always be sparse, Is this result from the signal which was originally sampled?, such as RIP, which states that if the matrix Φ be such that satisfies restrictive isometric property (RIP), the reconstructed solution of is accurate [13]. But it is difficult task of checking whether the matrix Φ satisfies the RIP inequality, however, it is proved that a series of random matrices satisfy RIP condition with high probability, and therefore, have ability to accurate reconstruct. Among them, can be noted to family of Gaussian/random Bernoulli matrices [14], and structured random matrices [15,16].

In this paper, we focus on structured random matrices. In summary, a random structured matrix can be indicated as multiplied by three operators: A Pre-randomizer which makes the input signal as random, then a fast transform operator, such as Walsh–Hadamard (WHT), and finally, a Random down sampler, which performs down sampling in order to select M random sample. In fact, in the sampling using structured random matrix, first, the desired signal using a randomizer vector ‬ randomized, and then, sampling is done. Because randomize signal is simpler than randomize sampling matrix, complexity of‬ sampling system will be less. Among the desirable features of structured random matrices, which has encouraged us to use them, we can mention the following: ‬

- These matrixes for a range of sparse signals are used.

- The number of samples needed to reconstruct the signal is optimal. ‬

- ‬Unlike random matrices, which are completely unstructured, they are structured, ‬ therefore, complexity of the sampling system is low and calculations speed is high.

# hyperspectral compressed sensing

In this section, we examine the sampling system of hyperspectral images based on compressed sensing method, and then based on the specific spectral structure of the hyperspectral images we present a scheme for compressing these images using the compressed sensing.

## Sparsity of hyperspectral images

As we know, the sparsity is necessary for compressed sensing a signal. A N-dimensional sparse signal, which has K non-zero coefficients, and other (N-K) coefficients all zeros, it is called a K-sparse signal. Generally, signals and images themselves are not sparse, but them view in some transform space, is sparse, that is, the transform coefficients of Fourier transform, DCT, DWT for them contain few large components, and many other small coefficients are negligible.

In this work, we consider discrete wavelet transform (DWT) as sparseness transform, because, as is clear from Figure 1, hyperspectral images like the most natural images have the sparseness view in the DWT basis.

DWT coefficients for image "lena" and band 40 of the hyperspectral data of "AVIRIS"

## The common scheme of the hyperspectral compressed sensing (HCS 1)

Common scheme of hyperspectral compressed sensing can be seen as Figure 2, where each band of the hyperspectral image is assumed as a common 2D image. In the encoder, CS measurements of each band independently are taken. As mentioned above, the Φ measurement matrix used in this work is a type of structured random matrices which both the block Walsh Hadamard and block discrete Cosine transformations (BWHT&BDCT) have been utilized in simulations as fast transformation operators. In decoder well, each band through GPSR-BB algorithm with default values ​​will be reconstructed, and finally, each band by inverse wavelet transform, is reconstructed.

common scheme of the hyperspectral compressed sensing (HCS 1)

In Table 1, the results obtained from this design for Lunar lake hyperspectral data with size 512 × 512 × 150 is presented [17]. As shown in Figure 3 and Table 1, both transforms have a similar PSNR performance.

simulation results of common scheme with different measurement matrices

| BDCT |  | BWHT |  | MR |
| --- | --- | --- | --- | --- |
| Time(s) | PSNR(dB) | Time(s) | PSNR(dB) |  |
| 1.81 | 44.55 | 1.87 | 44.54 | 50% |
| 2.47 | 43.04 | 2.56 | 43.04 | 40% |
| 2.98 | 41.5 | 3.06 | 41.5 | 30% |
| 4.02 | 39.83 | 3.8 | 39.78 | 20% |
| 3.76 | 37.63 | 4.23 | 37.7 | 10% |

PSNR performance of common scheme with different measurement matrices

One of the interesting features of hyperspectral images, which can be effective in compression method design, is their extremely high correlation. As we all know, correlation of the hyperspectral images, including spectral and spatial correlation, which spatial correlation means the correlation of adjacent pixels in a band, and spectral correlation means the similarity of adjacent bands. Spectral and spatial correlations of the lunar lake image in Figure 4 is shown, which, as we said, the Spectral correlation (blue curve) of adjacent bands often is close to 1, and spectral correlation always is stronger than spatial correlation (pink curve) (a ).

Figure b, by calculating and drawing the correlation coefficient matrix shows spectral correlation of the adjacent bands that based on the image dimensions we have a matrix of 150 × 150. In dark places, correlation is low and in bright places, correlation is high. On the main diagonal, correlation coefficient of each band with its own is shown, it is obvious that the maximum value is 1, and white, and the area around the main diagonal shows the correlation between adjacent bands that almost in all places is high, except bands of 106 to 112.

(a)

(b)

spectral and spatial correlation of hyperspectral images

This high correlation helps us which if restructured a band, we are able to use it as reference band to reconstruct adjacent bands. Indeed, a band can be predicted fully or partially from reconstructed adjacent band.

proposed scheme of hyperspectral Compressed sensing (HCS 2)

## proposed scheme of hyperspectral Compressed sensing (HCS 2)

In this scheme, the hyperspectral image bands are divided into reference and non- reference bands, so that we separated them as one among. Here, we showed reference bands as X, which including bands 1,3,5, ..., and we showed the non-reference bands as Y, which including bands 2,4,6, .... (Figure 6). First predicting coefficients Y from X through a linear predictor are estimated. In this work, a linear prediction algorithm based on the criterion of minimum mean square error (MMSE), has been used, so that,



Thus, we transfer the obtained prediction coefficients α and β to the Decoder. Then, any reference band X is sampled similar to the common scheme of compressed sensing.  For non-reference bands Y, again measurements CS, is done independently for each band. But for the reconstruction of each non-reference band, previous reconstructed band (which was a reference band) is used. In this way, first based on a previous band and the prediction coefficients which obtained previously, a prediction band for each non-reference band is obtained. Based on this band, we slightly change GPSR-BB algorithm. In this way, we consider the initial solution of the algorithm equal to this amount of prediction band Y. This can cause the algorithm to reach the main solution in a less repetitions and the speed of reconstruction be higher. Another advantage is that, due to the initial value, even with the low measurement rates, we get the desired solution. For this reason, in the proposed algorithm, we consider measurement rates of reference band lower than measurement rates of non- reference bands.

Also, for the stopping criteria, the third stopping criteria presented in the paper [18] is used, which to improve the quality of the reconstruction, an error term is added to the algorithm cost function GPSR-BB, and to keep fit, weighting coefficients w1 and w2, which are respectively 0.9 and 0.1, it is adopted. Therefore, a new cost function is as follows:

Where, F1θi=12y-ΦΨθi22+τθi1is the common cost function of algorithm GPSR-BB, and F2θi=θi-θS2 is error term And finally stopping criteria is as follows:

That is, when the relative change of Fθiin two adjacent iterations, is smaller than a threshold TF, algorithm stops, which amount TFis considered as 0.05.

obtained results of proposed scheme with BWHT, BDCT matrices and hybrid

| BWHT &BDCT |  | BDCT |  | BWHT |  | Non ref MR | ref MR | MR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time (s) | PSNR (dB) | Time (s) | PSNR (dB) | Time (s) | PSNR (dB) |  |  |  |
| 0.19 | 46.56 | 0.17 | 45.86 | 0.94 | 45.82 | 40% | 60% | 50% |
| 0.19 | 44.86 | 0.17 | 44.31 | 1.07 | 44.3 | 30% | 50% | 40% |
| 0.18 | 43.24 | 0.18 | 42.81 | 1.43 | 42.82 | 20% | 40% | 30% |
| 0.22 | 41.32 | 0.18 | 41.29 | 1.69 | 41.27 | 10% | 30% | 20% |
| 0.19 | 38.25 | 0.17 | 38.02 | 2.25 | 38.06 | 8% | 12% | 10% |

The results of implementing the proposed scheme in the different average MRs based on different measurement matrices are given in Table 2. It should be noted that all values ​​are average.

Comparison Results of PSNR and reconstruction time of two schemes

| Time of proposed scheme (s) (HCS2) | Time of common scheme (s) (HCS1) | PSNR of proposed scheme (HCS2) | PSNR of common scheme (HCS1) | MR |
| --- | --- | --- | --- | --- |
| 0.17 | 1.81 | 45.86 | 44.55 | 50% |
| 0.17 | 2.47 | 44.31 | 43.04 | 40% |
| 0.18 | 2.98 | 42.81 | 41.5 | 30% |
| 0.18 | 4.02 | 41.29 | 39.83 | 20% |
| 0.17 | 3.76 | 38.02 | 37.63 | 10% |

Comparison of the PSNR performance

comparison of the reconstruction time

If we compare the overall results with common method, we see that both PSNR and reconstruction time are improved, PSNR has approximately 1.3 dB increased, and the time has been reduced approximately 10 times (Table 3 and Figures 7 and 8). Also, if both block cosine transform matrix and block Walsh Hadamard is used, which means that, for example, for reference bands, block Walsh Hadamard, and for non- reference bands, the block cosine transform (or vice versa) are used, we have the improvement in PSNR, but it may not be too significant, and only in rates 30% and more rates, we have improvement about 0.5 dB. However, the reconstruction time is similar to states with a single matrix. An example of the reconstructed images with common scheme and the proposed scheme is shown in Figure 8.

(a) (c) (d) (b)

reconstruction results of 60th band of the lunar lake. (a) original image, (b) common scheme (PSNR=39.292) ;(c)HCS2 & BDCT matrix (PSNR=40.643); (d) HCS2 & both matrices(PSNR=40.674).

# Conclusions

In this paper, a scheme based on compressed sensing technique for hyperspectral images compression was presented. This scheme, due to the use of this technique has a simple encoder, so suitable for onboard applications, and actually transfers all calculation load to the decoder .As we have seen, in this scheme, by using available high spectral correlation between hyperspectral bands, we were able to simultaneously improve reconstruction time and the PSNR. In general, the time, cost and very low power consumption are considered as features of this scheme.

##### References

S. Sanjith, R.Ganesan, "A review on hyperspectral image compression", in International Conference on Control, Instrumentation, Communication and Computational Technologies (ICCICCT), Kanyakumari , India, Jul. 2014, pp. 1159-1163.

J. Mielikainen, "Lossless compression of hyperspectral images using lookup tables", IEEE Signal Processing Letters, vol. 13, no. 3, pp. 157-160, Mar. 2006.

E. Magli, G. Olmo, E. Quacchio," Optimized onboard lossless and near-lossless compression of hyperspectral data using CALIC", IEEE Geosci. Remote Sens. Lett. vol. 1, no. 1, pp.  21–25, Jan. 2004.

S.E Qian, M. Bergeron, I. Cunningham, L. Gagnon, A Hollinger," Near lossless data compression onboard a hyperspectral satellite", IEEE Trans. Aerospace Electron. Syst, vol. 42, no. 3, pp. 851–866 , Jul. 2006.

L. Wang, J. Wu, L. Jiao, G. Shi," Lossy-to-lossless hyperspectral image compression based on multiplierless reversible integer TDLT/KLT",IEEE Geosci. Remote Sens. Lett, vol. 6, no. 3, pp. 587– 591 , Jul. 2009.

E. Candès, "Compressive Sampling," in Proc. International Congress of Mathematics, Madrid, Spain, 2006 , pp. 1433-1452.

D.Donoho, "Compressed Sensing," IEEE Transactions on Information Theory, vol. 52, no. 4, pp. 1289-1306, Apr. 2006.

R. Baraniuk, "Compressive Sensing," IEEE Signal Processing Magazine, vol. 24, pp. 118-121, Jul. 2007.

S. S. Chen, D. L. Donoho and M. A. Saunders, "Atomic Decomposition by Basis Pursuit," SIAM Review,  vol. 43, pp. 129-159, Jan. 2001.

M. Figueiredo, R. Nowak and S. Wright, "Gradient Projection for Sparse Reconstruction: Application to Compressed Sensing and Other Inverse   Problems", IEEE J. Sel. Top. Signal Process., vol. 1, no. 4, pp. 586-597, Dec. 2007.

J. Barzilai and J. Borwein, "Two-Point Step Size Gradient Methods", IMA Journal of Numerical Analysis, vol. 8, no. 1, pp. 141-148, 1988.

E. Candès and J. Romberg, "Sparsity and incoherence in compressive sampling," Inverse Problems, vol. 23, no. 3, pp. 985-969, Jun 2007.

E. Candès, J. Romberg, and T. Tao, "Robust uncertainty principles: Exact signal reconstruction from highly incomplete frequency information," IEEE Trans. Inform. Theory, vol. 52, no. 2, pp. 489-509, Feb 2006.

E. Candès and T. Tao, "Near-optimal signal recovery from random projections: universal encoding strategies," IEEE Trans. Inform. Theory, vol. 52 , no. 12, pp. 5406-5425, Dec. 2006.

T. Do, L. Gan, N. Nguyen and T. Tran, "Fast and Efficient Compressive Sensing Using Structurally Random Matrices", IEEE Transactions on Signal Processing, vol. 60, no. 1, pp. 139-154, Jan. 2012.

T.T Do, L Gan, N.H Nguyen, and T.D Tran, "Fast compressive sampling with structurally random matrices," in IEEE Int. Conf. on Acoustics, Speech and Signal Processing (ICASSP), May 2008, pp. 3369-3372.

AVIRIS project website. [Online]. http://aviris.jpl.nasa.gov/.

L.-W. Kang and C.-S. Lu, "Distributed compressive video sensing, " in IEEE Int. Conf. on Acoustics, Speech and Signal Processing (ICASSP),April 2009, pp. 1169–1172.

Basic Gradient Projection