Distributed Model Predictive Control of Discrete-Time Interconnected Systems with Time Delays

Iman Hosseini

PhD Student

School of Elect. and Comp. Eng.

Shiraz University

Shiraz, Iran

Email: Iman.hosseini@shirazu.ac.ir

S. Vahid Naghavi

Assistant Professor

Zarghan Branch

Islamic Azad University,

Zarghan, Iran

Email: naghavi@zariau.ac.ir

Ali Akbar Safavi

Professor

School of Elect. and Comp. Eng.  Shiraz University

Shiraz, Iran

Email: safavi@shirazu.ac.ir

Abstract— This paper investigates a distributed model predictive control strategy for discrete-time interconnected linear systems. The MPC controllers of subsystems communicate their information to handle the interconnection effects. It is known that distributed MPC relies on communication networks to transmit and share information among subsystems. Communication delays, resulting in delayed information exchange among subsystems, may inhibit the use of any distributed MPC approach. To address this problem, a delay handling mechanism is employed. Besides, it is showed that the proposed method guarantees input-to-state stability characterization for both local subsystems and the global system under some predetermined assumptions. The simulation results are exploited to illustrate the effectiveness of the proposed method.

Keywords-component; Distributed netwoked control system; MPC; time delay; ISS stability

# Introduction

Model predictive control (MPC) (or receding horizon control (RHC)) is a control strategy that offers attractive solutions for regulatory and tracking problem of constrained linear or nonlinear systems. MPC algorithms provide the intuitive way of addressing the control problem. In last decades, MPC has become an important research interest in control community within industry and academia. This is illustrated by its applications to a wide industrial problems and many theoretical developments. See for example [1-5].

Most processes in modern industries are physically distributed and generally composed of different subsystems, which are interconnected and characterized by significant interactions. Besides, the implementation of a control strategy for distributed systems can be achieved better in both economical and physical aspects via communication networks. In these networked systems, the information exchange through the communication networks unavoidably exhibits time delays. The study of these distributed networked control systems (DNCSs) is quite important yet a challenging issue. The MPC-based approach to study NCSs has appealing features in comparison to other approaches. First, the MPC strategy can predict the states and generate a sequence of future control signals by optimizing a control performance function at each time instant. The generated future control sequence is particularly effective in compensating for communication constraints in NCSs such as packet dropouts and delays. Second, the MPC is capable of handling various system constraints including input constraints and state constraints, which is also desirable in many NCS applications. Third, there have been a lot of applications of MPC in many industrial systems. Thus, the study of MPC for NCSs would facilitate the modification and advancement of the network-based control implementations.

It is obvious that safe operation of plants relies, among other things, on controller designs that account for the inherently complex dynamics of the processes (manifested as nonlinearities), operational issues, such as constraints and uncertainties [6], [7]. These problems force control strategy to tackle the obstacles of constraints, nonlinearities, and uncertainties. In centralized architecture, it could be challenging to address these difficulties for interconnected systems since there are computational and communication constraints. With this in mind, researchers are interested in distributed or decentralized control strategies [8-10].

The MPC algorithms are generally based on optimization control methods and usually in the centralized form [11, 12]. For centralized MPC, the computation requirement is the main obstacle to expanding the areas of applications, especially for nonlinear large-scale systems. Utilizing the distributed MPC is a solution to reduce the computation time. Optimization in distributed MPC in comparison with the centralized MPC is totally decentralized into a number of small-scale optimizations. Because of limitation in communication bandwidth, the subsystems have not full information of the neighboring subsystems in order to solve the optimization problem. Consequently, performance in centralized MPC may be better than decentralized one from this aspect, but its advantages in general are more than the centralized one. There have been many papers on distributed MPC [14-16], but not addressing the issues considered in this paper.

Subsystems are distributed in most cases; thus, it is required to share information between them. Nevertheless, if the amount of subsystems is enormous, the communication load will be huge. In [17] a distributed model predictive control is presented to improve the performance of the overall system. In this method, a local MPC is designed to control each subsystem and a reduced set of information is exchanged between these controllers via network. In [18] a framework for distributed model predictive control of discrete-time nonlinear systems with decoupled dynamics,  subject to coupled constraints and a common cooperative task is presented but delay is not included. It is assumed in some papers that information transmission can be done successfully between subsystems [22, 23, 25].  In [19] the problem of stability analysis and stabilization of the network distributed control systems (which have both random delay and random packet loss in their communication networks) is investigated, yet the system is considered to be linear. In [20] for a class of continuous-time decoupled nonlinear systems the distributed MPC problem subject to communication delays is studied, in which a waiting mechanism is used and robustness constraint is exploited to handle the time delay. Note that a described  large-scale system is associated with the communication topology/structure has been used for modeling many systems such as a group of vehicles [21], a team of robots[22, 23] and power generation systems[24].

In [26], we presented a novel approach to address the decentralized fault tolerant model predictive control of discrete-time interconnected nonlinear systems. The overall system is composed of a number of discrete-time interconnected nonlinear subsystems at the presence of multiple faults occurring at unknown time-instants. In order to deal with the unknown interconnection effects and changes in model dynamics due to multiple faults, both passive and active fault tolerant control design were considered. In this paper, we investigate a distributed model predictive strategy for discrete-time interconnected linear systems. The overall system is composed of a number of discrete-time interconnected linear subsystems. The MPC controllers communicate their information to handle the interconnection effects. It is known that distributed MPC relies on communication networks to transmit and share information among subsystems. Communication delays, resulting in delayed information exchange among subsystems, may inhibit the use of the proposed distributed MPC approach. To address this problem, a delay handling mechanism is employed [20]. Besides, it is showed that the proposed method guarantees input-to-state stability characterization for both local subsystems and the global system under some predetermined assumptions. The simulation results are exploited to illustrate the effectiveness of the proposed method.

This paper is organized as follows: In Section II preliminary definitions are presented. Section III describes the problem and formulations. In Section IV main results and delay handling mechanism are presented. Section V give the simulation results and we conclude the paper in Section VI.

# Preliminary definitions

Consider a discrete time system of the form

where x(k)∈Rn is the state, u(k)∈Rm is the control input, wk∈ W ⊂ Rl is an unknown input at discrete instant k ∈ Z+.

Definition 1: Regional ISS: The system (1) is said to be ISS in  if there exist a KL-function , a K-function  such that, for each, all W, it holds that the corresponding state trajectory of (1) satisfies

Definition 2: CLF (Control Lyapunov Function) Consider system  and a function V: Rn → R+ such that satisfies

for all x ∈ X is called a control Lyapunov function (CLF) for system

If for all x ∈ X; ∃u ∈ U such that the following holds

# Problem description

We consider a system composed of n interconnected subsystems. The ith subsystem i = 1,2,… n is described by:

where  is the state of the ith subsystem,  where   is the state of the system,  is the control action of the ith subsystem,  is the state matrix, is the input matrix and is nonlinear interconnection from other subsystems.

Assumption 1: The state and the control variables are restricted to fulfill the following constraints

,

where  and  are compact sets, both containing the origin as an interior point. Let us denote  the overall constraint set on the state.

Assumption 2: The state of the plant   can be measured at each sample time.

Assumption 3: The term  is such that

where  is a compact set containing the origin as an interior point. Let us denote the overall interconnection set.

As was stated before, information which are exchanged between the subsystems and controllers are subject to the delay that is a challenging problem in distributed systems and may result in instability of the overall system. Hence, the control goal is to design a control law  for each subsystem, which satisfies the constraints on the input and the state along the system evolution for any possible interconnections and overcome the delay problem so that the overall system moves to the origin or a neighbor of it.

# Main Results

## The Waiting Mechanism

Transmitting the information between subsystems and controllers is subject to delay. To handle distributed NCS subject to time delay and interconnections, the waiting mechanism of [20] is employed. A simple example of delay mechanism is depicted in Figure 1.

In this example, there are two subsystems. One is subsystem i, neighbors i1, i2, and i3, and the other is subsystem j, neighbor j1.

Figure 1: Example of applying the control actions according to the communication delays[20]

Suppose at time tk, the distributed system is synchronized and the optimal control inputs are applied for subsystems i and j, and the assumed state information begins to be transmitted from their neighbors to subsystems i and j. The system states are measured at time instant tk + δ. For subsystem i, the time delays from its neighbors to itself are τkii1 , τkii2 , and  , respectively. Since τkii1 < τkii2< τkii3, it has τki = τkii3 , and all the neighbors’ information has been received at time tk+1i = tk + δ + τki. For subsystem j, the time delay from its neighbor to itself is τkjj1 . Thus, tk+1i = tk + δ + τkj = tk + δ + τkjj1 . Since τkj > τki, the synchronized time of subsystems i and j will be at tk+1 = tk + δ + τkjj1 , and subsystems i and j will generate and apply the new control signals at time tk+1  ADDIN EN.CITE <EndNote><Cite><Author>Li</Author><Year>2013</Year><RecNum>28</RecNum><DisplayText>[20]</DisplayText><record><rec-number>28</rec-number><foreign-keys><key app="EN" db-id="x5dttvzfdpadtue25zsxr5zowee5rsea0zp0">28</key></foreign-keys><ref-type name="Journal Article">17</ref-type><contributors><authors><author>Li, Huiping</author><author>Shi, Yang</author></authors></contributors><titles><title>Distributed model predictive control of constrained nonlinear systems with communication delays</title><secondary-title>Systems &amp; Control Letters</secondary-title></titles><periodical><full-title>Systems &amp; Control Letters</full-title></periodical><pages>819-826</pages><volume>62</volume><number>10</number><dates><year>2013</year></dates><isbn>0167-6911</isbn><urls></urls></record></Cite></EndNote>[20]. Moreover, ISS approach is used to show the stability of the overall system at the presence of time delay.

## Distributed Model Predictive Control

In order to implement the distributed MPC algorithm a cost function is defined for the subsystem as

where  is the prediction horizon,  is the control horizon ,  and  denote the predicted state and control action of the  subsystems at step ,  and  are positive definite states control weights respectively and  are the assumed state trajectories of the  neighbors of the subsystem , 
 j are the given cooperation matrices;  are the cooperation weight.

Suppose that each subsystem i can receive information from its neighboring subsystems (neighbors) whose indices are denoted by  with  and  . The cooperation among the subsystems is achieved by designing the cooperation weights and the control objective function of (6).

Problem 1: Let in (5.7) and a CLF be given for the  subsystem. At time k measure the state and minimize the cost  over subject to the following constraints

Note that).

Let  and

denote the difference inclusion corresponding to the system without considering the disturbance in closed loop with the set of feasible solutions obtained by solving Problem 1 at each instant  .

Theorem 1:

Let for some ,  , continuous and convex CLFs  and costs   be given for all systems indexed by . Suppose Problem 1 is feasible for each subsystem and for all states. Then eachsubsystem is ISS () and the global interconnected dynamically coupled nonlinear system described by the collection of difference inclusions

is ISS(X,W).

## Proof: We have that each continuous and convex CLF Vi(xi(k)) is globally Lipchitz on Xi . Hence, assuming Li ∈ R+   is a Lipschitz constant of Vi(.) in Xi, one may write

for all x ∈ X and w ∈ W . From this property and the second inequality in (7) we have

Adding inequalities (9) and (10) yields

Thus, one can find  such that

For all .

From  or  and inequality (12) we have

Now let  and  then one can obtain

where . Then we have

For all xi0∈Xi, wi∈Wi and kϵZ+. The following inequalities hold:

From  and inequality (16) one can yield that

Then it is trivial to show that

Let us consider

where βi and γi are KL-function and K-function respectively. Consequently each subsystem is ISS(Xi,Wi) according to definition 1.

In order to prove the ISS characterization of the global system, one can consider

which denotes the difference inclusion for the global interconnected system in closed loop with the set of feasible solutions of each  subsystem.

Now let  be a CLF for the global interconnected system in .

Each continuous and convex CLF implies Lipschitz continuity on. Thus is also a Lipschitz continuous CLF for the global interconnected system. Hence, assuming   is a Lipschitz constant of    in X, one can work out that

for all. Then the ISS(X,W) of the difference inclusion (5.21) follows similar to   subsystem.

Remark: The stability proof approach can be generally used for nonlinear systems [26].

# Simulation results

The linearized discrete-time version of the interconnected system presented in [12] is considered to illustrate the design methodology for the distributed model predictive control. Thus, to study the behavior of this system via numerical simulation, this dynamic is discretized by Euler backward method and sampling time is assumed to be 10 milliseconds (i.e. T = 0.01):

The proposed control method is used to show the effectiveness of MPC-based controller. Figure 2 shows the cost function of the distributed interconnected subsystems which is decreasing in time. The distributed MPC control effort and state responses are shown in Figure 3 and Figure 4. As one sees, the state responses of different subsystems are stabilized at presence of the interconnection effects.

Moreover, the proposed distributed MPC is compared with the state feedback control method. One could see that this approach cannot even stabilize the system. Figure 5 shows that the control effort grows unbounded and Figure 6 shows state responses which diverge to infinity.

Figure 2: Cost function of subsystems in distributed MPC

Figure 3: Distributed MPC control effort

Figure 4: State response in Distributed MPC

Now consider the delay between controllers information exchange. The simulation results show that the delay effect can be handled if it is bounded. Simulation results show that the upper bound of delay is about 30 samples. Two different time delay in the case of 10 and 30 sample delay in controller information exchange is studied.

Figure 5: control effort in state feedback control

Figure 6: State response in state feedback control

In Figure 7 cost function of subsystems subject to 10 samples delay is shown. The cost functions are decreasing so that the overall system can be stabilized. Figure 8 indicates that the control efforts corresponding to each subsystem remain bounded and the control inputs are satisfied the constraint 
-20≤u≤20.

The state responses are shown in Figure 9 where all states converges to desired values. In other hand, a delay in the information exchange of each subsystem causes the control input to adapt itself so as to accommodate the delay.

Figure 7: Cost function of subsystems in distributed MPC with 10 sample delay

Figure 8: Distributed MPC control effort with 10 sample delay

Figure 9: State response in Distributed MPC with 10 sample delay

It can be deduced from Figures 7 to 9 that the system subject to 10 samples delay is stable and also the states converge to zero. To show the effectiveness of the proposed distributed MPC strategy, the delay bound is increased. Simulation results show that the closed loop system can tolerate the delay up to 30 samples which cause marginal stability. Figure 10 shows that state responses do not converge to zero and the responses are oscillatory. This oscillatory responses originate from nondecreasing cost function, and oscillatory control effort due to time delay in information exchange. From Figures 10 it can be seen that the upper bound of delay is about 30 samples. Consequently, system will be unstable with more than 30 samples delay.

Figure 10: State response in Distributed MPC with 30 sample delay

# CONCOLUSION

This paper presented an approach for distributed model predictive control with time delay. The overall system is consists of a number of discrete-time interconnected subsystems at the presence of time delay. In order to deal with problem of time delay in exchanging the information between controllers a waiting mechanism is employed, and an upper bound is considered for the delay.  Furthermore, the distributed MPC strategy was implemented for each subsystem with the MPC algorithm subject to some constraints. It was showed that the proposed method guarantees input-to-state stability characterization for both local subsystems and the global system under some predetermined assumptions. The proposed approach was investigated for a linear discrete-time system to make a practical sense.

##### References

[1]	S. J. Qin and T. A. Badgwell, "A survey of industrial model predictive control technology," Control engineering practice, vol. 11, pp. 733-764, 2003.

[2]	R. Findeisen, L. Imsland, F. Allgower, and B. A. Foss, "State and output feedback nonlinear model predictive control: An overview," European journal of control, vol. 9, pp. 190-206, 2003.

[3]	E. F. Camacho and C. B. Alba, Model predictive control: Springer Science & Business Media, 2013.

[4]	A. Rahideh and M. H. Shaheed, "Stable model predictive control for a nonlinear system," Journal of the Franklin Institute, vol. 348, pp. 1983-2004, 2011.

[5]	J. Figueroa, S. Biagiola, M. Alvarez, L. Castro, and O. Agamennoni, "Robust model predictive control of a Wiener-like system," Journal of the Franklin Institute, vol. 350, pp. 556-574, 2013.

[6]	M. Lazar, W. Heemels, and A. Jokic, "Self-optimizing robust nonlinear model predictive control," in Nonlinear Model Predictive Control, ed: Springer, 2009, pp. 27-40.

[7]	M. Lazar, D. M. De La Peña, W. Heemels, and T. Alamo, "On input-to-state stability of min–max nonlinear model predictive control," Systems & Control Letters, vol. 57, pp. 39-48, 2008.

[8]	H. Wu, "Decentralized adaptive robust control for a class of large scale systems with uncertainties in the interconnections," International Journal of Control, vol. 76, pp. 253-265, 2003.

[9]	X. Xiao and Z. Mao, "Decentralized guaranteed cost stabilization of time-delay large-scale systems based on reduced-order observers," Journal of the Franklin Institute, vol. 348, pp. 2689-2700, 2011.

[10]	P. Panagi and M. M. Polycarpou, "Decentralized fault tolerant control of a class of interconnected nonlinear systems," Automatic Control, IEEE Transactions on, vol. 56, pp. 178-184, 2011.

[11]	B. Gu and Y. P. Gupta, "Control of nonlinear processes by using linear model predictive control algorithms," ISA transactions, vol. 47, pp. 211-216, 2008.

[12]	B. Zhang, W. Yang, H. Zong, Z. Wu, and W. Zhang, "A novel predictive control algorithm and robust stability criteria for integrating processes," ISA transactions, vol. 50, pp. 454-460, 2011.

[13]	M. Abu-Ayyad and R. Dubay, "Real-time comparison of a number of predictive controllers," ISA transactions, vol. 46, pp. 411-418, 2007.

[14]	E. Camponogara, D. Jia, B. H. Krogh, and S. Talukdar, "Distributed model predictive control," Control Systems, IEEE, vol. 22, pp. 44-52, 2002.

[15]	R. Scattolini, "Architectures for distributed and hierarchical model predictive control–a review," Journal of Process Control, vol. 19, pp. 723-731, 2009.

[16]	P. D. Christofides, R. Scattolini, D. M. de la Pena, and J. Liu, "Distributed model predictive control: A tutorial review and future research directions," Computers & Chemical Engineering, vol. 51, pp. 21-41, 2013.

[17]	Y. Zheng, S. Li, and N. Li, "Distributed model predictive control over network information exchange for large-scale systems," Control engineering practice, vol. 19, pp. 757-769, 2011.

[18]	M. A. Müller, M. Reble, and F. Allgöwer, "Cooperative control of dynamically decoupled systems via distributed model predictive control," International Journal of Robust and Nonlinear Control, vol. 22, pp. 1376-1397, 2012.

[19]	F. Kazempour and J. Ghaisari, "Stability analysis of model-based networked distributed control systems," Journal of Process Control, vol. 23, pp. 444-452, 2013.

[20]	H. Li and Y. Shi, "Distributed model predictive control of constrained nonlinear systems with communication delays," Systems & Control Letters, vol. 62, pp. 819-826, 2013.

[21]	A. Richards and J. How, "Robust distributed model predictive control," International Journal of Control, vol. 80, pp. 1517-1531, 2007.

[22]	W. B. Dunbar and R. M. Murray, "Distributed receding horizon control for multi-vehicle formation stabilization," Automatica, vol. 42, pp. 549-558, 2006.

[23]	E. Franco, L. Magni, T. Parisini, M. M. Polycarpou, and D. M. Raimondo, "Cooperative constrained control of distributed agents with nonlinear dynamics and delayed information exchange: A stabilizing receding-horizon approach," Automatic Control, IEEE Transactions on, vol. 53, pp. 324-338, 2008.

[24]	A. N. Venkat, I. A. Hiskens, J. B. Rawlings, and S. J. Wright, "Distributed MPC strategies with application to power system automatic generation control," Control Systems Technology, IEEE Transactions on, vol. 16, pp. 1192-1206, 2008.

[25]	T. Keviczky, F. Borrelli, and G. J. Balas, "Decentralized receding horizon control for large scale dynamically decoupled systems," Automatica, vol. 42, pp. 2105-2115, 2006.

[26]	S. V. Naghavi, A. Safavi, and M. Kazerooni, "Decentralized fault tolerant model predictive control of discrete-time interconnected nonlinear systems," Journal of the Franklin Institute, vol. 351, pp. 1644-1656, 2014.