\documentclass[conference]{IEEEtran}

%\usepackage{doublespace}

%\usepackage{float} %allows the [H] tag to stop figures from "floating"

\usepackage{amsmath,amssymb,amsthm, amscd,amsfonts,mathtools}

\usepackage{graphicx}

\usepackage{latexsym}

\usepackage{amsmath}

\usepackage{amsthm}

\usepackage{amssymb,amsmath}

\usepackage{indentfirst}

\date{}

\pagestyle{plain}

\begin{document}

% author names and affiliations

% use a multiple column layout for up to three different

% affiliations

\title{Two Algorithms for Spread-Spectrum Sequence Estimation for DSSS

Signals in Non-Cooperative Communication Systems}

\author{\IEEEauthorblockN{Saeed Mehboodi,Ali Jamshidi,

Mahmoud Farhang}

\IEEEauthorblockA{Department of Electrical and Computer Engineering,

Shiraz University\\ Email: {\tt saeedmehboodi@yahoo.com \quad,\quad jamshidi@shirazu.ac.ir,\quad mfarhang@shirazu.ac.ir} }

}

\author{\IEEEauthorblockN{Saeed Mehboodi}

\IEEEauthorblockA{Department of Electrical and\\ Computer Engineering\\

Shiraz University\\

Email: saeedmehboodi@yahoo.com}

\and

\IEEEauthorblockN{Ali Jamshidi}

\IEEEauthorblockA{Department of Electrical and\\ Computer Engineering\\

Shiraz University\\

Email: jamshidi@shirazu.ac.ir}

\and

\IEEEauthorblockN{Mahmoud Farhang}

\IEEEauthorblockA{Department of Electrical and\\ Computer Engineering\\

Shiraz University\\

Email: mfarhang@shirazu.ac.ir}}

\maketitle

\begin{abstract}

Two estimation algorithms for the spreading sequence in DSSS signals are proposed. Some of previously proposed algorithms have some drawbacks such as large computational complexity, low estimation accuracy and poor accuracy by increasing the length of spreading sequence period. In this paper by using an initial estimate of chips, we can improve recursively the final decision with low computational complexity. The results show that our proposed algorithms have better estimation accuracy even for large number of chips.

\end{abstract}

\IEEEpeerreviewmaketitle

\section{Introduction}

Spread spectrum signals have been used for secure communications and also in multiple access communications. Spread spectrum communication has strong ability of anti-interference and resistance to intercept, so, it is also widely used in wireless communication systems \cite{viterbicdma,flikkema1997spread}.

The spread spectrum transmitter uses a periodical pseudo-noise (PN) sequence to modulate the baseband signal before transmission. In the receiver, by having the PN code, despreading operation can be used in order to retrieve the data symbols. In non-cooperative communication systems, the PN sequence is unknown to the receiver; therefore, the estimation of PN sequence is crucial to information reception.

Some algorithms have been proposed in this respect. An eigenvalue decomposition (EVD) algorithm has been proposed for the estimation of PN sequence in \cite{burel2000blind}. This algorithm has some problems such as huge computational complexity and partial-encode problem. The problem of partial-encoding, has been solved in \cite{wu2010blind,wang2012blind}. In these algorithms, after sequence synchronization the autocorrelation matrix of the received data is computed, then, the spreading sequence can be recovered from the eigenvector of the biggest eigenvalue. Although these algorithms overcome the partial-encode problem, but they have large computational complexity, yet.

In \cite{zhan2005spread}, an innovative algorithm has been proposed to estimate the PN sequence. In this algorithm, after sequence synchronization; first chip of the PN sequence is fixed to +1 or -1 and by multiplying the first received symbol to the second one the sign of data is omitted and second chip is estimated. This algorithm has low computational complexity but it has two problems. At first, it has low estimation accuracy and the second one, is that by increasing the length of PN period the estimation accuracy gets worse.

In this paper, in order to solve the problems in \cite{zhan2005spread} two algorithms are proposed. In the first proposed algorithm, which is called algorithm 1, by using an initial estimation of chips, we can improve recursively the final decision. Although this algorithm improves the estimation accuracy greatly and its accuracy does not decrease by increasing the length of PN period, but, its deficiency is the computational complexity. For reducing the computational complexity of algorithm 1, an algorithm is proposed that is called algorithm 2. In this algorithm only a fraction of estimated chips are used in the next chips’ estimation. Simulation results show a little performance degradation by using this method and we have a large reduction in computational complexity.

The remainder of this paper is organized as follows. In Section \ref{system model}, notations and signal model are presented. Then, the proposed algorithm in \cite{zhan2005spread} and two new algorithms are demonstrated in Section \ref{proposed algorithms}. Computational complexity discussion is presented in section \ref{computational complexity discussion}. Section \ref{simulation results} is devoted to a performance analysis of algorithms using computer simulations and finally, section \ref{conclusion} concludes the paper.

\section{System Model}\label{system model}

Without loss of generality, we assume perfect timing and carrier synchronization are achieved. Period of the spreading sequence, chip rate and baud rate can be estimated using correlation characteristics of PN sequence \cite{yu2012chip,tianqi2007estimation,tianqi2001new,miao2009period}.The received DSSS signal can then be written in a discrete-time form as:

\begin{equation}

r(kT_{c})=\sqrt{S} d(kT_{c})c(kT_{c})+n(kT_{c})

\end{equation}

where $\{d(kT_c)\}$ denotes the original data symbol with symbol interval $T$, $\{c(kT_c)\}$ is the spread spectrum sequence with chip interval $T_{c}$ and $T=NT_{c}$ ($N$ is the period of the PN sequence). $S$ denotes the signal power and $\{ n(kT_{c})\}$ is additive white Gaussian noise (AWGN) with zero mean and variance $\sigma^{2}_{n}$. For convenience, $T_c$ is omitted in the following and the spread spectrum sequence will be represented by $\{c_1 , \ldots , c_N\}$, and $d_i$ denotes the $i$th transmitted symbol.

Since the starting point of the received data is not always the first chip of the pseudo-noise signal, sequence synchronization is necessary to determine the position of starting chip $(c_{1})$. Some algorithms have been proposed for sequence synchronization based on correlation characteristics of the PN sequence \cite{wu2010blind,wang2012blind,zhan2005spread}. In this paper, we assume perfect sequence synchronization is achieved.

\section{Proposed algorithms}\label{proposed algorithms}

This section has two subsections. In section \ref{proposed algorithm 1} initially the proposed algorithm in \cite{zhan2005spread}, is explained briefly and algorithm 1 is proposed to improve the performance of PN estimation. In section \ref{proposed algorithm 2}, algorithm 2 is proposed in order to reduce the computational complexity of the first proposed algorithm.

\subsection{Proposed Algorithm 1}\label{proposed algorithm 1}

After sequence synchronization, and normalizing signal power to $1$, the first $ LN$ received symbols can be written in a $L\times N$ matrix as follows:

\begin{equation}

\mathbf{R}=

\begin{pmatrix}

d_1c_1+n_{1,1} &d_1c_2+n_{1,2} &\cdots&d_1c_N+n_{1,N} \\

d_2c_1+n_{1,1} &d_2c_2+n_{1,2} &\cdots&d_2c_N+n_{2,N} \\

\vdots&\vdots&\vdots&\vdots\\

d_Lc_1+n_{1,1} &d_Lc_2+n_{1,2} &\cdots&d_Lc_N+n_{L,N} \\

\end{pmatrix}

\label{matrix1}

\end{equation}

Where each row of $ \mathbf{R}$ contains a whole period of the PN sequence. In the algorithm that was proposed in \cite{zhan2005spread}, the authors assumed $c_{1}=1$, and for estimation of $c_{i}$, $ (2 \leq i \leq N)$ the first column of R is multiplied to the second column element-wise, and the summation of all products is computed; that is

\begin{equation}

F(c_{i})=\sum_{j=1}^{L} r_{j,1}r_{j,i}^\ast

\end{equation}

Then, we have

\begin{equation}

\hat{c_{i}}=\mathrm{sgn}(\mathrm{Re}(F(c_{i})))

\end{equation}

where $ \mathrm{sgn}(x) $ is the sign function with value $1$, if $x>0$ and $-1$ if $x<0$.

It’s clear that in this algorithm, for computing each chip only the first chip is used. By utilizing the other estimated chips the estimation can be improved. Algorithm 1 is proposed as follows:

We assume $c_1=1$ and estimate $c_2$ like the algorithm that was proposed in \cite{zhan2005spread}, then the estimated value of $c_1$ is multiplied to the second column of $ \mathbf{R}$. If $c_2$ was estimated correctly, by multiplying $c_2$ in the second column of $\mathbf{R}$, we have:

\begin{equation}

(d_ic_2+n_{i,2})\hat{c_{2}}=d_i+n_{i,2}\hat{c_{2}}

\end{equation}

Note that $n_i$ is AWGN, therefore:

\begin{equation}

n_{i,2}\hat{c_{2}}\equiv{ n_{i,2}}

\end{equation}

Now, the elements in the second column of $\mathbf{R}$ are replaced by the new values, then, for estimation of $c_3$ the first and second columns of $ \mathbf{R}$ are multiplied to the third column element-wise separately and the summation of all products is computed; that is:

\begin{equation}

F(c_{3})=\sum_{j=1}^{L} r_{j,1}r_{j,3}^\ast + \sum_{j=1}^{L} r_{j,2}r_{j,3}^\ast

\end{equation}

Then,

\begin{equation}

\hat{c_{3}}=\mathrm{sgn}(\mathrm{Re}(F(c_{3})))

\end{equation}

Similarly, for estimating $c_{i}$ the estimated chips $(2 , \cdots , i-1)$ are multiplied to the corresponding columns and used in the estimation process as following:

\begin{equation}

F(c_{i})=\sum_{j=1}^{L} r_{j,1}r_{j,i}^\ast + \cdots + \sum_{j=1}^{L} r_{j,i-1}r_{j,i} ^\ast

\end{equation}

Then,

\begin{equation}

\hat{c_{i}}=\mathrm{sgn}(\mathrm{Re}(F(c_{i})))

\end{equation}

\subsection{Proposed Algorithm 2}\label{proposed algorithm 2}

In the proposed algorithm 1 most of error probability is related to the estimation of the beginning chips, therefore, by accurate estimating the beginning chips, other chips can be estimated with large accuracy, because the estimated values of earlier chips are used to estimate the next chips. Based on these observations, algorithm 2 is proposed for reducing the computational complexity as follows:

First chip is assumed to be $+1$. Chips $\{c_2 ,c_3 , \ldots , c_m\}$ are estimated like algorithm 1, where $(m\ll{N})$. It is clear that, if chips $\{c_2 ,c_3 , \ldots , c_{m-1}\}$ are estimated correctly, $c_{m}$ can be estimated with a great accuracy in algorithm 1, therefore $\hat{ c_{m}}$ is the most accurate estimated chip and $\hat{ c_{2}}$ is the least accurate one.

Now, for estimating $c_{m+1}$, we omit $\hat{ c_{2}}$ and use the estimated values of $\{c_1 ,c_3 ,c_4, \ldots , c_m\}$. Note that $c_1$ is fixed to $1$ and assumed to be error free and will be used in all estimations. Every estimated chip is multiplied to its corresponding column and the new values are replaced in $\mathbf{R}$, then the new values of $\mathbf{R}$ are used in the estimation of $c_{m+1}$, so, for estimating $c_{m+1}$, we have:

\begin{equation}

F(c_{i})=\sum_{j=1}^{L} r_{j,1}r_{j,m+1}^\ast + \sum_{j=1}^{L} r_{j,3}r_{j,m+1} ^\ast+ \cdots + \sum_{j=1}^{L} r_{j,m}r_{j,m+1} ^\ast

\end{equation}

\begin{equation}

\hat{c_{m+1}}=\mathrm{sgn}(\mathrm{Re}(F(c_{m+1})))

\end{equation}

Similarly, for estimating $c_{i}$ $ (m+1 \leq i \leq N)$, instead of using all the estimated chips, only the first chip (that is error free) and the $(m-1)$ chips before $c_{i}$ i.e. $c_{j}$ $ (i-m-1 \leq j \leq i)$, will be used in the estimation. Therefore, for estimating each chip only $ m$ or fewer columns of $\mathbf{R}$ will be used. Simulation results show a little performance degradation and we have a large reduction in computational complexity.

\section{Computational complexity discussion}\label{computational complexity discussion}

In this section computational complexity of the proposed algorithms and some other algorithms is presented. One of the best criterions that is used for computational complexity analysis is the number of additions and multiplications; therefore this criterion is used for comparison of algorithms complexity in this paper.The number of additions and multiplications of all algorithms depend on the period of spreading sequence and the number of bits that are used in the estimation. For some algorithms some other factors affect the complexity.

The proposed algorithm in \cite{zhan2005spread}, estimates every chip separately. For each chips’ estimation, $L$ multiplications and $ L-1$ additions are needed. We have $N-1$ chips’ estimation (note that $c_{1}$ is fixed to $+1$ and not estimated), therefore for all chips, $(N-1)L$ multiplications and about $ (N-1)(L-1)$ additions are needed.

In EVD algorithm that is proposed in \cite{wu2010blind}, first, for computing the autocorrelation matrix of the received data $N^2L$ multiplications and about $N^2L$ additions are needed , which is more than all of computational complexity of most of other algorithms. After the autocorrelation matrix computation, eigenvalue decomposition is needed, which has additions and multiplications of order of $N^3$ in theory, but the complete eigenvalue decomposition is not necessary and only the biggest eigenvalue and its corresponding eigenvector is used to PN estimation which can be computed with less complexity \cite{sorensen1992implicit}.

Two algorithms were proposed in this paper.

In algorithm 1, $L$ multiplications and $(L-1)$ additions are needed for estimating $c_2$.for estimating $c_3$, we need $2L$ multiplications and about $2(L-1)$ additions. Similarly $(N-1)L$ multiplications and $(N-1)(L-1)$ additions are needed for estimating $c_N$. Finally we need $\frac{N(N-1)}{2}L$ multiplications and $\frac{N(N-1)}{2}(L-1)$ additions in algorithm 1.

Algorithm 2 was proposed in order to reduce the computational complexity of algorithm 1.In this method the computational complexity of estimation of the first $m$ chips is equal to complexity of estimation of these chips algorithm 1. For example, there are $(m-1)L$ multiplications and about $(m-1)(L-1)$ additions are needed for estimating $c_m$, but the estimation complexity does not increase for estimating $c_{i}$, $ (m+1 \leq i \leq N)$. Finally, in algorithm 2 we need about $\frac{(m-1)m}{2}+(N-m)L$ multiplications

and about $\frac{(m-1)m}{2}+(N-m)(L-1)$ additions.

Table \ref{table1} demonstrates a comparison of four algorithms.

\begin{table}

\begin{center}

\caption{Computational complexity comparison}

\label{table1}

\begin{tabular}{lcc}

\hline Algorithm& number of multiplications &

number of additions \\

\hline

Algorithm proposed in \cite{zhan2005spread}&$ (N-1)L $&$ (N-1)(L-1)$ \\

\hline

Algorithm 1& $\frac{N(N-1)}{2}L$& $\frac{N(N-1)}{2}(L-1)$\\

\hline

Algorithm 2&$\frac{(m-1)m}{2}+(N-m)L$ & $\frac{(m-1)m}{2}+(N-m)(L-1)$ \\

\hline

EVD algorithm&$N^2L+N^3$ &$N^2L+N^3$ \\

\hline

\end{tabular}

\end{center}

\end{table}

\section{Simulation results}\label{simulation results}

In this section, simulation results are presented to demonstrate the performance of the algorithms. Signal is produced based on the model in section \ref{system model}. Only PN estimation is evaluated and sequence synchronization is assumed to be perfect. In this paper, we have used BPSK modulation for data symbols, i.e. $d_i\in\pm1$.

At first, the capability of accurate detection of the PN sequence is evaluated for some algorithms.

The period of the PN sequence is assumed to be $63$ and for all algorithms, $200$ data symboles of the PN sequence is used in the estimation $(L=200)$.

In algorithm 2 the factor $m$ is equal to $15$ in this comparison. $ 200 $ trials of simulation were carried out and curves are shown in figure \ref{fig1}. Note that the detection error is defined as the percentage of incorrect PN sequences estimation in the whole trials. If one chip is estimated incorrectly, the whole sequence is assumed to be wrong. As it shown in figur \ref{fig1}, there is a great difference between the algorithm that was proposed in \cite{zhan2005spread} and the proposed algorithms in this paper. EVD algorithm shows a great performance but it has huge computational complexity that is not operational in large periods of the PN sequence.

\begin{figure}

\centering

\includegraphics[width=95mm]{det1.eps}

\caption{Comparison of algorithms in terms of detection error}

\label{fig1}

\end{figure}

In figure \ref{fig2}, the other problem of the algorithm that is proposed in \cite{zhan2005spread} is evaluated and the curves of detection error per sequence period are presented for all algorithms.Since the detection error for algorithms are not equal, SNR is different for some algorithms in order to show all curves in one figure. SNR is equal to $-12dB$ for EVD algorithm, it is $-7dB$ for the algorithm was proposed in \cite{zhan2005spread} and it is $-10dB$ for algorithm 1 and algorithm 2.

This figure shows that the performance of the proposed algorithms are not changed noticeably by increment of the period of the PN sequence and they can be used for large periods of the PN sequence with acceptable complexity.

\begin{figure}

\centering

\includegraphics[width=95mm]{vsn1.eps}

\caption{Detectoin error vs. N for SNR= $-12$ for the EVD algorithm and SNR= $-8$ for algorithm was proposed in \cite{zhan2005spread} and SNR= $-8$ for algorithm 1 and algorithm 2}

\label{fig2}

\end{figure}

In figure \ref{fig3}, the reduction of computational complexity is evaluated. In this evaluation the PN period is $N=63$, and proposed algorithm 2 with $ m=\{15,30,45,63\}$, are compared. The results show that the performances of the different values of $m$ are not different so much and we can have a tradeoff between the computational complexity and the accuracy in the PN estimation.

\begin{figure}

\centering

\includegraphics[width=95mm]{algorithm22.eps}

\caption{Detectoin error vs. SNR for algorithm 1 and algorithm 2 for different values for different valus of $m$ }

\label{fig3}

\end{figure}

It is clear that the final purpose of PN estimation is data estimation. Therefore in this part after the estimation of PN sequence the receiving symbols are despreaded with the estimated PN sequence that is estimeted by different algorithms and transmitted data is extracted. Figure \ref{fig4} demonstrates the error probability of transmitted data using foer methods.The parameters are equal to ones that were used in figure \ref{fig1}. This figure shows that there is a little difference between the EVD algorithm and proposed algorithm, while the EVD algorithm have huge computational complexity in compare with the proposed algorithms.

\begin{figure}

\centering

\includegraphics[width=95mm]{datae2.eps}

\caption{Estimation error of transmitted data, after despreading vs SNR for $N=63$}

\label{fig4}

\end{figure}

\section{Conclusion}\label{conclusion}

Two estimation algorithms for the spreading sequence in DSSS signals are proposed. The first algorithm shows good estimation accuracy in compare with the similar algorithms. In order to reduce the computational complexity of the first proposed algorithm another algorithm is proposed that demonstrates a little performance degradation and large reduction in computational complexity.

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%______________________________figures_____________________________________

\bibliographystyle{IEEEtran}

\bibliography{mlimp}

\end{document}