A Low Leakage Grid Connected Inverter Based on HERIC Topology

Adib Abrishamifar, Ehsan Karimi, Arman Najafizadeh

School of Electrical Engineering, Iran University of Science and Technology, Tehran, Iran

Abrishamifar@iust.ac.ir, Ekarimi@elec.iust.ac.ir, Najafizadeh@elec.iust.ac.ir

Abstract— Leakage current in photovoltaic systems is an important issue that besides reducing the immunity of the system, results in electromagnetic interference and reduces the efficiency of the system. This problem has led to many topologies in order to reduce leakage current. The difference among these topologies are the efficiency and reliability. In this paper, an inverter based on HERIC topology is proposed that uses an LCL filter to reduce leakage current dramatically. The proposed structure along with reduced leakage current, has the ability to inject reactive power into grid. Low number of elements, high efficiency and reliability and simple modulation manner are from other features of the proposed structure.

Keywords- leakage current, electromagnetic interference, system efficiency, reliability.

# Introduction

Today the use of renewable energy has been expanded due to easy access and also causing no pollution [1]. Photovoltaic energy is a clean energy. Photovoltaic power generation systems usually include solar panels and power conversion unit [2], [3]. The inverter can deliver maximum power produced by solar panels [4], [5] and inject a sinusoidal current to grid [6]. Today, the use of PV grid connected inverters has significantly increased in comparison with independent inverters.

In many regions and countries, to create isolation between photovoltaic panels and grid, low-frequency transformers in AC side or high frequency transformers in DC side are used. But this causes the cost, size and weight of the system to increase and a further decreases efficiency [8].

Transformerless inverter topologies due to high efficiency and low cost, are very attractive [9]. The main constraint of transformerless inverters is leakage current due to parasitic capacitance of solar panels. In inverters with transformer because of a break in current loop, there is no leakage current [10], [11].

Risk of electric shock, conduction and radiation interferences, reducing efficiency and quality of injection current are problems caused by the leakage current [12]. These reasons have made the designers of PV grid connected inverters to duplicate their efforts to reduce leakage current.

# leakage current and its elimination manners

General scheme of a transformerless PV grid connected inverter is shown in ‎Figure 1.

General scheme of PV trnsformerless grid connected inverter

In transformerless inverters, there is electric connection between PV array and the grid. Due to presence of parasitic capacitances between PV modules and earth, variation of inverter common mode voltage causes to flow leakage current in this path. In (1) and (2) we define common mode and differential mode voltages, respectively.

vCM=v1N+v2N2                                                                          (1)

vDM=v1N-v2N=v12                                                              (2)

By assuming inductors Lac1 and Lac2 to be equal in both the inverter to the grid connections as well as the capacitors C1 and C2 that are parasitic capacitance of switches and their heat sinks connection, a simple model of inverter common mode circuit is shown in ‎Figure 2.

Simple common mode model of  a PV grid connected inverter

Different manners to reduce leakage current have been proposed by researchers. One of them is to separate PV array and grid in freewheeling mode. Another method that has been used in many of topologies, is fixing the common mode voltage of the  circuit. also, other work has been done that we can point  to using of passive and active filters.

(a) H5 and (b) H6 structure

As shown in ‎Figure 3. , H5 [13] and H6 [14] topologies by disconnecting PV array from the grid in freewheeling mode, can reduce the leakage current. The problem with this topologies is that by disconnecting the grid from PV array, A and B points will be floated and their voltages will depend on junction capacitance of switches.

The voltage of points A and B of H5 and H6 topologies with presence of switches junction capacitance can be expressed as follow [14]:

uAN=uBN=C2+C5C2+C5+C4Udc                                                 (3)

If all junction capacitances of switches be equal, common mode voltage in freewheeling mode compare to active modes will be deferent.

The Other disadvantage of the structures shown in ‎Figure 3.  is that conduction losses are high because of the many switches on the path of the current in active mode. HERIC structure is shown in ‎Figure 4. (a), known as high efficiency. This structure for separating the grid from solar panels use two switches S5 and S6 have been in reverse and parallel connection with grid. Each of these switches is turned on in a half cycle and passing current in freewheeling time. Low switching frequency for the switches S5 and S6 as well as the presence of only two switches in current path in active mode causes high efficiency in this structure.

One problem of the structure is the inability to inject reactive power into the grid. In this paper to create this ability, modulation is used to turn on the switches S5 and S6 in the freewheeling time, return current passes of two switches passes S5 and S6. The modulation and switching method of the inverter is shown in ‎Figure 4. (b). Although this type of modulation, provide the ability to reactive power injection, but to increase switching frequency switches S5 and S6, the switches have more losses and efficiency is reduced compared to conventional modulation of the structure.

(a) HERIC structure (b) HERIC structure modulation to make the ability to inject reactive power

# Analysis of the impact of junction capacitors of switch in the freewheeling mode FOR the HERIC structure

HERIC structure like the structure of the H5 has 4 modes. It has two disconnected mode and two other states are connected mode. When we switch from a state of connected to a disconnected mode, the junction capacitor of switch determines VAN and VBN voltages. In this part for the analysis of this issue, transient condition of positive active mode to freewheeling mode in the positive half cycle is investigated.

‎Figure 5. ‎Figure 5. shown HERIC structure with junction capacitors of switches in transient condition from positive active state to freewheeling mode. In ‎Figure 5. ‎Figure 5. (b) equivalent circuit is shown and in parentheses is the initial voltage of the capacitors. Therefore it is seen that capacitor C4 is charged with inductors filter and C2 is also charged in reverse. Hence voltage point A is reduced and voltage of point B will also increase. This process continues until the voltage between the two points are equal. At the end of this process, the S6 switches and parallel diode of S5 switch are in conduction mode.

(a) HERIC circuit in transiant situation from positive active mode to freewheeling mode (b) equvalent circuit

(a) HERIC transiant circuit in freewheeling mode (b) equvalent circuit

‎Figure 6.  shown HERIC structure with junction capacitors of switches in freewheeling mode.

According to this circuit and the primary voltage capacitors, voltage V in ‎Figure 6. (b) shows the relationship (4) will be achieved.

VAN=VBN=2C2+C1-C4C1+C2+C3+C4UDC                                                    (4)

This relationship indicates that if all junction capacitors of switches have a similar amount,   the voltage points A and B will be equal to 1/2 UDC which means no change in common mode voltage to the positive and negative modes.

However, because in practice, the junction capacitors of the switch depends on the voltage across the drain-source or the collector-emitter, so we cannot expect that common mode voltage to be constant. But we can certainly say that the HERIC structure compared with the structures of H5 and H6, has more stable common mode voltage.

# modified inverter structure

‎Figure 7. ‎Figure 7. shows the structure of the proposed inverter. Proposed inverter is based on HERIC structure. As mentioned in section ‎III, points A and B voltages of HERIC structure in freewheeling mode with taking into consideration the junction capacitors of switches, take 1/2(UPV) approximately. This has an influence on the stability of common mode voltage inverter and reducing of leakage current in all situations.

Also another advantage of this structure, is its high efficiency that is due to the low conduction losses in the freewheeling mode and active states. These reasons led to the HERIC to select as the basic structure.

Proposed inverter structure

Because the structure is connected to the grid, a harmonic LCL filter must be used in addition to the use of EMI filter. This harmonic LCL filter with the EMI filter inductor has the potential to reduce leakage current. CDM_P and two split CDM_S are the differential mode capacitors. As shown in ‎Figure 7. , LCM inductor, which is part of the EMI filter, placement between first stage of differential inductor (Lac1) and CDM_S divided capacitors. Then the midpoint of divided CDM_S capacitors connected to midpoint of DC capacitors. The filter structure was used in references [15] and [16] with two different design method in full bridge structure based on unipolar modulation. Full bridge structure with unipolar modulation has high leakage current that using the filter cannot reduce leakage current in an acceptable way.

For a better understanding of reducing leakage current in this structure, the common mode model is shown in ‎Figure 8. In this model, a low-pass filter is formed that result reduced leakage current. Common mode voltage of negative supply line by equation (5) is obtained. In this equation, ωr is resonant frequency of the LC circuit and is equal to 10.5Lac1+LCM2CDMS||2Cdcs. Usually ωr is much higher than 120Hz. Lower amount of ωr cause vCM been further weakened. Value of the attenuation in terms of dBV is obtained from (6).

vN=-vCMω1-ω20.5Lac1+LCM×2CDMS||2Cdcs=-vCMω1-ω2ωr2                 (5)

Attenω=20log101-ω2ωr2                                                                        (6)

Common mode model of proposed structure

As shown in ‎Figure 7. , because of a common mode route in the used filter, a common mode current (ic) is created in this path. The current can be estimated by the (7). As seen the current consist of two frequency components: the switching frequency component and low frequency components.

ic=vCMω×kω    ,     kω=ω1-ω2ωr22CDMS||2Cdcs          (7)

Because the resonance frequency is much smaller than the switching frequency, more common mode noise instead of passing through 2CDMS||2Cdcs, pass from the LCM , so the ic current is very small and filter does not need to damper resistance.

# filter structure

In order to optimize filter performance, a design method is presented. ‎TABLE I. , shown the inverter's specifications.

Inverter system specification

Designed filter value

The first inductor of filter (Lac1) is achieved by imposing restriction for rippling current (Lac1) [17]. Equation (8) shows the restrictions.

2Vdc16fsLac1=∆ipp_max<25%Prate240                                                       (8)

CDM capacitor is restricted with reactive power level. For the inverter, 2.5% of the total power inverter is considered. From equations (9), CDM capacitance range is obtained. Also The CDM_S and CDM_P capacitors calculated according to equation (10).

CDM<2.5%Prate2πfoVac2                                                                              (9)

0.5CDM_S+CDM_P=CDM                                                                      (10)

Second inductors of harmonic filter (Lac2) based on the injection harmonic current level is designed [18]:

-vDM(ω)2ω-ω2Lac1Lac2CDM+Lac+Lac2=IGω<IR                    (11)

Select the CDM_S and LCM can be achieved by adjusting the amount ωr that ωr, is selected according to required weakening value. Usually ωr is chosen less than one-tenth of resonance frequency that ic current be weakened and Does not need to use a damper resistance.

# simulation

To simulate the proposed inverter and regarding the inverter specifications that listed in ‎TABLE I. , The filter with the characteristics shown in ‎TABLE II. is designed. For parasitic capacitors of Solar panels, two 5nF capacitors in the positive and negative ends of the solar panels are placed. Circuit shown in ‎Figure 8. ‎Figure 8. , is a model that leakage current is measured by it.

Simulation results with presence of the LCL filter and without LCL filter shown in ‎Figure 9. ‎Figure 9. and ‎Figure 10. ‎Figure 10.  respectively. As seen in the figures, common mode current 20KHz component, which is called leakage current, has about 60dB reduction in presence of LCL filter.

Leakage current of proposed structure

Leakage current of HERIC structure without LCL filter

# Conclusion

In this paper, an inverter based on the HERIC structure and with use of LCL filter, with the aim of reducing the leakage current is proposed. In the proposed inverter by preserving the HERIC structure advantage as the high efficiency and reliability, the ability to inject reactive power is added. Also the leakage current of the circuit, thanks to use either the HERIC structure or LCL filter together is greatly reduced. Another benefit of this structure, is its simple SPWM modulation.

# Reference

[1]	I. Patrao, E. Figueres, F. González-Espín, and G. Garcerá, “Transformerless topologies for grid-connected single-phase photovoltaic inverters,” Renew. Sustain. Energy Rev., vol. 15, no. 7, pp. 3423–3431, 2011.

[2]	S. B. Kjaer, J. K. Pedersen, and F. Blaabjerg, “A review of single-phase grid-connected inverters for photovoltaic modules,” Ind. Appl. IEEE Trans., vol. 41, no. 5, pp. 1292–1306, 2005.

[3]	B. Sahan, A. N. Vergara, N. Henze, A. Engler, and P. Zacharias, “A single-stage PV module integrated converter based on a low-power current-source inverter,” Ind. Electron. IEEE Trans., vol. 55, no. 7, pp. 2602–2609, 2008.

[4]	N. Femia, G. Petrone, G. Spagnuolo, and M. Vitelli, “A Technique for Improving P&O MPPT Performances of Double-Stage Grid-Connected Photovoltaic Systems,” IEEE Trans. Ind. Electron., vol. 56, no. 11, pp. 4473–4482, Nov. 2009.

[5]	F. Liu, S. Duan, F. Liu, B. Liu, and Y. Kang, “A Variable Step Size INC MPPT Method for PV Systems,” IEEE Trans. Ind. Electron., vol. 55, no. 7, pp. 2622–2628, Jul. 2008.

[6]	J. M. Carrasco, L. G. Franquelo, J. T. Bialasiewicz, E. Galván, R. C. P. Guisado, M. A. M. Prats, J. I. León, and N. Moreno-Alfonso, “Power-electronic systems for the grid integration of renewable energy sources: A survey,” Ind. Electron. IEEE Trans., vol. 53, no. 4, pp. 1002–1016, 2006.

[7]	I. E. A. PVPS, “Trends in photovoltaic applications,” Surv. Rep. Sel. IEA Ctries. between, 1992.

[8]	H. Xiao and S. Xie, “Leakage current analytical model and application in single-phase transformerless photovoltaic grid-connected inverter,” Electromagn. Compat. IEEE Trans., vol. 52, no. 4, pp. 902–913, 2010.

[9]	E. Gubía, P. Sanchis, A. Ursúa, J. López, and L. Marroyo, “Ground currents in single-phase transformerless photovoltaic systems,” Prog. Photovoltaics Res. Appl., vol. 15, no. 7, pp. 629–650, Nov. 2007.

[10]	J. M. A. Myrzik and M. Calais, “String and module integrated inverters for single-phase grid connected photovoltaic systems-a review,” in Power Tech Conference Proceedings, 2003 IEEE Bologna, 2003, vol. 2, p. 8–pp.

[11]	M. Calais, V. G. Agelidis, and M. Meinhardt, “Multilevel converters for single-phase grid connected photovoltaic systems: an overview,” Sol. Energy, vol. 66, no. 5, pp. 325–335, Aug. 1999.

[12]	H. Xiao and S. Xie, “Transformerless split-inductor neutral point clamped three-level PV grid-connected inverter,” Power Electron. IEEE Trans., vol. 27, no. 4, pp. 1799–1808, 2012.

[13]	S. Bremicker, F. Greizer, U. Hübler, and M. Victor, “Method of converting a direct current voltage from a source of direct current voltage, more specifically from a photovoltaic source of direct current voltage, into a alternating current voltage.” Google Patents, 12-Aug-2008.

[14]	B. Yang, W. Li, Y. Gu, W. Cui, and X. He, “Improved transformerless inverter with common-mode leakage current elimination for a photovoltaic grid-connected power system,” Power Electron. IEEE Trans., vol. 27, no. 2, pp. 752–762, 2012.

[15]	D. Dong and F. Luo, “Leakage current reduction in a single-phase bidirectional AC–DC full-bridge inverter,” Power Electron. IEEE …, vol. 27, no. 10, pp. 4281–4291, 2012.

[16]	D. Dong, “Low-frequency leakage current reduction using active control of single-phase PWM rectifier,” … Convers. Congr. …, pp. 3778–3785, 2011.

[17]	H. Kim and K. Kim, “Filter design for grid connected PV inverters,” 2008 IEEE Int. Conf. Sustain. Energy Technol., vol. 2, no. 1, pp. 1070–1075, Nov. 2008.

[18]	O. Garcia, J. Cobos, and R. Prieto, “Single phase power factor correction: a survey,” … , IEEE Trans., vol. 18, no. 3, pp. 749–755, 2003.