Different Topologies for Linear Switched Reluctance Motor with Segmental Translator

Babak Ganji, Mohamad Hasan Askari

Faculty of Electrical and Computer Engineering, University of Kashan, Iran

bganji@kashanu.ac.ir, mha.askari@gmail.com

Abstract— Linear switched reluctance motor (LSRM) with segmental translator is introduced in this paper and two different topologies are presented for this type of motor. In addition, the electromagnetic modeling of these new types of LSRM is done using ANSYS finite element (FE) package. Carrying out 2D FE transient analysis in the developed simulation models, the dynamic characteristics of these two motors are predicted precisely. The simulation models are created totally in ANSYS parametric design language as parametric models and they can be used easily for different designs. The simulation results are presented for the two new structures and they are compared to those obtained for the conventional LSRM.

Keywords— Linear switched reluctance motor; segmental translator; finite element transient analysis

I.   Introduction

The switched reluctance motor (SRM) has many advantages for instance high reliability and robustness, simple structure, lack of permanent magnet and winding on the rotor, low manufacturing cost and large output power/weight ratio. In addition, this motor is inherently suitable for high-speed drives in high-temperature and hazardous environments because of its ruggedness. Due to these significant characteristics, special attention has been paid to this motor in the last three decades [1]. The structure and performance principles of LSRMs are very similar to that for the rotary SRM and consequently it can be used appropriately for traction force application because of all above-mentioned features. However, the researchers have concentrated mostly on different aspects of the rotary SRM and not many works have been reported for modeling and design of LSRMs.

A longitudinal field LSRM with multiple air gaps is introduced and designed in [2]. The transverse flux LSRMs are described in [3-5] and their inductances and force are calculated using finite element analysis. A linear switched reluctance actuator with cylindrical form is introduced and constructed in [6]. The double-sided, double-translator LSRM is described and manufactured in [7]. A magnetic equivalent circuit and a standard design algorithm are introduced for a single-sided and longitudinal flux-based LSRM in [8]. A conventional LSRM is designed and manufactured in [9] for a test track of autonomous railway vehicles. Describing a different structure for the conventional types of LSRM in [10], it is constructed and applied to the positioning tasks.

At the following, two new topologies are introduced for the LSRM in section II. Using ANSYS FE package, the simulation electromagnetic models are developed for them in section III and they are then used in section IV in order to present simulation results for these two motors. Finally, the paper is concluded in section V.

II.   New Topologies For LSRM

The rotary SRM with segmental rotor has been already introduced in [11] in which the rotor is made of separate segments instead of a toothed rotor. The stator structure and the winding connections in this motor are similar to those in the fully-pitched SRM which is described in [12]. The magnetic flux paths for the fully-pitched SRM are shorter than those in the short-pitched SRM and consequently its phase inductance is larger. This leads to higher torque for the SRM with segmental rotor in comparison to the conventional SRM [13]. Due to this significant feature, introducing segmental translator linear switched reluctance motor (STLSRM) is considered and two different topologies of this motor are developed here based on the works done for the rotary SRM in [11, 14].

A.  The fully-pitched STLSRM

The cross-section of the three-phase STLSRM with fully-pitched winding is shown in Fig. 1.  Each slot is completely filled with conductors related to one phase and each coil encloses three stator poles. The translator is included a series of separate laminated segments instead of a toothed translator. Aligned and unaligned positions for this motor are presented in Fig. 2. In the aligned position, a slot is closed magnetically by a segment and as a result the magnetic flux path only encloses a stator slot. This results in increasing the aligned inductance in comparison to the conventional LSRM.  For the unaligned position depicted in Fig. 2-b, flux must flow between segments which are magnetically insulated from each other. The translator segments are designed using below criteria:

1) With regard to Fig. 2-a, the length of overlap between each translator segment and the stator pole in the aligned position should be equal to the stator pole width. In addition, height of the segment equals to the stator pole width. These result in having the same flux densities for stator pole, air-gap and translator segments.

2) To minimize the unaligned inductance, gap between translator segments is equal to the slot opening length as it can be observed in Fig. 2-b.

Fig. 1. Three-phase STLSRM with fully-pitched winding

(a)

(b)

Fig. 2. Important positions for the fully-pitched STLSRM, (a) aligned position, (b) unaligned position

B.  The short-pitched STLSRM

Another possibility for STLSRM is that created using the short-pitched winding as it is shown for the three-phase STLSRM in Fig. 3. Similar to the fully-pitched STLSRM, each slot is filled completely with conductors of a coil related to a phase but each coil is wrapped here around one stator pole. As it is obvious from Fig. 3, there are two different types for stator pole in the short-pitched STLSRM: double-width one having coil and the standard-width one without coil. The second type is required to provide a return path for flux when the related phase is excited.

Dimensions of translator segments are selected based on the rules similar to those given for the fully-pitched STLSRM. It is noted that the width of the wound stator pole is twice the width of the unwound stator pole. In addition, the overlap length between one translator segment and the unwound stator pole in the aligned position (Fig. 4-a) is equal to half of the wound stator pole width. The heights of both the stator core and the translator segments are the same and they are equal to the width of the unwound stator pole because they only carry the flux of one unwound stator pole.

Fig. 3. Cross section of the short-pitched STLSRM

(a)

(b)

Fig. 4. Important positions for the short-pitched STLSRM, (a) aligned position, (b) unaligned position

III.   Electromagnetic Modeling

In order to predict the dynamic characteristics of the introduced segmental translator linear switched reluctance motors, the electromagnetic modeling is done here using ANSYS FE package. Carrying out 2D FE transient analysis, the important characteristics of the two motors described at above are predicted in the developed simulation models. In the modeling, the geometrical structure of the motor is build up in the FE model as a parametric model using ANSYS parametric design language and therefore it can be applied easily to different designs. Widths and heights of the poles, widths of the slots, depths of stator and translator cores, core stack thickness, air-gap length, number of poles and number of turns per phase are chosen as the geometrical parameters. A time-stepping analysis is carried out for prediction of the dynamic characteristics. The coupling equations between the stator and translator in the middle of the air gap are cleared at each time step, translator is moved by 0.1 mm, and then the coupling equations are rearranged and the analysis is restarted. More details about method of modeling the electric machine using ANSYS FE package are available in [15].

IV.   Simulation Results

According the design procedure described in [8], a three-phase conventional LSRM whose cross-section is shown in Fig. 5 is proposed and its specifications are given in Table I. As it is clear from this figure, the three-phase conventional LSRM is equivalent of a rotary 6/4 SRM with two extra poles for translator due to reduction of core loss and noise. The windings areas related to one phase have been only plotted on this figure because one phase excitation is considered in the done modeling. This motor is called the proposed LSRM at the following.

Fig. 5. Geometric model of conventional type of LSRM

A three-phase STLSRM with fully-pitched winding is considered here in which the stator dimensions, number of turns per phase, and air-gap length are chosen the same with those in the proposed LSRM. For the selected stator, translator is then designed based on the rules presented on section II-a. At the following, this motor is called the discussed fully-pitched STLSRM. In addition, a three-phase STLSRM with short-pitched winding is considered here which its critical design parameters including stator pole width, air-gap length and number of turns per phase are selected similar to those for the considered fully-pitched STLSRM. The other geometric parameters of this motor are then obtained using the rules given in section II-b. This motor is titled the discussed short-pitched STLSRM at the remaining of the paper.

Similar to the procedure described in section III, modeling of the proposed LSRM is done and then 2D FE transient analysis of this motor is carried out for the operating point given in Table II. The predicted instantaneous thrust force waveform is shown in Fig. 6. The calculated average thrust force is 5 N (853 N/m3) and the computation time is about 15 min on a 2.27 GHz Intel Core 2 with 4GB RAM. The flux-lines representation predicted for the aligned position is shown in Fig. 7 and it is noted that the flux encloses three stator poles.

Table I.        Specifications of The Proposed LSRM

| Stator pole width [mm] | 46 |
| --- | --- |
| Stator slot width [mm] | 46 |
| Stator pole height [mm] | 67 |
| Stator core depth [mm] | 46 |
| Air-gap length [mm] | 1 |
| Translator pole width [mm] | 49 |
| Translator slot width [mm] | 89 |
| Translator core depth [mm] | 49 |
| Core stack thickness [mm] | 125 |
| Turns per phase | 160 |

Fig. 6. Thrust force waveform predicted for the proposed LSRM

Table II.        Motor Control Parameters

| Phase voltage [V] | 100 |
| --- | --- |
| Speed [m/s] | 2 |
| Turn-on angle [electrical degree] | 54 |
| Turn-off angle [electrical degree] | 125 |
| Current limitation [A] | 5 |

(a)

(b)

Fig. 7. Predicted flux-lines of the proposed LSRM: (a) unaligned position, (b) aligned position

Using the electromagnetic model described in section II, 2D FE transient analysis of the discussed fully-pitched STLSRM is carried out for the considered operating point and thrust force waveform is predicted and given in Fig. 8. Having the predicted thrust force waveform, it is possible to calculate the average thrust force which is 35.3 N (21393 N/m3). With regard to the flux-lines representation predicted for the aligned position (Fig. 9), it is obvious that flux route is shorter than that obtained for the proposed LSRM. In addition, there are only 4 segments on translator which makes it lighter than the proposed LSRM.

For the considered operating point, the 2D FE transient analysis of the discussed short-pitched STLSRM is also completed and the predicted instantaneous thrust force waveform is shown in Fig. 10. The average force derived from the predicted thrust force waveform is 19.5 N (4756 N/m3). The flux-lines representation predicted for the aligned position is shown in Fig. 11 and it is observed that the flux path is shorter in comparison to the proposed LSRM.

Fig. 8. Thrust force waveform predicted for the discussed fully-pitched STLSRM

(a)

(b)

Fig. 9. Predicted flux-lines representation of the discussed fully-pitched STLSRM: (a) unaligned position, (b) aligned position

Fig. 10. Thrust force waveform predicted for the discussed short-pitched STLSRM

(a)

(b)

Fig. 11. Predicted flux-lines representation of the discussed short-pitched STLSRM: (a) unaligned position, (b) aligned position

V.   Conclusion

Introducing linear switched reluctance motor with segmental translator, the two various topologies for this motor titled the fully-pitched STLSRM and the short-pitched STLSRM were described and the design criteria for them were identified. Using ANSYS finite element package, electromagnetic modeling of these two motors was done to predict their dynamic characteristics such as instantaneous thrust waveform. The developed electromagnetic simulation models were created totally in ANSYS parametric design language as parametric models and they can be utilized easily for various designs.

Based on an available design algorithm, a typical three-phase conventional linear switched reluctance motor was proposed and its instantaneous thrust waveform was predicted using 2D finite element transient analysis. Considering a fully-pitched STLSRM and a short-pitched STLSRM with the specifications similar to those for the proposed conventional linear switched reluctance motor, their instantaneous thrust waveforms were predicted using the developed electromagnetic simulation models. For the considered operating point, the discussed fully-pitched STLSRM showed the best performance and its force/weight ratio was 25 times larger than that of the proposed conventional linear switched reluctance motor. Due to smaller translator and higher force/weight ratio, the introduced fully-pitched STLSRM could be considered appropriately in some special applications.

References

- T. J. E. Miller, “Switched reluctance motors and their control”, Magna Physics Publishing and Clarendon Press: Oxford, 1993.

- D. Matt, R. Goyet, J. Lucidarme, and C. Rioux, “Longitudinal-field multi-airgap linear reluctance actuator,” Electric Mach. Power Syst., vol. 13, no. 5, pp. 299–313, 1987.

- K. Takayama, Y. Takasaki, R. Ueda and T. Sonoda, “Thrust force distribution on the surface of stator and rotor poles of switched reluctance motor”, IEEE Trans. Magn., vol. 25, pp. 3997–3999, 1989.

- P. M. Cusack, G. E. Dawson, and T. R. Eastham, “Design, control and operation of a linear switched reluctance motor,” in Proc. Canadian Conf. Electrical and Computer Engineering, Quebec, PQ, Canada, pp. 19.1.1–19.1.5, 1991.

- C. T. Liu and Y. N. Chen, “On the feasible polygon classifications of linear switched reluctance machines,” in Proc. IEEE Int. Electric Machines and Drives Conf., Milwaukee, WI, pp. TB1/11.1–TB1/11.3, 1997.

- J. Corda and E. Skopljak, “Linear switched reluctance actuator”,  6th International  Conference on Electrical Machines and Drives, Oxford, U.K., pp. 535–539, 1993.

- U. S. Deshpande, J. J. Cathey and E. Richter, “A high force density linear switched reluctance machine”, IEEE Trans. Ind. Appl., vol.31, pp. 345–352, 1995.

- B. Lee, H. Bae, P. Vijayraghavan and R. Krishnan, “Design of a linear switched reluctance machine”, IEEE Trans. Indus. Appl., vol. 36, no. 6, pp. 1571-80, 2000.

- L. Kolomeitsev1 and et. al., “Linear switched reluctance motor as a high efficiency propulsion system for railway vehicles”, International Symposium on Power Electronics, Electrical Drives, Automation and Motion, pp. 155-60, 2008.

- A. Santo, M. Calado, C. Cabrita, “Design and evaluation of a linear switched reluctance actuator for positioning tasks”, Turk J Elec Eng & Comp Sci, vol.18, no.6, pp.925-41, 2010.

- B. C. Mecrow, J. W. Finch, E. A. El-Kharashi and A. G. Jack, “Switched reluctance motors with segmental rotors”, IEE Proc.-Electr. Power Appl., vol. 149, no. 4, pp. 245-254, 2002.

- B. C. Mecrow, “Fully pitched-winding switched reluctance and stepping-motor arrangements”, IEE Proc.-Electr. Power Appl., vol. 140, no. 1, pp. 61-70, 1993.

- C. Laudensack, Q. Yu, D. Gerling, “Analysis of different winding topologies for canned switched reluctance machines”, International Conference on Electrical Machines (ICEM), pp. 66-72, 2012.

- B. C. Mecrow, E. A. El-Kharashi, J. W. Finch and A.G. Jack, “Segmental rotor switched reluctance motors with single-tooth windings”, IEE Proc.-Electr. Power Appl., vol. 150, no. 5, pp. 591-599, 2003.

- J. Faiz, B. Ganji R. W. De Doncker and J. Fiedler, “Electromagnetic modeling of switched reluctance motor using finite element method”, IECON'06, France, pp. 1557-1562, 2006.