Curve Fitting, Filter Bank and Wavelet Feature Fusion for Classification of PCG Signals

Maryam Imani

Faculty of Electrical and Computer Engineering

Tarbiat Modares University

Tehran, Iran

maryam.imani@modares.ac.ir

Hassan Ghassemian

Faculty of Electrical and Computer Engineering

Tarbiat Modares University

Tehran, Iran

ghassemi@modares.ac.ir

Abstract—The use of efficient feature extraction methods is very important to correctly classify the heart sound signal and to diagnosis the heart disease. In this paper, we propose two feature extraction algorithms for feature extraction of cardiac phonocardiography (PCG) signal. The both methods use the sequence discipline of PCG obtained by curve fitting model. In the first and the second methods, the sequence information is fused with features extracted by filter banks and by wavelets respectively. We used a dataset of PCG signals which contains the heart sounds of 98 persons (40 cases without heart disease and either no murmur or an innocent murmur and 58 cases with a variety of cardiac diagnoses and a pathologic systolic murmur). The experimental results show the efficiency of our proposed methods compared to some popular feature extraction methods from five different classification accuracy measures point of view.

Keywords—curve fitting; filter bank; classification; cardiac phonocardiography (PCG).

- INTRODUCTION

## After cerebrovascular accident, the heart disease is the second mortality since 1985. The heart failure and coronary heart disease are the main cardiac problems which cause deaths. Because of high concerning about health management, the home medical instruments, especially for heart disease diagnosis, have a rapid development in the recent years. A cheap and efficient method for cardiac disorders diagnosing is interpreting of the heart sound [1]-[2]. The heart sound signal characteristics are changed according to types of pathological conditions in the heart. Auscultation is the monitoring of sounds heard over the chest walls, which is usually done by the stethoscope. The heart sound can be measured with analog or digital stethoscope or by phonocardiography. Stethoscope is a simple tool to transmit the heart sounds from the chest wall to the ear, but, the correct use of it is not easy due to some reasons: 1- The human ear is not sufficiently sensitive to detect all frequencies. So, some components of signal can be

## omitted. 2- A long time practice and experience of the doctors is required to make diagnosis. 3- The signal acquired by an acoustic stethoscope has very less sound amplitude. According to aforementioned reasons, it is a need to provide a device, which can make an automatic recording and interpretation of a heart sound signal. Feature extraction, which is an important step in pattern recognition problems [3]-[4], has a main role in classification of heart sounds. Principal component analysis (PCA) [5] and wavelet transform are two popular methods for feature extraction to determine the abnormality of the heart signal [6]. The Mel Frequency Cepstral Coefficient (MFCC) values are cepstral features based on mel-scale filter banks and can be considered as good features when the time and frequency domain resolutions are the same for all the heart sound classes [7]-[8]. A mathematical model of systolic murmurs based on multivariate matching pursuit to extract crucial features for identifying the heart diseases has been proposed in [9]. Authors in [10] proposed a segmentation of cardiac sound signals into heart beat cycles using tunable-Q wavelet transform which can remove the murmurs from cardiac sound signals. A time-frequency approach and an automated spectral analysis for pediatric cardiac auscultation have been proposed in [11] and [12] respectively.

There is a special sequence in a cycle of heart sound. A normal cardiac phonocardiography (PCG) cycle contains four major parts. The systolic pause segment is after the first heart sound (S1), and diastolic pause period is after the second heart sound (S2).  The third heart sound (S3), the fourth heart sound (S4) and the heart murmurs may be heard in systolic interval and diastolic interval parts. There is information in the sequence of heart sounds in time domain, frequency domain, and time-frequency domain. The extraction of these features can simple the classification of normal and abnormal PCG signals. Finding an appropriate model to fit the curve of a PCG signal (curve fitting) [13] and extraction of related features from the obtained model and fusing these features with other features obtained by filter banks or wavelets is the basic idea of our proposed algorithms in this paper. We suggest two algorithms for feature extraction from a heart sound. The first proposed algorithm use the fusion of features extracted by curve fitting and filter banks and the second algorithm uses the fusion of features extracted by curve fitting and wavelet.

The reminder of this paper is continued as follows: In section two, we introduce the proposed methods. Section III represents the experimental results and discussions. Finally, section IV gives a conclusion of this paper.

- PROPOSED ALGORITHMS

We propose two fusion based algorithms for feature extraction and classification of heart sound (PCG). At first, we extract features from PCG signal using curve fitting, filter banks, and wavelets. Then, we fuse the extracted features using an appropriate way.

- Curve fitting

Curve fitting is a useful and traditional method to find the mathematical relationship between the observed signal and independent variables. Curve fitting finds a function f(n) for the data n, sn;n= 1, 2,…,N which minimizes the distance between the samples of data and f(n). The Least squares (LS) method minimizes the following weighted mean squared error to find f(n):

1Nn=1Nwnfn-s(n)2                          (1)

where wn (n= 1, 2,…,N) are the weights. Different curve fitting models like polynomial, linear, spline, etc can be used. A nonlinear curve fitting using rational functions is utilized in this paper [14].

Assuming two integers L ≥ 0 and M ≥ 0, the rational function approximant of order (L, M) for function f is defined as follows:

fn=j=0Lcj+M+1nj1+j=1Mcjnj                             (2)

If we consider equal models for two signals, then, differences between signals curves yield differences in the coefficients of fitted curves. Therefore, these coefficients can be used as features for discrimination between different signals. So, the obtained coefficients are used as new features for classification.

We minimize the following function to determine the coefficients vector c=[c1 c2 …cM+L+1]T:

φ=1Nn=1Nfn-f(n)2                      (3)

By computing the partial derivatives of φ with respect to the coefficients, we obtain a system of nonlinear equations and set them to zero. A sufficient (not necessary) condition to solve this equation system is:

j=0Lcj+M+1nj1+j=1Mcjnj=fn;      n=1,2,…, N             (4)

Now, a system of linear equations with M+L+1 unknowns and N equations can be obtained as:

j=0Lcj+M+1nj-fnj=1Mcjnj=fn

;n=1,2,…, N                                 (5)

The above equations can be rewritten in the matrix form as follows:

AN×M+L+1CM+L+1×1=BN×1                   (6)

where

AN×M+L+1=anj                       (7)

anj=-fnnj               ;j=1,…,Mnj;  j=M+1,…,M+L+1                 (8)

and

B=f1,f2,…,fNT                      (9)

We extract M+L+1<<N coefficients as new features. The matrix A is not square. So, the Moore-Penrose pseudo inverse of A is used to find C such that the norm of AC-B be minimized.

- Filter bank

For feature extraction using filter banks, at first, we estimate the power spectrum of PCG signals for c class using periodogram method: pif;i=1,…,c. Then, the entropy of power spectrum is calculated as follows:

Af=-i=1cpiflog2pif               (10)

Entropy is a criterion for measuring the uncertainty.  So, in each f that the entropy Af is maximum, there is more information for discrimination between different classes. A filter bank can be designed using the obtained entropy curve. For extraction of m features from signal, we choose m picks of entropy curve among the local maximums. Each frequency f corresponding to each pick is considered as a filterʼs center in the filter bank. The width of filter is manually selected proportionate with the entropy curve in the pickʼs location. Then, We

Fig. 1. The first proposed algorithm.

Fig. 2. The second proposed algorithm.

apply the PCG signal to this filter bank. The energy of each output (corresponding to each filter) is considered as an extracted feature.

- Fusion

Fig. 1 and Fig. 2 show the first and the second proposed algorithms respectively. In the first algorithm, after applying a fast Fourier transform (FFT), which transform the signal space from time domain to frequency domain, we find an appropriate model to fit the sequence of heart sound in the frequency domain. Moreover, we extract features from signal using filter banks. After that, we fuse the features extracted by curve fitting and features extracted by filter banks in a vector using stacking. The feature vector is given to a classifier to diagnose that signal is normal or abnormal. In the second proposed algorithm, at first, using an appropriate wavelet, we decompose the PCG signal to two parts: approximation and details. We extract m1 and m2 features from the approximation and detail parts, using curve fitting, respectively. Moreover, using wavelet, we decompose the original signal to m3-1 levels. The energy of detail parts of all obtained levels in addition to energy of the approximation part of last level provide a m3-dimensional feature vector. After that, we fuse the m1+m2 features extracted by curve fitting with m3 features extracted by wavelet. The final feature vector, which contains m1+m2+m3 features is given to a classifier.

- EXPERIMENTS AND DISCUSSIONS

An electronic stethoscope was used during the auscultation. The digital recordings were made at a sampling frequency of 44.1 KHZ with 16 bit precision and saved onto a computer in the uncompressed WAV file format. For each patient, a 10 second heart sound recording was made at the Apex in the supine position. It is important to locate the microphone of stethoscope in the appropriate location on the surface of body. Cardiac auscultatory examinations of 98 persons were recorded, digitized, and stored along with corresponding echocardiographic diagnoses. 40 cases were diagnosed as normal and the other 58 cases were found to have one of the pathological cardiac conditions namely ventricular septal defect (VSD), atrial septal defect (ASD) and pulmonary stenosis (PS). Some samples of normal, PS, ASD, and VSD PCG signals are shown in Fig. 3. In this paper, we classify the PCG signals into two groups: normal and abnormal cases.

In our experiments, we use two different classifiers: the maximum likelihood (ML) classifier with Gaussian distribution and the support vector machine (SVM) with polynomial kernel and default parameters defined in LIBSVM [15]. We used the symlet wavelet with order 8 (sym8) in our experiments. In the filter bank methods, Gaussian filters, with a width of 21 HZ, are chosen. Fig. 4 shows the selection of 6 local maximums as filtersʼs centers as an example.

We use 5 different measures to assess the classification accuracy: average accuracy, average reliability, specificity, sensitivity, and overall accuracy. The accuracy and reliability for each class are calculated by

Fig. 3. Samples of PCG signals related to normal and abnormal cases.

Fig. 4. The entropy of PCG signals.

Acc=N/A and Rel=N/B respectively where N denotes the number of testing samples which are correctly classified, A is the total testing samples of that class and B is the total samples which are labeled as that class. Specificity and sensitivity are the percentage of correct diagnosis of normal class and abnormal class respectively. Overall accuracy is the percentage of total correctly classified samples.

Table I shows the classification results of PCG signals by 7 extracted features using 5 training samples and SVM classifier. The first proposed method provides the maximum average accuracy, average reliability, and overall accuracy. From the sensitivity and specificity criteria point of view, curve fitting and filter banks have the best performance respectively. The second proposed method has the highest overall accuracy after the first proposed method. Table II contains the classification results with 20 training samples and SVM classifier. The number of extracted features by each method is shown in the parentheses. The first proposed method has the best performance with all criteria except sensitivity. The second proposed method is better than curve fitting and wavelet method with average accuracy, average reliability and overall accuracy measures. The results of classification with ML classifier and 5 training samples are shown in Table III. The first proposed method is the best from average accuracy and overall accuracy point of view and the second proposed method is better than curve fitting in the most cases. Table IV shows the classification results with ML classifier and 20 training samples. The first and the second propose methods obtain the best results (with all criteria except specificity) respectively.

In general, we can see that the first proposed method has the best performance with average accuracy, average reliability, and overall accuracy. Moreover, the second proposed method has better performance than curve fitting and wavelet methods in the most cases.

Table I. The classification results obtained by SVM classifier, 5 training samples, and 7 extracted features.

|  | Proposed 1 | Proposed 2 | Curve fitting | Filter bank | Wavelet | PCA |
| --- | --- | --- | --- | --- | --- | --- |
| Average Accuracy | 66.49 | 64.28 | 62.72 | 64.92 | 64.72 | 59.16 |
| Average Reliability | 66.65 | 65.38 | 64.92 | 66.08 | 64.49 | 62.32 |
| Specificity | 68.50 | 66.50 | 52.00 | 69.50 | 66.00 | 49.00 |
| Sensitivity | 64.48 | 62.07 | 73.45 | 60.34 | 63.45 | 69.31 |
| Overall Accuracy | 66.12 | 63.88 | 64.69 | 64.08 | 64.49 | 61.02 |

Table II. The classification results obtained by SVM classifier and 20 training samples.

|  | Proposed 1 (15 features) | Proposed 2 (15 features) | Curve fitting (9 features) | Filter bank (6 features) | Wavelet (15 features) | PCA (15 features) |
| --- | --- | --- | --- | --- | --- | --- |
| Average Accuracy | 82.21 | 78.46 | 71.24 | 79.40 | 78.38 | 78.86 |
| Average Reliability | 81.73 | 78.57 | 70.96 | 78.93 | 77.66 | 81.46 |
| Specificity | 81.67 | 74.17 | 68.33 | 78.33 | 79.17 | 67.50 |
| Sensitivity | 82.76 | 82.76 | 74.14 | 80.46 | 77.59 | 90.23 |
| Overall Accuracy | 82.31 | 79.25 | 71.77 | 79.59 | 78.23 | 80.95 |

Table III. The classification results obtained by ML classifier, 10 training samples, and 6 extracted features.

|  | Proposed 1 | Proposed 2 | Curve fitting | Filter bank | Wavelet | PCA |
| --- | --- | --- | --- | --- | --- | --- |
| Average Accuracy | 72.08 | 69.72 | 64.68 | 70.06 | 70.98 | 70.53 |
| Average Reliability | 73.56 | 73.68 | 65.58 | 71.33 | 74.25 | 74.36 |
| Specificity | 84.50 | 91.50 | 73.50 | 81.50 | 93.00 | 94.50 |
| Sensitivity | 59.66 | 47.93 | 55.86 | 58.62 | 48.97 | 46.55 |
| Overall Accuracy | 69.80 | 65.71 | 63.06 | 67.96 | 66.94 | 66.12 |

Table IV. The classification results obtained by ML classifier and 20 training samples.

|  | Proposed 1 (15 features) | Proposed 2 (15 features) | Curve fitting (9 features) | Filter bank (6 features) | Wavelet (15 features) | PCA (15 features) |
| --- | --- | --- | --- | --- | --- | --- |
| Average Accuracy | 81.49 | 77.14 | 72.52 | 70.09 | 75.35 | 74.44 |
| Average Reliability | 81.51 | 78.61 | 72.94 | 70.58 | 77.29 | 77.41 |
| Specificity | 88.50 | 96.00 | 84.00 | 79.50 | 94.50 | 97.50 |
| Sensitivity | 74.48 | 58.28 | 61.03 | 60.69 | 56.21 | 51.38 |
| Overall Accuracy | 80.20 | 73.67 | 70.41 | 68.37 | 71.84 | 70.20 |

- CONCLUSION

Two feature extraction methods are proposed in this paper, which fuse the information contained in the sequence of PCG signals with features extracted by filter banks and wavelets respectively. Experiments are done by SVM and ML classifiers using a two-class database of heart sounds. The results show that the first proposed method has generally the best performance compared to the second proposed method, curve fitting, filter bank, wavelet, and PCA methods. Moreover, the second proposed method is superior to curve fitting and wavelet methods in the most cases.

REFERENCES

- M. SK, B. JL Jr, C. MA, “Teaching cardiac auscultation: effectiveness of a patient-centered teaching conference on improving cardiac auscultatory skills,” in Mayo Clinic Proceeding, vol. 80, no. 11, pp.1443-1448, Nov 2005.

- C. Papadaniil and L. Hadjileontiadis, “Efficient heart sound segmentation and extraction using ensemble empirical mode dcomposition and kurtosis features,” IEEE Journal of Biomedical and Health Informatics, vol. 18, no.4, pp.1138-1152, 2014.

- M. Imani, H. Ghassemian, “Feature Extraction Using Median-Mean and Feature Line Embedding, International Journal of Remote Sensing,”vol. 36, no. 17, pp. 4297-4314, 2015.

- M. Imani, H. Ghassemian, “Feature space discriminant analysis for hyperspectral data feature reduction,” ISPRS Journal of Photogrammetry and Remote Sensing, vol. 102, pp. 1-13, 2015.

- S. Akbari and H. Ghassemian, “Feature Extraction Based on Power Spectrum Estimation and PCA algorithm for Heart Sound Classification,” 12th Iranian Conference on Intelligent Systems (ICIS), Bam, Iran, Feb. 2014 (Printed in Persian).

- M.I. Abdalla and H. S. Ali, “Wavelet-Based Mel-Frequency Cepstral Coefficients for Speaker Identification using Hidden Markov Models,” Journal of Telecommunications, vol. 1, no. 2, pp. 16-21, March 2010.

- S. Akbari and H. Ghassemian, “Heart Sound Classification Using MFCC and SVM classifier,” 5th Conference on Information and Knowledge Technology (IKT 2013), Shiraz, Iran, May 2013 (Printed in Persian).

- L. Muda, M.Begam and I. Elamvazuthi, “Voice Recognition Algorithms using Mel Frequency Cepstral Coefficient (MFCC) and Dynamic Time Warping (DTW) Techniques,” Journal of Computing, vol. 2, no. 3, pp. 138-143, March 2010.

- S. Jabbari, H. Ghassemian, “ Modeling of heart systolic murmurs based on multivariate matching pursuit for diagnosis of valvular disorders,” Computers in Biology and Medicine , vol. 41, pp. 802–811, 2011.

- S. Patidar, R. B. Pachori, “Segmentation of cardiac sound signals by removing murmurs usingconstrained tunable-Q wavelet transform,” Biomedical Signal Processing and Control, vol. 8, pp. 559– 567, 2013.

- S. Jabbari and H. Ghassemian, “A Time-Frequency Approach for Discrimination of Heart Murmurs,” J. Signal and Information Processing, vol. 2, no. 3, pp. 232-237, 2011.

- A.R. Kenari and H. Ghassemian, “Automated spectral analysis for pediatric cardiac auscultation,”  21st Iranian Conference on Electrical Engineering (ICEE 2013), Mashhad, Iran, May 2013.

- L. Fang and D.C. Gosard, “Multidimensional curve fitting to unorganized data points by nonlinear minimization,” Computer Aided Design, vol. 27 no. 1, pp.48-58, 1995.

- S. A. Hosseini, H. Ghassemian, “A Novel Approach to Hyperspectral Data Feature Extraction Using Rational Function Curve Fitting,” 2015 IEEE International Conference on Signal and Image Processing Applications (ICSIPA), Kuala Lumpur, Malaysia, pp. 494-499, Oct. 2015.

- C. Chang and C. Linin, LIBSVM—A Library for Support Vector Machines, 2008. [Online]. Available: http://www.csie.ntu.edu.tw/~cjlin/libsvm.