Impact of Load Power Factor on Sympathetic Inrush Current

Arash Moradi

Master Student

Department of Electrical Engineering

University of Isfahan

Isfahan, Iran

Email: arashmoradi@eng.ui.ac.ir

Seyed M. Madani

Assistant Professor

Department of Electrical Engineering

University of Isfahan

Isfahan, Iran

Email: m.madani@eng.ui.ac.ir

Ramtin Sadeghi

Phd Student

Department of Electrical Engineering

University of Isfahan

Isfahan, Iran

Email: r.sadeghi@eng.ui.ac.ir

Abstract—This paper investigates the impact of load power factor on sympathetic inrush current. Sympathetic inrush current is caused by the voltage drop across the system resistance due to energizing current of the parallel adjacent transformer. It has destructive effect on the protection system of transformers, and it is a reason for many of transformers outages. In this paper, the model is validated by simulation using DIgSILENT. Further, results are shown that sympathetic inrush current’s magnitude is changed by different load power factors. The causes of this changes are illustrated and analyzed in order to reach a deeper comprehension and understanding of sympathetic inrush current that is useful for transformer protection applications.

Keywords-component; Inrush Current; Sympathy Inrush Current; Load Power Factor; Inductive Load; DIgSILENT;

Introduction

Sympathetic inrush current is one of the side effects of energizing transformer. When two transformers are paralleled to each other that one of them had energized before inrush currents is created by energizing another transformer and unsymmetrical voltage drops across system resistance happen. Voltage drop is happened due to a DC offset of the energizing current. The DC offset causes unexpected saturation condition and it is named as magnetizing current. It is obvious that when a system has higher resistance, the voltage drop of the system is greater. So it can cause worse saturation condition and higher sympathetic inrush current [1].

Some studies analyses the transformer inrush current transient and assuming that there is no other transformer that is connected to the system. The assumption is not true, because in practice, energizing transformers is conducted either in parallel or in series with other adjacent transformers. So it is necessary to consider adjacent transformers. Sympathetic inrush current has effect on magnitude and duration of the inrush current. The sympathetic inrush current of adjacent transformers creates damping time of energizing inrush current that is not Specified [1-3].

Numerous investigations on the inrush current phenomenon in power transformers and its impact on the design and operation of protection schemes are available in [4-5]. two real mal-operation of protection system due to sympathetic inrush current is analyzed and investigated in [6-7]. Most of the approaches are based on the electrical equivalent circuit [8-10] or magnetic equivalent circuit [11-13] but they neglect certain adjacent devices in the electrical system which could have some influence on the transients.

Sympathetic inrush current with its equations is illustrated in part I. part II is shown impact of load power factor on the sympathetic inrush current, and in part III, reason for changes of sympathetic inrush current’s magnitude because of  the different load power factors is discussed.

# Sympathetic inrush current

As it is mentioned before, sympathetic inrush current is caused by energizing inrush current of another transformer which is having energized after. In fig.1 schematic of a typical grid is shown. It has two transformers which are connected to a grid. One of the transformers is under operating condition. To investigate the sympathetic inrush current, first inrush current for parallel transformer is needed, so transformer which hadn’t been energized before would be energized. Therefore, Inrush current phenomenon would be happened which is shown in fig. 2. When transformer core is saturated, Inrush current is occurred. Saturation of transformer’s core is due to the fact that flux must flow continuously for every instant of time. Switching time and residual flux of transformer’s core are most important parameter to inrush current phenomenon. In fig.4 flux of transformer is shown. Residual flux is neglected in the picture.

The inrush energizing current caused voltage drop in equivalent system resistance. So parallel transformer would be affected by this voltage drop and it can obtain from the rule that flux is the integral of magnetizing voltage which will be describe by equations after, can put working transformer to the saturation condition. Inrush current and the transformer fxul are shown in fig.2 and fig.4. In fig. 5 sympathetic inrush current due to energizing inrush current for other transformer is shown [14].

Schematic of a typical grid with two transformer (one of transformers is operating and another is not even energize yet.)

Inrush current due to energizing transformer (Up to down: phase a, b and c)

Schematic diagram of sample grid

Magnetizing Flux of the transformer (up to down: phase a, b and c)

Sympathetic inrush current of the parallel transformer (Up to down:a, b and c)

Flux equation for one transformer is obtained as below [14]:

- Vb=ri+e

- Vb=ri+dλdt

- i=Vbr*(1-e-tτ)

- e=Vb-Vb(1-e-tτ)

It is known from System that Vb=vmsinωt

So

- i=vmR2+X2sinωt-φ+vmR2+X2sinφ*e-tτ

- e=vmsinωt-r(vmR2+X2sinωt-φ+vmR2+X2sinφ*e-tτ)

- λ=vmωsinωt-rvmωR2+X2sinωt-φ+vmωR2+X2sinφ*e-tτ+λ0

Due to fig.3 Vb, 𝜆, i, vmand ω are bus voltage of transformer, number of winding turns(N) multiply with core flux, magnetizing current, system voltage amplitude and angle speed. Inrush current passes through one side of the transformer, so the transformer can be modeled by a resistant (r=amount of winding resistant in Ω) that is series with an inductive element (L=LmΗ). So 𝛕 and X are obtained as below:

𝛕=Lmr  ,   X=ω Lm and  φ=tan-1XR

Sympathetic inrush current equations are as follow:

- Vb=Vs-I(Rs+jXs)

- I=ITr1+ITr2

- VmagT2=-ZPITr2-Rs+jXs-IVs

- Φ=VmagT2

Which Vs, ITr1 and ITr2 are system voltage, current due to two transformers and Z is stand for transformer impedance.

Equations are shown that inrush current causes voltage drop and the voltage drop makes changes in VmagT2,. Consequently, transformer flux which is integral of  VmagT2 increases and puts transformer in saturation condition.

Fig.6 is shown the inrush current and sympathetic inrush current for one phase of two parallel transformers simultaneously. From fig.6, it is clear that inrush current and sympathetic inrush current have opposite nature.

Sympathetic inrush and inrush current for two parallel transformers for  similar phase

# Effect of load power factor on sympathetic inrush current

In order to investigate load power factor effect on sympathetic inrush current, two similar sample grid is considered. The schematics is shown in fig.7. Two different load is used for each energized transformer (LOAD A and LOAD B) to compare sympathetic inrush due to them and see how different type of loads have effect on sympathetic inrush current.

Generally, three load types exist: inductive, resistive and capacitive. Combination of them is more common. Although resistive-inductive or even capacitive-resistive load are more common in the power systems than other types. In this research, each load type without any combination is considered to find out the impact of power factor on sympathetic inrush. Inductive load is stand for industrial devices and the results is adopted to them. Capacitive load, which happens rarely for transformer, can happen when a capacitor bank is near to the transformer and the transformer is not feeding any load. It is caused by a fault for example.

Transformer which was not in operating condition have energized to make sympathetic inrush current. Fig. 8 depicts sympathetic inrush current for transformer with inductive load. It is obtained from fig.8, the inductive load has lower inrush current magnitude than the resistive load. Further, the same act has done for transformers with the resistive load and another without load. Fig.9 is shown that sympathetic inrush current’s magnitude for two similar transformers, which one of them is with the resistive load and another is without load is equal.

typical grid in order to investigate load power factor effect on sympathetic inrush current

Sympathetic inush current of two similar transformers due to the same inrush energizing current, one of them with inductive load (dash-dott) another with resistive load (line).

Sympathetic inrush current of two similar transformer due to the same inrush energizing current, one of the without load (dash-dott) another with resistive load (line).

The same simulation has done in DIgSILENT to investigate how capacitive load effects the sympathetic inrush current and their results is compared with two other types.

Sympathetic inrush current of the transformers with capacitive and resistive load can be compare in fig. 10. The results show that sympathetic inrush current is equal for both cases. The same simulation has done for transformers with capacitive and resistive load which sympathetic inrush current is shown in fig.11. Sympathetic inrush currents for these two cases is as same as the previous simulation. Sympathetic inrush current for transformers with inductive and capacitive loads is shown in fig.12. From fig.12, transformer with inductive load has lower sympathetic inrush current’s magnitude than the transformer with capacitive load.

Results show that when the load is inductive, sympathetic inrush current is lower than other type of loads, and sympathetic inrush current’s magnitude is equal for transformers that feeds capacitive and resistive loads or even it is no-load.

Sympathetic inrush current for two similar transformer due to the same inrush energizing current, one of the capacitive load (dash-dott) another with resistive load (line).

Sympathetic inrush current of two similar transformer due to the same inrush energizing current, one of the capacirive (dash-dott) another with no-load (line).

Sympathetic inrush current of two similar transformer due to the same inrush energizing current, one of the inductive (dash-dott) another with capacitive load (line).

# Investigate the reason

As mentioned before, sympathetic inrush current for transformers that feed inductive loads, has lower magnitude than the transformer that feeds capacitive and resistive loads or even no-load. Sympathetic inrush current’s magnitude is equal for transformers with capacitive, resistive and no-load loads. In this section, EMTP program was used. DC model of inrush current for typical grid in fig.1 with different loads was extracted which are shown from fig. 13 until fig.18.

DC model of typical grid which showed in fig.1

DC model of inrush current for typical grid

DC model of inrush current for typical grid with inductive load

DC model of inrush current for typical grid with resistive load

DC model of inrush current for typical grid with inductive load (after closing the CB )

DC model of inrush current for typical grid with resistive load

DC model is used to study the challenge, because when inrush current phenomenon is happened, DC current passes through the system. Inductive elements in DC model can be assumed as short circuit which it is shown in fig.17 but capacitive load and no-load act as open circuit, also resistive element acts as resistive element in DC model too.  The reason for sympathetic inrush current’s magnitude mitigation with inductive loads is because of the required DC magnetizing current of the energized transformer can be fed from the additional way caused by short circuited inductive load. So, the DC current magnitude which was fed by grid is reduced and therefore voltage drop is reduced. thus sympathetic inrush current’s magnitude is smaller than other types. Moreover, inductive load can improve the flux magnitude of transformer core by providing the required DC offset magnetizing current. Consequently, lower amount of required DC offset passes through the transformer core reduces the saturation and sympathetic inrush current’s magnitude. Resistive load cannot improve flux magnitude of the transformer’s core. Also, as it is obtained from fig.18, they haven’t any effect on the sympathetic inrush current’s magnitude. So all of the required DC offset magnetizing current passes through the transformer’s core. The more DC energizing current pass through the core, the worse saturation of core has happened. So higher sympathetic inrush current’s magnitude is obtained for this case. No-load and capacitive load have similar action to the resistive load.

# Conclusion

In this paper, the effect of load power factor of transformers on the sympathetic inrush current is investigated. It is obtained that sympathetic inrush current’s magnitude for transformers with different load types is unlike.

The sympathetic inrush current’s magnitude for transformers with resistive load is as same as the transformers that feed capacitive load or even no-load transformers, also they are different with inductive load transformers in magnitude. The results for inductive load transformer can be adopted with transformers in industrial areas. Importance of this investigation is useful to understand sympathetic inrush current phenomenon better and better and using its results in better configuration of transformers protection settings too. DigSILENT and EMTPworks program are used to validate study.

Finally, the reason of the sympathetic inrush current’s magnitude changes is determined. Simulation results are shown that flux magnitude of the transformer core is improved and DC energizing offset current of transformer core is reduced by inductive loads. The Smaller DC energizing current magnitude flow through the transformer core, the lower amount of the saturation is a result. Consequently, the sympathetic inrush current’s magnitude is mitigated. Also DC current magnitude which passes through the grid, is reduced because of the inductive load. Thus lower voltage drop causes lower sympathetic inrush current magnitude. However, in the transformer with resistive and capacitive loads or no-load transformers, all of the required DC magnetizing current passes through the transformer core and through the grid also, thus higher voltage drop is caused and worse saturation and higher sympathetic inrush current magnitude than transformer with inductive load transformers is created.

# References

| [1] | H. Bronzeado and R. Yacamini, "Phenomenon of sympathetic interaction between transformers caused by inrush transients," Science, Measurment an Technology, vol. 142, no. 4, pp. 323-329, 1995. |
| --- | --- |
| [2] | C. D. Hayward, "Prolonged inrush currents with parallel transformers affect differential relaying," Electrical Engineering, vol. 60, no. 4, pp. 225-232, 1941. |
| [3] | R. Yacamini and A. A. Naser, "Calculation of inrush current in single phase transformers," IEEE Proceeding-Electric Power Aplication, vol. 128, no. 6, pp. 327-334, 1981. |
| [4] | J. peng, "Assessment of Transformer Energisation Transients and Their Impacts on Power Systems," in Phd. Thesis, Manchester, 2013. |
| [5] | C. Paul, Y. Ling and A. Basak, "Investigation of magnetizing inrush current in a single-phase transformer," IEEE Transactions on Magnetics, vol. 24, no. 6, pp. 3217-3222, 1988. |
| [6] | G. Tsourakis, I. Milis and J. Prousalidis, "Transformer Sympatetic inrush: A case study," in *th Mediterranean Conference on Power Generation, Transmission, Distribution and Energy Conversion (MEDPOWER), Calgari, 2012. |
| [7] | J. Holbach and M. Sanders, "Underestanding Sympathetic Inrush Currents and Theeir Effect on Protective Relays," in WPRC, 2012. |
| [8] | C. E. Lin, L. C. Cheng, L. Huang and J. C. Yeh, "Investigation of magnetizing inrush current in transformers, Part I - numerical simulation," Power Delivery, vol. 8, no. 1, pp. 246-254, 1993. |
| [9] | G. Boaming, A. T. Almeida, Z. Qionglin and W. Xiangzheng, "An equivalent instantaneous inductance-based technique for discrimination between inrush current and internal faults in power transformers," Power Delivery, vol. 20, no. 4, pp. 2473-2482, 2005. |
| [10] | A. A. Adly, H. H. Hanafy and S. E. Abu-Shady, "Utilizing preisach models of hysteresis in the computation of three-phase transformer inrush currents," Electric System Research, vol. 65, pp. 233-238, 2003. |
| [11] | R. Yacamini and H. S. Bronzeado, "Transformer inrush calculation using a coupled electromagnetic model," in IEE-Sience, Measurment and Technlogy, 1994. |
| [12] | M. Elluech and M. Poloujadof, "A contribution to the modeling of three phase transformers using reluctances," Magntics, vol. 32, no. 2, pp. 335-343, 1996. |
| [13] | S. K. Smith, L. Ran and B. Leyman, "Analysis of transformer inrush transients in offshore electrical systems," IEE-Preceedings-Generation, Transmission and Distribution, vol. 146, pp. 89-95, 1999. |
| [14] | P. Hetric and M. Kovac, "Analytical Analysis of Transformer Interaction Caused by Inrush Current," Bratislava, 2014. |