A High Performance VLSI Implementation for Efficient Removal of Impulsive Noise Using a Novel Adaptive Median Filter

| Mohamad Momeny Department of Computer Engineering Yazd University Yazd, Iran Email: mohamad.momeny@stu.yazd.ac.ir | Gholamreza Zare Fatin Assistant professor Faculty of Engineering University of Mohaghegh Ardabili Ardabil, Iran Email: zare@uma.ac.ir | Mahdi Nooshyar Assistant professor Faculty of Engineering University of Mohaghegh Ardabili Ardabil, Iran Email: nooshyar@uma.ac.ir | Samaneh Abbasi Department of Computer Engineering Payame Noor University (PNU) Esfarayen, Iran Email: s.abbasy@gmail.com |
| --- | --- | --- | --- |

Abstract— Impulse noise is one of the most important factors in degrading the image quality. In this paper, an efficient VLSI implementation for detecting and removing of impulse noise is introduced. In the noise removal process image information, such as edges and texture, are unaffected. The proposed algorithm uses a weighted window with variable sizes and applies the median filtering on them. Simulation results, with various images and noise intensities, show that the proposed algorithm has better performance compared with state of the art methods and increases the PSNR value (of the reconstructed image) up to 3dBs.

Keywords-component; VLSI, Noise removal; Impulse noise detector; Image enhancement

# Introduction

Digital images as the output of different digital cameras and scanners are partly damaged with different kinds of noise. One of the common types of noise is impulsive noise. This kind of noise when affected a pixel, then makes a big difference between its value and nearby pixels. Impulse noise reduction from corrupted images plays an important role in various image processing functions such as image segmentation, object detection, edge detection, compression, etc. Usually a nonlinear filter such as median filtering is used for noise reduction.

With growing appeal of cellular automata and by combining with fuzzy theories, a new kind of impulse noise filter is introduced in [1]. The proposed technique in [1], exploits the fuzziness in neighborhood of the corrupted pixel to produce local state cellular automaton transition rules. In order to eliminate impulse noises from noisy images, the authors in [2] have used a hybrid method based on cellular automata (CA) and fuzzy logic called Fuzzy Cellular Automata (FCA) in two steps. In the first step and based on statistical information, noisy pixels are detected by CA; then using this information, the noisy pixels are changed by using FCA method. In [3] the proposed method for removing noisy pixels, contains two stages, which in the first one, Noisy pixels are detected based on an Adaptive Neuro-Fuzzy Inference System (ANFIS), then in the final stage, the new changes are applied on these pixels using the Fuzzy Wavelet Shrinkage (FWS). An efficient method for reduction of impulse noise is presented in [4] which include two stages. At the first stage impulse noise is detected using ANFIS, and in the second stage the impulse noise is estimated. Then, those corrupted noisy pixels are replaced with new values based on the results of the ANN.

The authors in [6] describe the application of cellular automata (CA) method to various image processing tasks such as denoising and feature detection. In [7] a new filter structure for the reduction of mixed noise in images is proposed. It is based on the evaluation of fuzzy similarities between pixels in a local processing window and is suitable for high-speed hardware implementation. The articles [8] was devoted to introduce a new filtering approach based on fuzzy-logic control concepts which removes impulsive noise and also smooth out Gaussian noise. The proposed filter at the same time preserves the edges and image details efficiently.

A new median-based filter which actually is a progressive switching median (PSM) filter was proposed in [11] to restore images corrupted by salt-pepper impulse noise. The algorithm is developed by the following two main points: 1) In the switching scheme, an impulse detection algorithm is used before filtering, thus only a proportion of all  pixels will be filtered out; 2) In the progressive method, both the impulse detection and the noise filtering procedures are progressively applied in several iterations.

In this paper, we have proposed a filter which can be easily implemented in VLSI form and also has low power consumption and a high running speed.  This filter improves the existing methods for removing the impulsive noise of the images and at the same time preserves the important information like the edge and structure of the image.

For real-time embedded applications, the VLSI implementation of switching median filter for impulse noise removal is necessary. The implementation methods are usually optimized to have low power consumption and small area. These are all intending to reduce the cost of realization in ASIC or FPGA forms. This is necessary task to reduce the final cost of the product used by end user.

The rest of the paper is organized as follows. In the section 2 of the paper, the proposed algorithm for detecting and removing of impulse noise is discussed. The implementation procedure for the proposed filter is explained in section 3. The simulation results of the realized filter are given in section 4 and finally some conclusions are drawn in section 5.

# Proposed Filter

In the proposed algorithm for noise detection and removal, the whole image is scanned and processed pixel by pixel. If the current pixel has the maximum (255) or minimum value (0) then it is considered as a noisy pixel. In order to correct the value of a noisy pixel, a two-dimensional window centered on that noisy pixel is created. The size of this window is varied according to the number of noise free pixels in neighborhood of the current pixel.

The proposed algorithm has been designed in a way that it uses just divide by 2 and its multiples. This is due to straight forward division by 2 in digital domain which is accomplished by shifting. In the synthesis of the written code all limitations of the real world hardware implementation has been considered. In order to reduce the hardware size which leads to low power consumption, the size of the window for noise reduction is limited to 3*3 and 5*5 entries. The proposed algorithm can be implemented with real world hardware which was shown by the synthesis results. Although, the proposed algorithm has been designed with great regard to low power consumption in practice, it is very effective in improving the noisy picture as well.

The resulted value after applying median filter and weighted mean filter on noise-free pixels within window is considered as the corrected value of the noisy pixel. Figure 1 shows the proposed algorithm assuming that P is the noisy image and Q is the enhanced image. The steps of the proposed algorithm are illustrated as follows.

ALGORITHM

Step 1: If 0 < Pij < 255 then

Pij is a noise-free pixel, Qij = Pij and go to Step 5;

Step 2: If Pij == 0 or Pij == 255 then

Pij is a corrupted pixel, then create a 2D 3×3 window centered at Pij, element of this window are Moor neighbors of the Pij;

Step 3: If the number of noise-free pixels is greater than or equal to the noisy pixels in the 3×3 selected window then

Ignore the pixels (in the 3×3 selected window) with maximum and minimum gray level values; and set the Qij as,

Qij =Median of remaining elements ​​in the 3×3 selected window;

Go to Step 5;

Step 4: If the number of noise-free pixels is less than the noisy pixels in the 3×3 selected window then

f≜number of noisefree pixels in the window(total number of pixels in the window)-1;

Count ≜ The number of remaining elements ​​in the 3×3 selected window;

Med1 ≜Median of remaining elements ​​in the 3×3 selected window;

Increase window size (change the 3×3 matrix to 5×5);

Two cases are possible here as given in the following Case i and ii;

Case i):

If Count > 0 then

Ignore the pixels (in the 5×5 selected window) with maximum and minimum gray level values;

Med2≜Median of remaining elements ​​in the 5×5 selected window;

Qij = (Med1 * f) + (Med2 * (1-f));

Case ii):

If Count == 0 then

Vmax≜ The number of maximum gray level values ​​in the 5×5 selected window;

Vmin≜ The number of minimum gray level values ​​in the 5×5 selected window;

Vtotal≜ The total number of pixels in the 5×5 selected window;

If Vmax ≥ 45Vtotalthen Qij=255;

If Vmin ≥ 45Vtotalthen Qij=0;

If Vmax<45Vtotaland Vmin<45Vtotalthen

Qij=128;

Step 5: Repeat the above process for all noisy pixels;

Three levels for Qij are selected because of hardware implementation restriction. Greater number of levels for Qij causes more complex hardware implementation procedure and consequently higher power consumption. The causation of 45 is selected here (in step four of the algorithm) through searching technique.

# Implementation Procedure

In this section, we explain the implementation details of various blocks and sub-blocks which have been used in the filter structure. This includes two major blocks, pixel and matrix analyzers which will be covered in the continue.

## Pixel analyzer

Figure 2 shows the block diagram of the pixel analyzer which gets every pixel in 5*5 matrix format which contains 8-bit numbers from 0-255.

In order to get the matrix output, element analyzer processes three types of data: 1) middle entry of the matrix, 2) 3*3 middle matrix, 3) 5*5 input matrix. The selector signal which determines the final output on the multiplexer is also generated by element analyzer.

The 3*3 mean calculator produce the mean value of the entries in the 3*3 matrix which are not equal to 0 or 255. In order to do this, the insertion sort task is accomplished which contains nine 8-bit registers with zero initial value. The numbers of entries with values which are neither 0 nor 255 are counted and then are put regularly in a vector called s_vector and the last index of array is increased. For odd number of counted elements the element itself and for even number, the average value of two means is transmitted to the output.

The 255s and 0s counters in element analyzer of figure 2 count the number of 255 and 0 entries of 5*5 matrix. If the output of counters is equal to 25 then there isn’t any entry with value other than 0 or 255. Based on the number of the entries, the proper value is given to the output which can be 0, 255 or 128. If counted number is less than 25 then enable signal of the mean calculator is activated and its output is the valid number.

Figure 3 shows the details of this operation. The 3x3 matrix is scanned as a nine segmented register file and in every step one of those registers is evaluated. If this register value equals to 0 or 255, then related counter value increases: otherwise it is sorted into a registers set and the counter value is increased. Finally, according to the number of the sorted registers which is either even or odd, their median will be calculated. The 5*5 mean calculator operation is similar to 3*3 mean calculator block but it has 25 registers to store data. The calculated mean of these blocks is going to output calculator which is obtained according to (1),

(1)

The Med1 and Med2 variables in (1) contain the mean values of previous two calculations and f is the number of entries of 3*3 matrix with value other than 0 and 255. Based on internal algorithm, the selector signal of the multiplexer decides about the selection of the proper output of the signals which is produced in previous sections.

## Matrixanalysis

Figure 4 depicts the block diagram of the matrix analyzer. .This section receives a large 512*512 matrix (i_matrix) as input and divides it into 5*5 matrixes and sends them to pixel analyzer. The generated output is stored in appropriate row and column of the output 512*512 matrix (o_matrix). The most challenging task here is to divide the big matrix into 5*5 matrixes and also to initialize them. This is because of border conditions which make it difficult to find a matrix with entries including 0 up to 255.

The initial value of -1 is set for pixels of 5*5 matrix which is chosen because of border conditions. If the numbers from 0 to 255 is less than 25 then according to border conditions other entries is set to -1. The realization of this section with vector data type isn’t possible. So, the integer data type with 9-bits is chosen and the “111111111” will be synthesized as -1.

The matrix analyzer includes two main processes which are pixel generator and output generator. The pixel generator scans the 512*512 matrix row by row and generates the pixels of 5*5 matrixes in independent loops according to border and central values. The same is done for the last two rows as well as the left and right two columns. Figure 5 shows the scan mechanism of input matrix to create the 5×5 pixels.

The generated output of 5*5 matrixes that was given to pixel analyzer are stored in output generator and the output matrix is produced.

Figure 6 shows the building blocks of row and column shifter of Figure 4. At the top of this figure the number of previous column is given to adder/comparator unit in order to add one more unit and to compare with the maximum number of columns (here is 512).If this obtained number is less than the defined maximum value, the number of new column is sent to the output; otherwise, the number of new column is set to 0 and the shifting unit of the row is activated. This section also consists of the blocks like adder, comparator and selector. This unit is designed in the same way as the previous one, but the difference between them is that if the number of rows exceeds512, then FIN signal is transmitted to show end of the action.

When scan of the 512*512 matrixes is finished, the output matrix will be generated and it will remain unchanged until the input matrix change.

## Synthesis of the proposed noise removal algorithm

Synopsys design compiler that is one of the best ASIC synthesis tools is used here. The written code of the algorithm is based on VHDL language. The synthesis of the design has been done in CMOS 180 nm process. It should be mentioned that the synthesis was performed in order to build the net list file in gate level, which was optimized to get the final ASIC with minimum possible area occupation. The results of the occupied area, power consumption and time analysis can be used in physical design of the ASIC.

# Simulation Results and Comparison

The proposed algorithm is compared here with several other methods for detecting and removing of impulse noise such as U. Sahin et al. [1], Fuzzy Cellular Automata (FCA) [2], Adaptive Neuro-Fuzzy System and Fuzzy Wavelet Shrinkage (ANFISFWS) [3], Neural Network Detector Adaptive Neuro-Fuzzy Inference System (NNANFIS)[4], Neural Network Detector Median Filter (NNDMF)[5]. The result of the proposed filter in restoring noisy Barbara image corrupted by 70% impulse noise is shown in Figure 7. Figure 8 shows the qualitative results of our method and others in the literatures for color Lena image with noise density of 80%.

Flow chart of the proposed filter.

Restoration performance is quantitatively evaluated by peak signal-to-noise ratio (PSNR) and mean structural similarity (MSSIM) [35] which are deﬁned as:

(2)

(3)

(4)

(5)

The dynamic range of the pixel intensities for 8-bit gray-level images lie between 0 and 255 and is shown by D. M and N are the sizes of the images; P and Q denote the original image and the filtered image, respectively; the image contents at the kth local window in the original and ﬁltered images are shown by Pk and Qk; B is the total number of local windows in the image; μpk and μqkare the mean intensity of Pk and Qk; σpk and σqk are the standard deviation of Pk and Qk;σpkqk is the covariance between Pk and Qk; two small constants C1=(k1D)2 and C2=(k2D)2are used to stabilize SSIM using the following parameter settings: K1=0.01 and K1=0.03 [11].

The performance of the proposed algorithm and the related works are compared in Tables 1 and 2with images that contain different impulse noise densities. The results of Tables 1 and 2 show the PSNR values of the enhanced images obtained from proposed algorithm are about 3dB higher than other methods in the Table.

Table 3 lists MSSIM values resulted from applying different ﬁlters to four images corrupted by 40% and 80% impulse noise. Likewise, the proposed ﬁlter out performs other ﬁlters in terms of the performance metrics mentioned before. The objective evaluation based on MSSIM measurement indicates that the proposed ﬁlter has the best restoration performance among all the ﬁlters used in comparison. According to these figures, the proposed ﬁlter produces higher MSSIM values in compare to other ﬁlters used at different values of noise ratios.

Block diagram of the pixel analyzer.

The block diagram for median calculation of a 3*3 matrix.

Block diagram of the matrix analyzer.

f

i

-

2

,

j

f

i

-

1

,

j

f

i

,

j

f

i

+

1

,

j

f

i

+

2

,

j

f

i

-

2

,

j

+

2

f

i

-

1

,

j

+

2

f

i

,

j

+

2

f

i

+

1

,

j

+

2

f

i

+

2

,

j

+

2

f

i

-

2

,

j

-

1

f

i

-

1

,

j

-

1

f

i

,

j

-

1

f

i

+

1

,

j

+

1

f

i

+

2

,

j

+

2

f

i

-

2

,

j

+

1

f

i

-

1

,

j

+

1

f

i

,

j

+

1

f

i

+

1

,

j

+

1

f

i

+

2

,

j

+

1

f

i

-

2

,

j

-

2

f

i

-

1

,

j

-

2

f

i

,

j

-

2

f

i

+

1

,

j

-

2

f

i

+

2

,

j

-

2

f

i

-

2

,

j

f

i

-

1

,

j

f

i

,

j

f

i

+

1

,

j

f

i

+

2

,

j

f

i

-

2

,

j

+

2

f

i

-

1

,

j

+

2

f

i

,

j

+

2

f

i

+

1

,

j

+

2

f

i

+

2

,

j

+

2

f

i

-

2

,

j

-

1

f

i

-

1

,

j

-

1

f

i

,

j

-

1

f

i

+

1

,

j

+

1

f

i

+

2

,

j

+

2

f

i

-

2

,

j

+

1

f

i

-

1

,

j

+

1

f

i

,

j

+

1

f

i

+

1

,

j

+

1

f

i

+

2

,

j

+

1

f

i

-

2

,

j

-

2

f

i

-

1

,

j

-

2

f

i

,

j

-

2

f

i

+

1

,

j

-

2

f

i

+

2

,

j

-

2

Column i

Row j

A typical pixel inside the input matrix.

MUX

Adder

/

Comparator

Row

+

1

>

511

Old Row

Row

+

1

FIN

New Row

MUX

Adder

/

Comparator

Column

+

1

>

511

Old

Column

Column

+

1

0

New

Column

0

1

0

1

(

a

)

(

b

)

The shifter block diagram for (a) row, and (b) column.

(a); (b); (c).

Results of the proposed filter in restoring noisy Barbara image corrupted by 70% impulse noise: (a) original image Barbara, (b) noisy image, and (c) Output of proposed filter.

| (a) Original color Lena image | (b) Noise image | (c) MDBUTMF | (d) ATSM | (e) Proposed |
| --- | --- | --- | --- | --- |
| Comparison of the enhanced images obtained from different filters for theLena color image with impulse noise density of 80%. |  |  |  |  |

- Restoration results in PSNR (dB) of the different filters operating on the (512×512) Lena test images corrupted impulse noise.

| Methods | Noise ratio |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 20% | 30% | 40% | 50% | 60% | 70% | 80% |  |
| NNDMF [5] | 32.3 | 32.0 | 31.5 | 30.0 | 28.9 | 27.7 | 26.1 | 23.6 |  |
| NNANFIS [4] | 34.9 | 31.5 | 29.1 | 27.5 | 25.4 | 23.2 | 20.9 | 18.0 |  |
| ANFISFWS [3] | 29.2 | 28.2 | 26.8 | 23.8 | 22.0 | 15.4 | 12.9 | 10.8 |  |
| FCA [2] | 35.0 | 33.5 | 32.7 | 31.2 | 29.2 | 27.8 | 26.5 | 23.7 |  |
| Sahin et al. [1] | 40.8 | 37.1 | 34.9 | 33.2 | 31.2 | 30.6 | 29.3 | 27.6 |  |
| Proposed | 43.0 | 39.3 | 36.8 | 34.9 | 33.3 | 31.9 | 30.3 | 28.1 |  |

- Restoration results in PSNR (dB) of the different filters operating on the (512×512) Baboon test images corrupted impulse noise.

| Methods | Noise ratio |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10% | 20% | 30% | 40% | 50% | 60% | 70% | 80% |
| NNDMF [5] | 22.7 | 22.6 | 22.5 | 22.3 | 22.0 | 21.6 | 21.2 | 20.5 |
| NNANFIS [4] | 31.1 | 28.0 | 26.1 | 24.5 | 23.0 | 21.5 | 19.6 | 17.2 |
| ANFISFWS [3] | 28.7 | 23.8 | 23.0 | 22.1 | 19.9 | 18.1 | 15.4 | 13.9 |
| FCA [2] | 31.2 | 28.5 | 27.2 | 24.6 | 23.3 | 22.3 | 21.5 | 21.0 |
| Sahin et al. [1] | 35.9 | 32.6 | 30.6 | 28.3 | 26.7 | 25.4 | 24.2 | 23.3 |
| Proposed | 38.0 | 34.4 | 31.9 | 30.0 | 28.3 | 26.7 | 25.3 | 23.7 |

- Restoration results in MSSIM of different filters operating on the four test images corrupted by 40% and 80% impulse noise.

| Filters | Goldhill |  |  | Boat |  |  | Barbara |  |  | Bridge |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40% | 80% |  | 40% | 80% |  | 40% | 80% |  | 40% | 80% |
| GLAM [6] | 0.9210 | 0.7294 |  | 0.9346 | 0.7756 |  | 0.9005 | 0.6937 |  | 0.8782 | 0.6056 |
| BDND ﬁlter [7] | 0.9211 | 0.6977 |  | 0.9363 | 0.7457 |  | 0.9021 | 0.7052 |  | 0.8628 | 0.5565 |
| OCS ﬁlter [8] | 0.8013 | 0.6959 |  | 0.8425 | 0.7399 |  | 0.7299 | 0.6558 |  | 0.6649 | 0.5381 |
| FSM ﬁlter [9] | 0.8838 | 0.6837 |  | 0.9032 | 0.7373 |  | 0.8612 | 0.6322 |  | 0.8217 | 0.5583 |
| EEP ﬁlter [10] | 0.9243 | 0.7194 |  | 0.9376 | 0.7655 |  | 0.9019 | 0.7009 |  | 0.8792 | 0.5959 |
| Proposed | 0.9720 | 0.8589 |  | 0.9699 | 0.8593 |  | 0.9342 | 0.7935 |  | 0.9757 | 0.8121 |

##### References

U. Sahin, S. Uguz and F. Sahin, “Salt and pepper noise filtering with fuzzy-cellular automata”, Computers and Electrical Engineering, 40 (2014) 59–69

S. Sadeghi, “An efficient method for impulse noise reduction from images using fuzzy cellular automata,” Int. J. Electron. Commun. , 66, 772– 779 (2012)

Tavassoli S, Rezvanian A and Ebadzadeh MM “A new method for impulse noise reduction from digital images based on adaptive neuro-fuzzy system and fuzzy wavelet shrinkage,” Proc. Int. Conf. on Computer Engineering and Technology (ICCET), 4, 297-301 (2010)

Rezvanian A, Faez K and Mahmoudi F,”A two-pass method to impulse noise reduction from digital images based on neural networks,” Proc. Int. Conf. on Computer Engineering, 400–5 (2008)

Turkmen I, “Efficient impulse noise detection method with ANFIS for accurate image restoration,” Int. J. Electron Commun. (AEU), 65, 132–9 (2010)

S.Q. Yuan, Y.H. Tan, “Impulse noise removal by a global–local noise detector and adaptive median filter”, Signal Processing, 86(8), 2123–2128 (2006)

P.-E. Ng, K.-K. Ma, “A switching median filter with boundary discriminative noise detection for extremely corrupted image”, IEEE Transactions on Image Processing, 15(6), 1506–1516 (2006)

Z.F. Deng, Z.P. Yin, Y.L. Xiong, “High probability impulse noise- removing algorithm based on mathematical morphology”, IEEE Signal Processing Letters, 14(1), 31–34 (2007)

K.S. Srinivasan, D. Ebenezer, “A new fast and efficient decision-based algorithm for removal of high-density impulse noises”, IEEE Signal Processing Letters, 14(3), 189–192 (2007)

P.Y. Chen, C.Y. Lien, “An efficient edge-preserving algorithm for removal of salt-and-pepper noise”, IEEE Signal Processing Letters, 15, 833–836 (2008)

Z. Wang, A.C. Bovik, H.R. Sheik, E.P. Simoncelli, "Image quality assessment: from error visibility to structural similarity", IEEE Transactions on Image Processing, 13(4), 1–14 (2004)

S. Esakkirajan, T. Veerakumar, Adabala N. Subramanyam, and C. H. PremChand, “Removal of High Density Salt and Pepper Noise Through Modified Decision Based Unsymmetric Trimmed Median Filter,” IEEE Signal Process. Lett, 18 (5), 287-290 (2011)

S. P and E. J. P S, "Fixed-Value Impulse Noise Suppression for Images using PDE based Adaptive Two-Stage Median Filter", Proc. Int. Conf. on Computer, Communication and Electrical Technology – ICCCET, 290-295 (2011)