Securing Key Exchange and Key Agreement Security Schemes for RFID Passive Tags

Behzad Abdolmaleki1, Karim Baghery1, Bahareh Akhbari2, Seyed Mohammad Alavi3, and Mohammad Reza Aref 4

1Information Systems and Security Lab (ISSL), Sharif University of Technology, Tehran, Iran.

2EE Dept., K. N. Toosi University of Technology, Tehran, Iran.

3Imam Hossein Comprehensive University (IHCU), Tehran, Iran.

4ISSL Lab, EE Dept., Sharif University of Technology, Tehran, Iran.

Email: b.abdolmaleki.ir@ieee.org, k.baghery.1988@ieee.org, akhbari@eetd.kntu.ac.ir, malavi@ihu.ac.ir, aref@sharif.edu

Abstract—In the last decade, various types of RFID security schemes have been proposed which have tried to provide all security and privacy requirements of end-users. In this paper, we cryptanalyze two new-found key exchange and key agreement security schemes which are proposed by Zhang et al. and Kim in 2012 and 2013 respectively. We present the reveal secret parameter, tag impersonation and traceability attacks on Zhang et al.’s protocol with success probability “1”, and also we show that an attacker can impersonate the tag and the reader of Kim’s protocol both with success probabilities “ 14 ”. We also present some modifications on the structure of studied schemes and propose two modified schemes which overcome all the discovered flaws. Finally, the improved protocols are compared with some similar protocols in terms of security and privacy.

Index Terms—RFID, Key Exchange Protocol, Key Agreement Protocol, Security, privacy, EPC C1 G2 standard.

- Introduction

Radio Frequency Identification (RFID) systems are beneficial and user friendly systems that have been utilized in most identification and authentication applications such as personal gadgets, E-passport and etc. [1]- [2]. Moreover, RFID systems play a critical role in the Internet of Thing (IoT) or Internet of Device (IoD) systems that are the next generation of internet. In IoT and IoD systems, all objects that are in our environment will be connected to each other and will share information with each other, in which a RFID tag or any sensing devices can make these connections [3]. RFID systems use radio wave to exchange messages between the “tag”, “reader” and “back-end server (database)” that basically constitute an RFID system. The tag is an electronic chip that is equipped with a microstrip antenna and can make a wireless connection with readers. The readers are located between the tags and the database and allows them to exchange messages. The third part of a RFID system model is the database that includes all secret information about all tags. In each run of a RFID system, in order to identify and authenticate a specific tag, the database performs some mathematical processing and after successful authentication allows it to access data.

Due to the usage of RFID systems in personal devices and applications, considering the security and the privacy of these systems have become one of the most challenging research areas. Until now, lots of literature have focused on security of RFID systems and have proposed lots of RFID authentication protocols [4]- [5]. All proposed protocols aimed to provide secure communication between RFID users and preserve privacy of users. Nevertheless, most of the proposed protocols have some weaknesses and need more challenges to optimize the security and the privacy of them.

As mentioned before, in the last decade, lots of RFID authentication protocols have been proposed [4], [6]-[8]. In 2007, Chien and Chen [6] proposed a mutual RFID authentication protocol (MRAP) that is under Electronic Product Code Class 1 Generation 2 (EPC C1 G2) standard which is the most popular and widespread standard for RFID systems. Their protocol uses Pseudo-Random Number Generator (PRNG) and Cyclic Redundancy Check (CRC) functions to update and protect exchanged data between the tag and the database. Recently, Zhang et al. [8] illustrated that the MRAP protocol has some drawbacks and is not resistant against several attacks including tag impersonation, Denial of Service (DoS), and traceability. Then, in order to increase the security and the privacy of MRAP protocol, Zhang et al. have proposed an improved version of MRAP protocol (MRAP+) and they claimed that MRAP+ protocol removes all weaknesses of the MRAP protocol [8]. Zhang et al. analyzed the security and the privacy of the MRAP+ protocol and claimed that the MRAP+ protocol is secure against various attacks. In this study, we evaluate the security and the privacy of MRAP+ protocol against different attacks. We show that still there are some weaknesses on MRAP+ protocol that makes it vulnerable against some attacks such as reveal secret parameters, traceability attack and two different tag impersonation attacks. It is worth to mention that by performing reveal secret parameter attack, some attacks such as tag impersonation, traceability, and forward traceability attacks can be performed on MRAP+ protocol with success probability of “1”. In addition, using other weaknesses of MRAP+ protocol we present some other practical attacks.

| In 2011, Cho et al. presented a secure RFID mutual authentication scheme which uses hash function to protect exchanged messages [9]. In 2013, Kim [10] analyzed the Database (Kold, Pold, Knew, Pnew, EPCs, DATA) Reader Tag Ki, Pi, EPCs |  |  |  |
| --- | --- | --- | --- |
| For each EPCs, Pold, Pnew, Kold, Knew Iold=D⊕Kold , Inew=D⊕Knew Matches M1 = ?PRNG(KX⊕NR⊕IX) If M1=PRNG(Knew⊕NR⊕Inew) => X=new Else if M1=PRNG(Kold⊕NR⊕Iold) => X=old End Then computes values below: If X=new: M2=PRNGEPCs⊕NT⊕Pnew Kold←Knew←PRNGKnew Pold←Pnew←PRNGPnew Else M2=PRNGEPCs⊕NT⊕Pold End |  | (1) NR→ | Generates random number NT M1=PRNGKi⊕NR⊕NT D=NT⊕Ki` |
|  |  | (2) ←M1, D |  |
|  | ←M1, D,NR, (3) |  |  |
|  | (4) M2, DATA→ | (5) M2→ | Verify: M2⊕Pi=? PRNGEPCs⊕NT Ki+1←PRNGKi, Pi+1←PRNGPi |

Fig. 1. The MRAP+ Protocol [8].

security of Cho et al.’s protocol and showed that their protocol still has some problems that make it vulnerable against DoS attack. Then he applied some changes on Cho et al.’s protocol and proposed a RFID mutual authentication protocol. He analyzed the security and the privacy of the developed protocol and claimed that his protocol is secure against various attacks [10]. In this study, we analyze the security of Kim’s protocol and we show that still there are some flaws on Kim’s protocol and an attacker can impersonate the tag and the reader both with success probabilities “ 14 ”. Then, in order to provide security and privacy of RFID users, we apply some revisions on MRAP+ and Kim’s protocols and present two strengthened versions of them that omit existing weaknesses. Moreover, our analysis show that both the improved protocols are secure and confidential against other attacks which are presented in other literature. In addition, the security and the privacy of proposed protocols are compared with some similar protocols.

This paper is organized as follows: In Section II, we introduce MRAP+ protocol and then we analyze its security and privacy. Kim’s protocol and its weaknesses are provided in Section III. In Section IV, the improved versions of MRAP+ and Kim’s protocols are presented that remove their flaws. Also in Section IV, we analyze the security and privacy of proposed protocols and then compare with some similar protocols. Finally, conclusions are reported in Section V.

- Vulnerabilities of the MRAP+

In [8] Zhang et al. claimed that the MRAP+ protocol provides security and privacy requirements of RFID users. Here, we show that still there are some flaws on MRAP+ protocol and it cannot provide secure and confidential communication for RFID users. First we introduce MRAP+ protocol and then we present our practical attacks on this protocol including reveal secret parameters and traceability attacks and also two different tag impersonation attacks.

## The MRAP+ Protocol

The MRAP+ protocol is a key agreement protocol which is under EPC C1 G2 standard and similar to most of RFID authentication protocols, it uses random numbers and a PRNG function to protect the transmitted messages. The structure of the protocol is shown in Fig. 1. According to the Fig. 1, one successful run of this protocol consists of five phases. The used notations are shown in Table I.

- The Notations

| Not. | Description |
| --- | --- |
| EPCi | Electronic Product Code (EPC) of the ith tag |
| ID | Unique identification information of the specific tag |
| Ki | The authentication key shared by the server and tag |
| Pi, Sj | The authentication key shared by the server and tag |
| NR, Rr | A random number generated by the reader |
| NT, Rt | A random number generated by the tag |
| Data | The corresponding record for the tag stored in the server |
| PRNG | Pseudo random number generator |
| .i | For ith run of protocol |
| ∥ | Concatenation operation |
| h(.) | Hash function |

## Reveal Secret Parameters

In RFID authentication protocols, it is very important that secret values be safe against the reveal attack. If an attacker obtains the secret values, he/she will be able to perform different attacks that threats the security and privacy of user. This subsection aims to show that in the MRAP+ protocol, the secret values are not protected completely and an attacker can reveal secret parameters. One of the main weaknesses of MRAP+ protocol is the structure of M1 and D that are dependent and M1=PRNG(D⊕NT). Due to this weakness, the attacker can perform some processes and discover Ki. These processes can be addressed in two phases as follows,

Learning phase: In ith round, the attacker eavesdrops one successful run of the protocol and saves the exchanged data between target the tag and the reader including NR, M1=PRNG(Ki⊕NR⊕NT), D=NT⊕Ki.

Processing phase: Using the obtained data in learning phase, the attacker can perform following operations. If Ki be l bit string, as a result Ki ϵ U, where U=u1,u2,…,u2l. Now

For 1≤i≤2l

Choose ui∈U, and set NT=ui⊕D

if  M1=PRNG(ui⊕NR⊕NT) then

return ui as Ki

End                                                                          (1)

It can be seen that using provided algorithm in (1), the attacker can obtain Ki after maximum 2l runs. Since in the EPC C1 G2 standard secret parameters are usually 16 bit string, the attacker needs maximum 216 runs. Another weaknesses of MRAP+ protocol is the way of updating Ki that is Ki+1=PRNG(Ki). Using this fact, after obtaining Ki, the attacker can compute Ki+j with performing j times PRNG of Ki.

Note that, after applying this attack, the attacker can perform different attacks including tag impersonation, traceability and forward traceability attacks with success probability of “1”. Moreover, without using the result of this attack, we report another weaknesses of MRAP+ protocol in the rest of this paper.

## Replay Attack and the First Tag Impersonation Attack

This subsection shows that how an attacker can impersonate a legitimate tag in the MRAP+ protocol. Due to similarity of M1 and D that is one of the main weaknesses in MRAP+ protocol, an attacker can impersonate a legitimate tag as follows,

Learning phase: In the ith round, the attacker eavesdrops one successful run of the protocol and obtains M1i and Di.

Attack phase: In the (i+1)th round, the attacker starts a new session with the reader and acts as follows,

- The spoofed tag or the attacker receives a random number NRi+1 from the reader and responses with M1,atti+1,  Datti+1 that are generated as follows,

M1,atti+1=PRNG(Di⊕NRi+1),   Datti+1=Di.          (2)

- According to the structure of the protocol, the reader sends M1,atti+1, Datti+1, NRi+1 to the back-end server.

- In order to authenticate, the back-end server acts as follows,

- For  Ki=Kold or Knew, it computes Ii=Datti+1⨁Ki for i=old or new. Note that since we use secret values of ith run, the back-end server selects Ki=Kold in its authentication process.

- It checks whether PRNGKi⨁Ii⨁NRi+1=?  M1,atti+1, where

PRNGKi⨁Ii⨁NRi+1=PRNGKi⨁Datti+1⨁Ki⨁NRi+1

=PRNGKi⨁Di⨁Ki⨁NRi+1

=PRNGDi⨁NRi+1=M1,atti+1.         (3)

As a result the back-end server authenticates the spoofed tag as a legitimate tag.

## The Second Tag Impersonation Attack

In addition to the mentioned weaknesses in the last subsection, there is another weakness in the authentication phase of MRAP+ protocol that makes it vulnerable to another tag impersonation attack as follows:

Similar to the above tag impersonation attack, in the learning phase the attacker obtains M1i and Di.

In the attack phase, it generates M1,atti+1,  Datti+1 as follows,

M1,atti+1=M1i,    Datti+1=Di⨁NRi+1⨁NRi.                 (4)

Also, similar to the firstly mentioned tag impersonation attack, in order to authenticate, the back-end server computes Ii=Datti+1⨁Ki for i=old or new.

Then the back-end server checks whether PRNGKi⨁Ii⨁NRi+1=?  M1,atti+1, where

PRNGKi⨁Ii⨁NRi+1=PRNGKold⨁Datti+1⨁Kold⨁NRi+1

=PRNGKold⨁Di⨁NRi+1⨁NRi⨁Kold⨁NRi+1

=PRNGDi⨁NRi+1⨁NRi⨁NRi+1

=PRNGNTi⊕Ki⨁NRi=M1i =M1,atti+1.                     (5)

Therefore, the back-end server authenticates the attacker as a legitimate tag.

## Traceability Attack

Privacy concern is one of the most important issues in RFID authentication protocols. Here we show that MRAP+ protocol is not safe against traceability attack and an attacker can trace a specific tag. This problem arises from similarity of tag responses including M1 and D. This attack can be summarized as follows,

Learning phase: In the ith round, he/she eavesdrops exchanged data between the reader and the specific tag including NRi, M1i=PRNG(Ki⊕NRi⊕NTi) and Di=NTi⊕Ki. Then, he/she terminates the fifth phase of protocol and prevents the updating operation by the tag.

Attack phase: The attacker starts a new session with a new tag and uses the eavesdropped messages and sends NRi to the target tag and receives following responses,

M1i+1=PRNG(Ki+1⊕NRi⊕NTi+1), Di+1=NTi+1⊕Ki+1.    (6)

Now using the received messages, firstly the attacker computes ζ=Di⊕Di+1=NTi⊕Ki⊕NTi+1⊕Ki+1. Then he/she verifies that PRNG(ζ⊕Di⊕NRi)=?  M1i+1. The tag that satisfy this equation, is the target tag.

Proof: It is worth to mention that if the selected tag be the target tag Ki=Ki+1, as a result ζ=NTi⊕NTi+1. By considering this fact, following equations can be written,

PRNGζ⊕Di⊕NRi=PRNG(NTi⊕NTi+1⊕Di⊕NRi)

=PRNG(NTi⊕NTi+1⊕NTi⊕Ki⊕NRi)

=PRNG(NTi+1⊕Ki⊕NRi)

=PRNG(NTi+1⊕Ki+1⊕NRi) =M1i+1                  (7)

- Vulnerabilities of Kim’s Protocol

In 2013, Kim [10] proposed a RFID mutual authentication protocol that is an improved version of Cho et al.’s protocol [9]. He claimed that his protocol has high resistance against various attacks that makes it suitable for RFID systems. However, we show that still there is a flaw in Kim’s protocol that in some cases makes it vulnerable against some attacks. In the rest of this Section, first we introduce Kim’s protocol and then we present two impersonation attacks on this protocol.

## Kim’s Protocol

Kim’s protocol is a hash-based key exchange protocol which protects exchanged messages via hash functions strongly. The communication channel between the reader and the tag is protected but the channel between the reader and the tag can be eavesdropped. Kim’s protocol saves old and new secret values in the back-end server that makes it secure against most of the attacks. Fig. 3 shows the structure of Kim’s protocol.

## Tag Impersonation Attack

In [10] Kim claimed that his improved protocol is secure against different attacks. Here now, we show that his scheme is not secure against the tag impersonation attack perfectly and an attacker can impersonate the tag with success probability "14". In Kim’s protocol, the value of RID is expressed as follows,

RID=Rt-RtmodSj+10:47∥Rt+Sj-RtmodSj48:95. (8)

It is clear that if Rt<Sj, (8)  can be simplified as follows,

RID=Rt-Rt+10:47∥Rt+Sj-Rt48:95

=10:47∥Sj48:95                                              (9)

It is worth to express that RtmodSj=Rt for Rt<Sj.

Now, we show that using this fact an attacker can impersonate a legitimate tag in two phases as follows,

Learning phase: In ith round, the attacker eavesdrops the transmitted messages in phases 1 and 2 (according to the Fig. 2) including Rri,  αi,Rti⊕βi, then blocks phase 3 of the protocol. Indeed the attacker prevents receiving messages by the tag transmitted form the reader. As a result the tag does not update its secret values.

Attack phase: In (i+1)th round, the attacker starts a new session with the reader and performs following steps,

- The attacker receives a random number Rri+1 from the reader and generates αatt,  γatt and sends them to the reader.

αatt=αi=hIDki⊕Rri⊕Rti⊕RIDi

γatt=Rti⊕βi⊕Rri⊕Rri+1.                      (10)

| Back-end Server ID,Sj,Sj-1 Reader Tag ID,Sj |  |  |  |
| --- | --- | --- | --- |
| For each tuple of ID,Sj,Sj-1 Generates β and obtains Rti and RIDi Verifies αi= ? hIDki⊕Rri⊕Rti⊕RIDi Calculates θ=DATA∥hβ⊕RIDi∥Rt⊕Sj+1∥hSj⊕Sj+1 and sends it to the tag and updates its secret values as follows: Sj-1←Sj←Sj+1 | Rri → (1) |  | Generates Rti Randomly RIDi=hRt-RtmodSj+10:47∥hRt+Sj-RtmodSj48:95 αi=hIDki⊕Rri⊕Rti⊕RIDi βi=hSj0:47∥IDki48:95 |
|  | ← (2) |  |  |
|  | ← Rri, αi, βi⊕Rti | ← αi, βi⊕Rti |  |
|  | The reader extracts DATA and sends following messages to the tag: hβ⊕RIDi∥Rt⊕Sj+1∥hSj⊕Sj+1 → (3) |  | (4): Calculates hβi∥RIDi, and extracts hβ⊕RIDi, then checks that: If hβ⊕RIDi==hβi∥RIDi the server is legitimate and the tag extracts Sj+1and it updates: Sj←Sj+1 |

Fig. 2. Kim’s Protocol [10].

The reader follows the structure of the protocol and sends αatt, γatt, Rri+1 to the back-end server.

- According to the structure of the protocol, the back-end server follows following stages,

- For each couple IDk and Sj it generates βi and extracts βi of target tag

- Then the back-end server uses received γatt and obtains Rti+1 as follows.

Rti+1=γatt⊕βi

=Rti⊕βi⊕Rri⊕Rri+1⊕βi=Rti⊕Rri⊕Rri+1.

- Now, using obtained Rti+1, the back-end server extracts RIDi via (8). As we mentioned above, if Rti<Sj and Rti+1<Sj, the value RIDi+1 can be rewritten as RIDi+1=10:47∥Sj48:95=RIDi .

- In this stage, the server uses the values IDki, Rri+1, Rti+1 and RIDi+1 and checks that αatt= ? hIDki⊕Rri+1⊕Rti+1⊕RIDi+1. According to stage 3, by assuming that Rti<Sj and Rti+1<Sj , we have observed that RIDi+1=RIDi. As a result, the back-end server authenticates the spoofed tag as a legitimate tag.

Proof:     hIDki⊕Rri+1⊕Rti+1⊕RIDi+1=h(IDki⊕Rri+1

- ⊕Rti⊕Rri⊕Rri+1⊕RIDi+1).

=hIDki⊕Rti⊕Rri⊕RIDi=αatt.        11

Lemma 1: For two numbers λ and μ that are random numbers from set χ=0,1,…,2n-1 and μ>1, the probability of inequality λ<μ is greater than 12 [11].

Lemma 2: For three numbers λ, μ and κ that are random numbers from set χ=0,1,…,2n-1 and μ>1, the probability of inequality λ⊕κ<μ is greater than 12 [11].

Using Lemma 1 and Lemma 2, it can be shown that inequalities Rti<Sj and Rti+1<Sj hold with probability greater than "12". Thus, the success probability of this attack is greater than "14" .

## Reader Impersonation Attack on Kim’s Protocol

Besides the tag impersonation attack that is presented in the last subsection, we show that an attacker can use the available flaws in the structure of RID and impersonate a legitimate reader. This attack is also based on assumptions Rti<Sj and Rti+1<Sj that are considered in (9), and the success probability of this attack is equal to "14". This attack can be performed in two phases as follows,

Learning phase: In ith round, the attacker eavesdrops exchanged messages between the tag and the reader including Rti⊕βi,hβi⊕RIDi∥Rti⊕Sj+1∥hSj⊕Sj+1, then blocks phase 3 of the protocol.

Attack phase: In (i+1)th round, the attacker starts a new session with the tag and operates as follows,

- The attacker sends a random number Ratti+1 to the tag.

- The tag generates Rti+1 and calculates messages, Rti+1⊕βi+1 and αi+1=hIDk⊕Ratti+1⊕Rti+1⊕RIDi+1, and sends them to the spoofed reader. Note that, since in the learning phase the attacker blocked phase 3 of the protocol, the tag does not update its secret value and consequently based on these assumptions, βi+1=βi and RIDi+1=RIDi.

- Using eavesdropped messages Rti⊕βi,Rti⊕Sj+1 and received message Rti+1⊕βi+1 from the tag, the spoofed reader or attacker calculates Rti+1⊕Sj+1 as follows,

- Rti+1⊕Sj+1=Rti⊕βi⊕Rti+1⊕βi+1⊕Rti⊕Sj+1.

and sends message δatt=hβi⊕RIDi∥Rti+1⊕Sj+1∥hSj⊕Sj+1 to the tag.

- Finally, the attacker follows the structure of protocol and authenticates the attacker as a legitimate reader.

According to the Lemma 1 and Lemma 2 it can be shown that success probability of this attack is greater than "14".

- Modified Protocols and Analysis

In Sections II and III we have shown that the security and the privacy of MRAP+ and Kim’s protocols have some problems that allow the attackers to perform different attacks. In this Section, two strengthened versions of MRAP+ and Kim’s protocols are proposed that omit all the mentioned weaknesses. Also, the security and privacy analysis of proposed protocols are provided.

## Improved Version of MRAP+ Protocol

There are two major problems in the structure of MRAP+ protocol. The first problem is the dependency between two responses (M1 and D) of the specific tag as M1=PRNG(D⊕NT), and the second one is the authentication procedure in the back-end server that is vulnerable. Thereby, in order to eliminate its weaknesses, we propose an improved version of MRAP+ protocol. In the improved version, we apply some changes in the authentication, updating procedure and tag’s responses. The authentication procedure of the improved protocol is as follows,

- Initialization phase

The stored values in the tag (K0 and P0) are initialized by random numbers that are generated in the factory. Note that in the initial phase, Kold=Knew=K0 and Pold=Pnew=P0.

- Authentication phase

- This phase consists of five steps as follows,

- Reader → Tag: The reader generates NR as a random number and sends it to the tag.

- Tag → Reader: The tag responses to the reader with the following messages,

- M1=PRNG(EPCs⊕NR)⊕PRNG(Pi⊕NT), D=NT⊕Ki

where NT is a random number that is generated by the tag.

- Reader → Back-end server: The reader sends the messages M1, D and NR to the back-end server.

- Back-end server→ Reader: Upon receiving the transmitted messages by the reader, the back-end server performs following steps,

- 4-1: For each tuple EPCs, Pold, Pnew, Kold, Knew computes Iold=D⊕Kold, Inew=D⊕Knew and matches M1 = ? PRNG(EPCs⊕NR)⊕PRNG(PX⊕IX). Then, the value that matches this equation will be set as X=old or new.

- 4-2: If X=old, the back-end server calculates M2=PRNG(EPCs⊕Iold)⊕Pold and sends M2 and DATA to the reader.

- Else, X=new and the back-end server calculates M2=PRNG(EPCs⊕Inew)⊕Pnew and sends M2 and DATA to the reader. After that, the back-end server updates its secret values as follows

- Kold←Knew←PRNG(Knew⊕NT⊕Pnew),

- Pold←Pnew←PRNG(Pnew).                         (12)

- Reader → Tag: Upon receiving the transmitted messages by the back-end server, the reader sends M2 to the tag. Now, the tag verifies M2⊕Pi = ?PRNG(EPCs⊕NT), and updates its secret values as follows,

- Ki+1←PRNG(Ki⊕NT⊕Pi), Pi+1←PRNG(Pi).

In the rest of this subsection, we analyze security and privacy of improved MRAP+ protocol and we show that it is secure against various attacks and also provides user privacy.

- Reveal Secret Parameter: In Section II.B it is shown that due to similarity of the structures of M1 and D, an adversary can use an algorithm and obtain the secret parameter Ki and consequently can calculate NT. Indeed, after this attack an attacker can perform most of attacks against the broken protocol. In the improved protocol, in order to eliminate this weakness, we apply some changes on the structure of tag’s response M1. Also, to increase the security and privacy of the proposed protocol, we add a secret value Pi to M1 that makes it strong against different attacks.

- Impersonation Attacks: In the improved protocol, in order to perform impersonation attacks and calculate M1, D and M2 the adversary needs EPCs, NT, Pi and Ki that are unknown for him/her. On the other side, since the dependency between M1, D and two consecutive M1 are omitted, the adversary cannot use exchanged messages to impersonate the tag or reader.

- Privacy Concerns: As it is shown in Section II.E, although the MRAP+ protocol uses two random numbers NR and NT in the tag’s responses, it is vulnerable against traceability attack. In order to remove this weakness, we use a random number  NT in the updating procedure and some changes in tag’s responses. It can be seen that if an adversary catches the secret values Ki and Pi, it cannot trace a specific tag.

Moreover, more security analysis of MRAP+ is reported in [8] that are also true about our improved protocol.

## Improved Version of Kim’s Protocol

In the security analysis of Kim’s protocol, we have shown that there are some weaknesses on his protocol that make it vulnerable to tag and reader impersonation attacks. In Kim’s protocol the main problems are the structures of RID and β. In this subsection, we aim to remove aforementioned weaknesses in Kim’s protocol. To this aim, we change the structures of RID and β in his protocol. We use a one-way function (i.e. hash function) in the structure of RID that increases its security. More precisely, in the improved protocol, we change the structures of RID=Rt-RtmodSj+10:47∥Rt+Sj-RtmodSj48:95 to RIDnew=hRt⊕Sj and β=Sj0:47∥IDki48:95 to βnew=IDki48:95∥Sj0:47.

With these changes, since the values of Rt and Sj change in each run of the protocol, and the attacker does not access to them directly, the attacker cannot impersonate the tag and the reader. Also, since the structures of the improved protocol and Kim’s protocol are similar, according to the provided analysis in [10], the improved protocol is secure against other attacks.

Note that, all phases of the improved protocol is the same as Kim’s protocol, and uses the new values of RIDnew and βnew.

In Table II, the security and privacy analysis of MRAP+ protocol, Kim’s protocol and their improved versions are provided. According to security analysis, it can be seen that all weaknesses of the analyzed protocols are omitted in the improved versions that make them secure against different threats.

- Comparison of Security and Privacy Analysis

| Protocols Attacks | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Reveal Secret Parameters |  | × |  |  |  |  |
| Tag Impersonation | × | × |  | × | × |  |
| Reader Impersonation |  |  |  | × | × |  |
| Dos Attack | × |  |  | × |  |  |
| Traceability attack | × | × |  | × |  |  |

: Secure    ×: Insecure   A. MRAP [6]     B. MRAP+ [8]       C. Improved MRAP+              
                                                            D. Cho et al.  [9]     E. Kim [10]       F. Improved Kim

- Conclusion

In this study, we have analyzed two new-found key exchange and key agreement RFID mutual authentication protocols in terms of security and privacy that have been proposed by Zhang et al. [8] and Kim [10] in 2012 and 2013 respectively. We have shown that both the studied protocols are unable to provide security and privacy requirments of RFID end-users. We have presented tag impersonation and traceability attacks on Zhang et al.’s protocol with success probability “1”, and tag impersonation and reader impersonation attacks on Kim’s protocol both with success probabilities “ 14 ”. Then, we have applied some changes on the analyzed protocols and proposed two strengthened versions of them. Our security analysis has shown that the proposed protocols omit all weaknesses of Zhang et al. and Kim’s protocols and also they are resistant against various attacks. Finally, the security analysis of the proposed protocols are compared with some similar protocols that are proposed recently.

References

| [1] | S. Vaudenay, "E-passport threats," IEEE Security & Privacy, vol. 5, no. 6, pp. 61-64, 2007. |
| --- | --- |
| [2] | D. He, and Sh. Zeadally, "An analysis of RFID authentication schemes for internet of things in healthcare environment using elliptic curve cryptography," IEEE Internet of Things Journal, vol. 2, no. 1, pp. 72-83, 2015. |
| [3] | B. Hameed, I. Khan, F. Durr, and K. Rothermel, "An RFID based consistency management framework for production monitoring in a smart real-time factory," in 2nd Int. Conf. on the Internet of Things (IOT), Tokyo, 2010. |
| [4] | E.-J. Yoon, "Improvement of the securing RFID systems conforming to EPC class 1 generation 2 standard," Expert Syst. Appl., vol. 39, no. 11, p. 1589–1594, 2012. |
| [5] | K. Baghery, B. Abdolmaleki, B. Akhbari and M. R. Aref, "Untraceable RFID authentication protocols for EPC compliant tags," in 23rd Iranian Conf. on Elec. Eng. (ICEE), Tehran, 2015. |
| [6] | H. Y. Chien, and C. H. Chen, "Mutual authentication protocol for RFID confirming to EPC class 1 generation 2 standards," Comp. Standards & Interfaces, vol. 29, no. 2, pp. 254-259, 2007. |
| [7] | B. Abdolmaleki, K. Baghery, B. Akhbari, and M. R. Aref, "Cryptanalysis of two EPC-based RFID security schemes," in 12th Int. ISC Conf. on Info. Security and Cryptology, 2015. |
| [8] | J. Zhang, W. Wang, J. Ma, and X. Li, "A novel authentication protocol suitable to EPC class 1 generation 2 RFID system," Journal of Convergence Info. Technology, vol. 7, no. 3, 2012. |
| [9] | J.-S.Cho, S.-S. Yeo, and S. K. Kim, "Securing against brute-force attack: A hash-based RFID mutual authentication protocol using a secret value," Comp. Comm., vol. 34, pp. 391-397, 2011. |
| [10] | H. Kim, "RFID mutual authentication protocol based on synchronized secret," International Journal of Security and Its Applications, vol. 7, no. 4, pp. 37-49, 2013. |
| [11] | M. H. Dehkordi, and Y. Farzaneh, "Improvement of the hash-based RFID mutual authentication protocol," Wireless Personal Communications, vol. 75, no. 1, pp. 219-232, 2014. |