A Backstepping Approach for Blood Glucose Control of Parker System

Bahare Farahmand & Maryam Dehghani

School of Electrical & Computer Engineering,

Shiraz University, Shiraz, Iran.

(E-mail: baharfarahmand_68@yahoo.com, mdehghani@shirazu.ac.ir)

Abstract— This paper proposes a novel backstepping approach to design a controller for the glucose-insulin model of type I diabetes modeled by the 19th-order Parker system. For this purpose, the system is divided into four parts including two nonlinear dynamic subsystems, a nonlinear static subsystem and a linear one. Based on Lyapunov stability and ISS theorems, the virtual control inputs are designed for each part. The stabilizing virtual inputs are derived based on linear matrix inequality theory which is directly applied to the linear subsystem and indirectly applied to the nonlinear subsystem, based on the Lipschitz condition. Simulation results verify the merits of the proposed approach in handling the effect of external meal disturbance and the blood glucose concentration convergence to its reference value.

Keywords-19th-order Parker glucose-insulin model, Type 1 diabetes, backstepping approach, Disturbance rejection

# Introduction

Diabetes mellitus (DM) is a group of metabolic diseases created by environmental and genetic reason. It causes abnormal increase in blood glucose (BG) levels which is called hyperglycemia [1]. The standard blood glucose concentration level is in the range of [60, 120] mg/dL [2]. The blood glucose concentration is regulated by insulin and glucagon hormones that secreted from α and β cells of the islets of Langerhans in the pancreas, respectively. Insulin and glucagon cause decrease and increase of the levels of blood glucose, respectively [1]. Diabetes is classified into two major groups. Type I or insulin dependent diabetes mellitus (IDDM) is caused when the β cells cannot produce enough insulin. Type II or non-insulin dependent diabetes mellitus (NIDDM) is result from inability of cells to use insulin appropriately [3]. This paper concentrates on type I diabetes that needs external insulin infusion (subcutaneous or intravenous).

Many researchers have developed variety of mathematical models to represent type 1 diabetes behavior. The most popular of these models are Bergman minimal model [4], 19th-order Sorensen model [5] and Hovorka model [6]. All of them are nonlinear models with various degrees of complexity and applicability. Among these models, the Bergman minimal model due to its simplicity is the most widely used for control purposes. The Sorensen’s high order model (the current paper concentrates on a modification of it, developed by Parker [7]) is the most complex one that precisely describes the behavior of glucose and insulin in different tissues.

Several control methods have been presented to glucose-insulin control based on these models [8]. In [2], the advanced model-based controller was designed based on the Bergman minimal model. In addition, parametric programming algorithms were employed to provide the optimal insulin delivery rate as a function of the current blood glucose concentration of patients. In [3], the simplified adaptive switching control strategy was proposed by using the aforementioned model for type 1 diabetic therapy. Furthermore, in that paper, the authors showed that the decreasing the number of proportional-integral-derivative (PID) controllers causes the performance improvement in switching method. In [9], the fourth-order single-input-single-output state space model was developed according modification of the Bergman minimal model. Then an observer based state feedback controller was designed based on the presented model. In [10], PID and fuzzy logic controllers were suggested by using minimal model for continuous drug infusion. In [11], Takagi-Sugeno (TS) fuzzy model and fuzzy controller were obtained according to a simplified model of nonlinear Hovorka system. The Hovorka model was considered to design a nonlinear model predictive controller in [12].

In [7], the Parker’s model was linearized and reduced to a third-order linear dynamic model. Then, a robust H∞ controller was designed based on the reduced model. The nonlinear identification of LPV model was proposed for Sorenson model in [13]. In [14], a robust H∞ controller was designed for linearized Parker model and then it was tested on the reduced nonlinear model. However, the nonlinear Parker model has been rarely used for control problems due to its complexity and high orders. Also, in most works on this model, the model was linearized or its order was reduced.

In this paper, we propose a novel backstepping approach to provide a systematic approach for nonlinear control of blood glucose concentration for type 1 diabetes patients. The proposed approach is based on the backstepping technique and the nonlinear 19th-order Parker glucose-insulin model. To apply the backstepping approach, the system is divided into 4 partitions including two nonlinear dynamic subsystems, a nonlinear static subsystem and a linear one. The objective is to propose a nonlinear control law for insulin injection such that the plasma glucose is forced to converge to its appropriate reference.

The paper is organized as follows: In Section II, the nonlinear Parker model of glucose-insulin phenomena is presented. Section III, presents the backstepping approach for glucose-insulin system. Simulation results are illustrated in Section IV. Finally, the conclusion remarks are given in Section VI.

# The Nonlinear Parker Model for Type I Diabetic Patients

The glucose-insulin system of the human body used in this work is based on Sorensen model modified by Parker. This model consists of six compartments that are illustrated in Fig. 1. These compartments include brain, heart/lungs, gut, liver, kidney, and periphery. The  equations  representing glucose submodel are [7]:

| GBC=GHC-GBCqBvBC-GBC-GBTvBTTBvBC |  | (1) |
| --- | --- | --- |
| GBC=GBC-GBT1TB-ГBUvBT |  | (2) |
| GHC=GBCqB+GLCqL+GKCqK+GPCqP-GHCqH-ГRBCU1vHC | (3) |  |
| GSC=GHC-GSCqSvSC+ГmealvSC-ГSUvSC |  | (4) |
| GLC=GHCqA+GSCqS-GLCqL1vLC+ГHGPvLC-ГHGUvLC |  | (5) |
| GKC=GHC-GKCqKvKC-ГKEvKC |  | (6) |
| GPC=GHC-GPCqPvPC+GPT-GPCvPTTPGvPC |  | (7) |
| GPT=GPC-GPT1TPG-ГPUvPT |  | (8) |

The  insulin submodel as follows [7] :

| IBC=IHC-IBCQBVBC | (9) |
| --- | --- |
| IHC=IBCQB+ILCQL+IKCQK+IPCQP-IHCQH-ГIVI1VHC | (10) |
| ISC=IHC-ISCQSVSC | (11) |
| ILC=IHCQA+ISCQS-ILCQL1VLC+ГPIR-ГLCVLC | (12) |
| IKC=IHC-IKCQKVKC-ГKCVKC | (13) |
| IPC=IHC-IPCQPVPC-IPC-IPTVPTTPIVPC | (14) |
| IRT=IPC-IPT1TPI+ГSIA-ГPCVPT | (15) |

The glucagon submodel is described by  a single compartment [7]:

| N=ГPNR-NFPNCVN | (16) |
| --- | --- |

The model also has three  additional (undimensional) variables that are defined as follows [7]:

| AIHGP=1251.2088-1.138tanh1.669ILC21.43-0.8885-AIHGP | (17) |
| --- | --- |
| ANHGP=1652.7tanh0.388N-12-ANHGP | (18) |
| AIHGU=125min2tanh0.549ILC21.43-AIHGU | (19) |

Glucose Meal Liver Arterial Glucose Measurement Brain Heart/Lung Venous Blood Insulin Infusion Portal Vein Kidney Periphery Gut Disturbance

Figure 1. Compartmental representation of the glucose -insulin system in a diabetic [5], [7].

The parameters Гis are the metabolic source and sink terms in the above dynamic equations. They are defined by [7]:

| ГBU=70 | (20) |
| --- | --- |
| ГRBCU=10 | (21) |
| ГSU=20 | (22) |
| ГHGP=155AIHGP2.7tanh0.388N-ANHGP× 1.425-1.406tanh0.6199GLC101-0.4969 | (23) |
| ГHGU=20AIHGU5.6648+5.6589tanh2.4375GLC101-1.48 | (24) |
| ГKE=71(1+tanh0.011GKC-460), GKC<4600.872GKC-300 ,GKC≥460 | (25) |
| ГPGU=35GPT86.817.035+6.516tanh0.338IPT5.304-5.821 | (26) |
| ГLC=FLCIHCQA+ISCQS+ГPIR | (27) |
| ГPIR=0, no pancreatic insulin release | (28) |
| ГKC=FKCIKCQK | (29) |
| ГPC=IPT1-FPCFPC1QP-1TPIVPT | (30) |
| ГPNR=1.3102-0.61016tanh1.0571IHC15.15-0.46981× 2.9285-2.095tanh4.18GHC91.89-0.6191 | (31) |

Overall, the parker model describes the diabetic patient with nineteen dynamic equations: Eleven equations for glucose dynamics, seven for insulin and a single compartment for glucagon. Values of the parameters are shown in Table 1 [7]. The used notations of the indexes are defined as follows [7]:

- A: hepatic artery

- B: brain

- BU: brain uptake

- C: capillary space

- G: glucose

- H: heart and lungs

- HGP: hepatic glucose production

- HGU: hepatic glucose uptake

- I: insulin

- IHGP: insulin effect on HGP

- IHGU: insulin effect on HGU

- IVI: intravenous insulin infusion

- K: kidney

- KC: kidney clearance

- KE: kidney excretion

- L: liver

- LC: liver clearance

- N: glucagon

- NHGP: glucagon effect on HGP

- P:  periphery (muscle / adipose tissue)

- PC: peripheral clearance

- PGU: peripheral glucose uptake

- PIR: pancreatic insulin release

- PNC: pancreatic glucagon clearance

- PNR: pancreatic glucagon release (normalized)

- RBCU: red blood cell uptake

- S: gut (stomach / intestine)

- SIA: insulin absorption into blood stream from subcutaneous depot

- SU: gut uptake

- T: tissue space

The meal disturbance was modeled based on the method proposed by Lehmann and Deutsch [15] that is followed by the first-order filter presented in [7]:

| Гmeal=160s+160wf | (32) |
| --- | --- |

where Гmeal is the glucose absorption rate into the gut compartment  and wf represents the input waveform signal [7]

Table 1. Value of the system parameters [7].

| TB=2.1 min | qB=5.9 dLmin | vBC=3.5 dL vBT=4.5 dL |
| --- | --- | --- |
|  | qH=43.7 dLmin | vHC=13.8 dL |
|  | qS=10.1 dLmin | vSC=11.2 dL |
|  | qL=12.6 dLmin qA=2.5 dLmin | vLC=25.1 dL |
|  | qK=10.1 dLmin | vKC=6.6 dL |
| TPG=5.0 min | qP=15.1 dLmin | vPC=10.4 dL vPT=67.4 dL |
|  | QB=0.45 Lmin | VBC=0.265 L |
|  | QH=3.12Lmin | VHC=0.985 L |
|  | QS=0.72 Lmin | VSC=0.945 L |
|  | QL=0.9 Lmin QA=0.18 Lmin | VLC=1.14 L |
|  | QK=0.72 Lmin | VKC=0.505 L |
| TPI=20 min | QP=1.05 Lmin | VPC=0.735 L VPT=6.3 L |
|  | FPNC=0.910 Lmin | VN=9.93 L |

# Designing backstepping controller for Parker model

The backstepping technique is a recursive Lyapunov-based approach that was developed by Krstic, Kanellakopoulos and Kokotovic in the early 1990s [16]. The main advantage of this method is to follow a step by-step algorithm. This property brings about a systematic approach to design stabilizing controllers. This technique is utilized for a special class of nonlinear dynamical systems described by cascade structures. Consequently, one starts the design procedure at the first subsystem of this structure and in the following steps, designs new controllers to stabilize the outer subsystems. The backstepping procedure ends when the final external control input law is computed [17].

According to the structure of Parker model [7], we propose a novel backstepping approach to control insulin dosage infusion for glucose-insulin regulation in Type 1 diabetes patients. To employ the backstepping technique, the system is divided into 4 parts including two nonlinear dynamic subsystems, a nonlinear static subsystem and a linear one. The schematic of the cascade Parker system is demonstrated in Fig. 2. The goal is to design insulin injection controller such that the plasma glucose regulates to its desirable value (≅82 mgdL). Therefore, each virtual control input vector of its corresponding subsystem should be designed. In the following, each subsystem and procedure of designing its virtual stabilizing control inputs are discussed.

Insulin Infusion Subsystem I Nonlinear subsystem Subsystem II Nonlinear static subsystem AIHGU, IPT,N Blood IHC, ILC, IPT Subsystem III Nonlinear subsystem Subsystem IV Linear subsystem Glucose ΓHGP,ΓHGU ΓPGU Figure 2. Cascade Parker system representation.

## Subsystem I

The subsystem I comprises the dynamic equations (1)-(8). Consider ΓHGP, ΓHGU and ГPGU virtual control inputs, and ГBU, ГRBCU and ГSU as disturbance inputs. Therefore, the nonlinear dynamics (1)-(8) can be rewritten in the form of  (33) in which the linear part is separated from the nonlinear one. The nonlinear part consists of the term ΓKE. The output of this subsystem is the plasma glucose concentration which is a linear function of the GHc (i.e. BG=0.925GHc). Finally, based on LMI techniques, a robust controller with  H∞ attenuation level will be designed such that the desire performance is achieved.

| x=Ax+fx+Bu+Ew y=Cx | (33) |
| --- | --- |

Lemma 1 [18]: Consider the nonlinear system  (33). Assume that the nonlinear term fx satisfies f0=0 and the Lipchitz condition as follows:

| fx<Lx | (34) |
| --- | --- |

where L is a non-negative scalar. The system  (33) is stabilizable with state feedback controller u=F1xand H∞ performance criterion is guaranteed, if the following LMIs are satisfied.

| X>0 | (35) |
| --- | --- |

| XAT+STBT+AX+BS+XL+LXEXCTET-γI0CX0-γI<0 | (36) |
| --- | --- |

where the feedback matrix is calculated by F1=SX-1.

By employing Lemma 1 on the subsystem I, the virtual control input vector u=ΓHGPΓHGUΓPGUT will be obtained.

## Subsystem II

The subsystem II comprises of equations (23), (24) and (26). These equations are static nonlinear transforms. The virtual control inputs of this part are AIHGU, IPT and N, and the outputs are ГHGP, ГHGU and ГPGU. Hence, in this subsystem, the objective is to calculate the virtual control inputs such that outputs track the desired virtual inputs of subsystem I. Based on (23), (24) and (26), the following virtual control inputs of the subsystem II are obtained.

| Nde=10.388arctanh12.7ΓHGP155AIHGP1.425-1.406tanh0.6199GLC101-0.4969+ANHGP | (37) |
| --- | --- |
| AIHGUde= ΓHGU205.6648+5.6589tanh2.4375GLC101-1.48 | (38) |  |
| IPdeT=5.30410.33827arctanh86.8135GPTΓPGU-7.0356.51623 | (39) |

## Subsystem III

The subsystem III includes dynamics N, AIHGU, AIHGP and ANHGP ((16),(19),(17) and (18)). The objective is to design the virtual control inputs IHC and ILC such that N and AIHGU track the desired values in the subsystem II, respectively. However due to the lack of space, only the design procedure of  ILC is briefly discussed.

To satisfy the Lyapunov stability conditions [19], V and V are considered as follows:

| V=12AIHGU-AIHGUde2 | (40) |
| --- | --- |
| V=-KAIHGU-AIHGUde2<0 (K≥0) | (41) |

By comparing the time derivative of (40) with (41), ILdeC is obtained as:

| ILdeC=21.430.549tanh-1252AIHGUde+AIHGU25-KAIHGU-AIHGUde | (42) |
| --- | --- |

Furthermore, the dynamics (17) and (18) are of the form (x=-ax+f(x,u)) with positive scalar a and bounded f. Therefore, based on the input-to-state stability theorem [20], the states AIHGP and ANHGP will be bounded.

## Subsystem IV

The subsystem IV is a linear system including insulin dynamics ((9)-(15)). The control input is the intravenous insulin infusion. The purpose is to design the controller input ГIVI by the means of LMI techniques in a way such that the outputs IHC, ILC and IPT track the desired set points in the prior subsystems.

To obtain a stabilizing feedback controller u=F2x for subsystem IV, one can use Lemma 1 with L=0.

# Simulation Results

In this subsection, Parker system is considered to design backstepping controller. In order to apply Lemma 1, one has to choose the constant L=0.9 such that the constraint  ΓKEvKC<L.GKC is held. Therefore, coefficients of state feedback controller are obtained in subsystem II, as follow:

F1=106-0.00120-4.2391-0.00010.001204.23910.00010.000100.05840

-0.0016-0.0018-0.004000.00160.00180.004000.00010.00020.00040

Also in subsystem IV, coefficients of state feedback controller are computed via LMI approach, as:

F2=0.4133-0.59390.08560.1274

0.42360.4690-0.0320

The meal disturbance, continuous insulin infusion dosage and blood glucose concentration are illustrated in Fig.3, Fig.4 and Fig.5, respectively. As it can be seen in these figures, the proposed controllers can effectively handle the meal glucose disturbances. Also, it can cause that the blood glucose to converge to its reference.

# Conclusion

In this paper, a backstepping approach was applied to the nonlinear 19th-order Parker system for regulation of glucose in type 1 diabetes patients. For this purpose, the overall Parker system was divided into 4 parts including two nonlinear dynamic subsystems, a nonlinear static subsystem and a linear one. Based on the Lyapunov stability theorem, the virtual control inputs were designed for each subsystem. Finally, the system input “insulin infusion” was derived. Furthermore, input-to-state theorem was considered to guarantee the boundedness of the Parker system states.   Simulation results confirmed that the proposed backstepping controller can efficiently (I) alleviate the effect of meal disturbance, (II) maintain the plasma glucose concentration in the normal region, (III) force the system output converge to its reference.

Figure 3. gastric emptying of glucose (meal disturbance)

Figure 4. Blood glucose concentration

Figure 5. Insulin Infusion rate for Lehmann and Deutsch meal disturbance

##### References

[1]	S. S. Hacısalihzade, Biomedical Applications of Control Engineering, vol. 441. Berlin, Heidelberg: Springer Berlin Heidelberg, 2013.

[2]	P. Dua, F. J. Doyle, and E. N. Pistikopoulos, “Model-based blood glucose control for type 1 diabetes via parametric programming,” IEEE Trans. Biomed. Eng., vol. 53, no. 8, pp. 1478–1491, Aug. 2006.

[3]	A. Hariri and L. Y. Wang, “Identification and low-complexity regime-switching insulin control of type I diabetic patients,” J. Biomed. Sci. Eng., vol. 04, no. 04, pp. 297–314, 2011.

[4]	R. N. Bergman, L. S. Phillips, and C. Cobelli, “Physiologic evaluation of factors controlling glucose tolerance in man: measurement of insulin sensitivity and beta-cell glucose sensitivity from the response to intravenous glucose.,” J. Clin. Invest., vol. 68, no. 6, pp. 1456–1467, Dec. 1981.

[5]	J. T. Sorenson, “A physiologic model of glucose metabolism in man and its use to design and assess improved insulin therapies for diabetes,” PhD Diss Mass. Inst. Technol., 1985.

[6]	R. Hovorka, F. Shojaee-Moradie, P. V. Carroll, L. J. Chassin, I. J. Gowrie, N. C. Jackson, R. S. Tudor, A. M. Umpleby, and R. H. Jones, “Partitioning glucose distribution/transport, disposal, and endogenous production during IVGTT,” Am. J. Physiol. - Endocrinol. Metab., vol. 282, no. 5, pp. E992–E1007, May 2002.

[7]	R. S. Parker, F. J. Doyle, J. H. Ward, and N. A. Peppas, “RobustH∞ glucose control in diabetes using a physiological model,” AIChE J., vol. 46, no. 12, pp. 2537–2549, Dec. 2000.

[8]	K. Lunze, T. Singh, M. Walter, M. D. Brendel, and S. Leonhardt, “Blood glucose control algorithms for type 1 diabetic patients: A methodological review,” Biomed. Signal Process. Control, vol. 8, no. 2, pp. 107–119, Mar. 2013.

[9]	A. Hariri, “Observer-Based State Feedback for Enhanced Insulin Control of Type ‘I’ Diabetic Patients,” Open Biomed. Eng. Journall, vol. 5, no. 1, pp. 98–109, Dec. 2011.

[10]	J. Chen, K. Cao, Y. Sun, Y. Xiao, and X. (Kevin) Su, “Continuous Drug Infusion for Diabetes Therapy: A Closed-Loop Control System Design,” EURASIP J. Wirel. Commun. Netw., vol. 2008, pp. 1–10, 2008.

[11]	D. Radomski, M. Lawrynczuk, P. Marusak, and P. Tatjewski, “Modeling of glucose concentration dynamics for predictive control of insulin administration,” Biocybernet Biomed Eng, vol. 30, no. 1, pp. 41–53, 2010.

[12]	R. Hovorka, V. Canonico, L. J. Chassin, U. Haueter, M. Massi-Benedetti, M. O. Federici, T. R. Pieber, H. C. Schaller, L. Schaupp, T. Vering, and M. E. Wilinska, “Nonlinear model predictive control of glucose concentration in subjects with type 1 diabetes,” Physiol. Meas., vol. 25, no. 4, pp. 905–920, Aug. 2004.

[13]	C. Vito, “LPV Identification of the Glucose-Insulin Dynamics in Type I Diabetes,” 2012, pp. 559–564.

[14]	S. Kamath, “Control of Blood Glucose Concentration in Type I Diabetic Patients,” 2011, pp. 1–6.

[15]	E. D. Lehmann and T. Deutsch, “A physiological model of glucose-insulin interaction in type 1 diabetes mellitus,” J. Biomed. Eng., vol. 14, no. 3, pp. 235–242, May 1992.

[16]	M. Krstić, I. Kanellakopoulos, and P. V. Kokotović, Nonlinear and adaptive control design. New York: Wiley, 1995.

[17]	Zhou, Adaptive Backstepping Control of Uncertain Systems, vol. 372. Berlin, Heidelberg: Springer Berlin Heidelberg, 2008.

[18]	K. Tanaka and H. O. Wang, Fuzzy control systems design and analysis: a linear matrix inequality approach. New York: Wiley, 2001.

[19]	H. K. Khalil, Nonlinear systems, 3rd ed. Upper Saddle River, N.J: Prentice Hall, 2002.

[20]	E. D. Sontag and Y. Wang, “On characterizations of the input-to-state stability property,” Syst. Control Lett., vol. 24, no. 5, pp. 351–359, Apr. 1995.