Structure Improvement of Active-Clamp to Achieve

High Step-Down Conversion

Dariush Cheshmdehmam, Ehsan Adib, Hosein Farzanehfard.

Department of Electrical and Computer Engineering

Isfahan University of Technology

Isfahan, Iran

d.cheshmdehmam@ec.iut.ac.ir, e.adib@cc.iut.ac.ir, hosein@cc.iut.ac.ir

Abstract— In this paper a new high step down non isolated soft switching buck converter is proposed. In order to achieve high step down conversion ratio, coupled inductor are charged in series and discharged in parallel in the output capacitor. In the proposed converter the clamp capacitor is in series in the power path which further reduces the voltage gain while for providing soft switching for both switches. Therefore, both switches operate under zero voltage switching condition. The proposed converter is analyzed and experimental results are presented.

Keywords—Series-capacitor, coupled inductor, high step-down, duty cycle, zero voltage switching, resonant inductor, active-clamp.

# Introduction

High step down converters are vastly used in battery charger application, VRM modules, Power Factor Correction (PFC) circuits and …. To achieve a high step-down conversion ratio in conventional buck converter, operating with very low duty cycle ratio is necessary which results in converter instability and low power conversion efficiency. In coupled-inductor buck converter [1] high step-down conversion can be achieved with operating duty cycles higher than conventional buck converter but this converter operates under hard-switching condition and voltage spikes across converter switch is its main problem. This problem is solved in [1]-[2] by passive clamp circuit however the converter is hard switched too. In [3]-[4], active-clamp coupled inductor buck converter is presented and the voltage spikes due to the leakage inductor energy is absorbed and recovered to the output and both of the switches operate under soft-switching condition as shown in Fig. 1. Converter in [5], can absorb voltage-spikes and also high step-down ratio is achieved with a large operating duty cycle and all of the switches can operate under soft-switching condition. However, this converter applies 3 switches.

In the proposed converter, the idea of coupled-inductors [6], series-capacitor in power path [7] and [5] and active-clamp techniques [3]-[4] are combined. There are 4 energy storage elements in power path and thus high step-down conversion is achieved. As a result, the proposed converter can operate with higher duty cycle in comparison to the converters mentioned above. In addition, both of the switches in the proposed converter operate under ZVS condition. Leakage and resonant inductors energy during switch off time are absorbed by series capacitor and converter output, therefore voltage spike across the switches is suppressed.

Fig. 1. Topology of ICBC with active-clamp circuits provide in [4]

Fig. 2. A module of ICBC topology with active-clamp circuits in [4]

One phase of  active clamp structure introduced in [4] is shown in Fig.2. In this converter, clamp capacitor is not in the main power path. In the proposed converter as shown in Fig.3, the clamp capacitor is in the power path which can further reduce the converter voltage gain. In addition, both of main and auxiliary switches can be switched under Zero-Voltage ZVS and therefore switching losses can be significantly reduced and significantly improve conversion efficiency. In the proposed converter, coupled inductors technique is also applied to further reduce the voltage gain. Resonant inductor used in the structure of the converter, can provide soft switching condition for converter switches and also can reduce losses caused by reverse recovery of output diodes. It should be noted that resonant inductor energy, is absorbed by series clamp capacitor in the path of power transmission.

The clamp circuit applied can provide three advantages:

- Providing Zero-Voltage condition for both of the converter switches.

- Suppressing voltage spike on the switches by recovering resonant inductor energy in the clamp capacitors C, then recovering this energy to output through the primary windings and transferring power to the secondary side.

- reducing reverse recovery losses of output diodes by zero current turn-off.

Fig. 3 shows the proposed converter, which contains two switches Q1 and Q2, one series capacitor C in power path transmission, output capacitor Cout, and coupled inductor composed of the primary winding N and two the secondary winding N1 and N2. The equivalent circuit of the proposed converter is shown in Fig. 4. The coupled inductor is modeled by an ideal transformer with primary winding N and two secondary winding N1 and N2, a magnetizing inductor Lm connected in parallel with N, also leakage inductance of coupled Lr.

Fig. 3. Proposed high step-down converter.

Fig. 4. Equivalent circuit model of the proposed converter.

# basic operating principles

To simplify the analysis of circuit, some assumptions are considered as follows.

- All semiconductor and passive elements are ideal;

- All capacitors include C and Cout are large and are considered as constant voltage source.

- Inductors L1 and L2 are equal and are coupled with the main inductor L.

There are seven operating modes in each switching period. Theoretical waveforms and equivalent circuit of each operating mode are shown in Fig. 5 and Fig. 6.

Fig. 5. Key waveforms of the proposed converter.

Mode1 (t0 < t ≤ t1):

As shown in Fig .6(a), this mode begins when main switch Q1 is tuned-on under ZVS condition. By tuning the main switch on, current of main inductor L and resonant inductor Lr increase to zero linearly from a negative value and then become positive. Since main switch current is not equal to output inductors current, as a result output diodes are still on and the output inductors current are charging the output capacitor.

Mode2 (t1 < t ≤ t2):

As shown in Fig .6(b), when primary winding current become equal to the secondary windings current, diodes D1 and D2 turned-off under ZCS condition. Inductors L, L1 and L2 that are coupled together in this mode are series together and are charged. Also series capacitor voltage increases linearly by constant current of the series inductors. This mode will end when main switch is turned off under ZVS due to snubber capacitor.

Mode3 (t2 < t ≤ t3):

As shown in Fig .6(c), by turning the main switch off, parallel capacitor with the main switch is charged and parallel capacitor with the auxiliary switch is discharged by leakage inductance energy. In this mode, D1 and D2 are off.

Mode4 (t3 < t ≤ t4):

As shown in Fig .6(d), when the parallel capacitor of main switch is charged, diodes D1 and D2 turn on and L1 and L2 charge the output capacitor in parallel. On the other hand, the leakage inductor energy is absorbed by output. After full discharge parallel capacitor of auxiliary switch in C, intrinsic anti-parallel diode of this switch is turn-on and energy of leakage inductor during this mode is absorbed by series capacitor C in path of power transmission. During this mode, Q2 can be turned on under ZVS condition.

Mode5 (t4 < t ≤ t5):

As shown in Fig .6(e), by turning Q2 on, when leakage inductor current become zero, then its current increases in the other direction and the energy stored in C is transferred to the output through coupled inductors.

Mode6 (t5 < t ≤ t6):

As shown in Fig .6(f), auxiliary switch Q2 is turned off under ZVS condition due to parallel snubber capacitor. Then leakage inductor current, discharges the parallel capacitor of main switch and charges parallel capacitor of auxiliary switch. In this mode D1 and D2 are still on. To achieved ZVS condition of the main switch Q1, energy stored in leakage inductor must satisfy the following inequality.

Mode7 (t6 < t ≤ t0):

As shown in Fig .6(g), when parallel capacitor of Q1 is discharged, intrinsic anti-parallel diode of main switch is turned-on. By applying the pulse to the gate-source of main switch, during this mode, Q1 can be turned-on under ZVS condition.

(a)

(b)

(c)

(d)

(e)

(f)

(g)

Fig. 6. Equivalent circuit modes of proposed converter over one switching cycle of period: (a) mode 1; (b) mode 2; (c) mode 3; (d) mode 4; (e) mode 5; (f) mode 6; (g) mode 7.

##### Voltage Gain of the Proposed Converter

To simplify the analysis, the modes associated with charging and discharging of snubber capacitors is ignored. Voltage gain of the proposed converter can be obtained from volt-second balance of magnetizing inductance of coupled inductors:

From mode 5, the following equation can be obtained to be:

By applying the voltage-second balance principle to Lm over one switching period, the following equation can be obtained:

To simplify equation it is considered that:

Next, based on (3), (4) and (5), the corresponding voltage gain can be expressed to be:

From (6), it can be seen that the voltage gain of the proposed converter can be adjusted by duty cycle and turns ratio.

Fig. 7 shows the curves of voltage gain versus duty cycle of the proposed converter, considering different values of turn ratio.

Fig. 7. Plot of voltage gain versus duty cycle for different values of turns ratio n.

##### Performance Comparison

To represent the performance of the proposed converter, a comparison is performed between the proposed converter and converters introduced in [4], [5]  and [6] in Table 1.

Table 1. Comparison of 3 step-down converters with the proposed converter.

| Converters Components | [4] | [5] | [6] | Proposed Converter |
| --- | --- | --- | --- | --- |
| Voltage Gain |  |  |  |  |
| Voltage Spike of Switch | NO | NO | NO | NO |
| Number of inductor | 2 | 2 | 3 | 3 |
| Number of Switch | 2 | 3 | 1 | 2 |
| Number of capacitor | 2 | 2 | 2 | 2 |

Fig. 8 shows the curves of voltage gain versus duty cycle for turns ratio n = 2/5, input voltage Vin = 150Vdc and output voltage Vo= 12Vdc compared with 3 converter in [6], [5] and [4]. From this figure, it can be seen that the proposed converter, can achieve high step-down conversion ratio in comparison to other converters. According to this figure, for duty cycle less than 0.4, due to parabolic voltage gain of [6], this converter can operate with higher duty cycle but for duty cycle higher than 0.4, the proposed converter can provide better step down characteristics.

IQ1

VQ1

Fig. 8. Comparison of voltage gain versus duty cycle between 3 step-down converter and the proposed converter.

Fig. 9 shows efficiency of the proposed converter. It can be seen that the maximum efficiency is 92% under Pout=120W.

Fig. 9. Efficiency versus Output Power of the Proposed Converter

# simulation result:

To verify the performance of the proposed converter, the simulation results are shown in this section. The electric specifications and circuit parameters are: input voltage Vin = 150 V, output voltage VO = 12 V, switching frequency fs = 100 kHz, output power PO = 120 W, inductor L = 137 μH, coupled-inductor L1 = L2 = 22 μH, resonant-inductor Lr = 9.25 μH capacitor C = 2.2 μF and Cout = 2200 μF. Duty ratio of main switch is equal to 0.4 and duty cycle of auxiliary switch is equal to 0.5. Fig 10 and 11 show the voltage stress across and current through the switches. From these figures, it can be observed that both switches of proposed converter operate under ZVS condition. Fig 12 and 13 show the voltage and current of L and L1 respectively.

Fig. 10. Waveforms of voltage stress and current through of Q1.

IQ2

VQ2

Fig. 11. Waveforms of voltage stress and current through of Q2.

IL

VL

Fig. 12. Waveforms of voltage and current of L.

IL1

VL1

Fig. 13. Waveforms of voltage and current of L1.

- V.conclution

In the proposed converter high step down converter using coupled inductors, the clamp capacitor is also in series in the power path which further reduce the voltage gain. Both of main and auxiliary switches are switched under ZVS, as a result switching losses can be significantly reduced and efficiency of converter is significantly improved. The proposed converter is analyzed and operating modes are discussed. Simulation results are presented to verify the theoretical analysis.

##### References

[1]	K. Yao, M. Ye, M. Xu, and F. C. Lee, “Tapped-Inductor Buck Converter for High-Step-Down DC–DC Conversion,” IEEE Trans. Power Electron., vol. 20, no. 4, pp. 775–780, Jul. 2005.

[2]	C. Tsai and C. Shen, “Interleaved soft-switching buck converter with coupled inductors,” 2008 IEEE Int. Conf. Sustain. Energy Technol., pp. 877–882, 2008.

[3]	C.-T. Tsai, C.-L. Shen, J.-C. Su, and Y.-C. Kuo, “Analysis and implementation of an interleaved ZVS buck converter with coupled-inductors,” 2011 6th IEEE Conf. Ind. Electron. Appl., pp. 1392–1397, Jun. 2011.

[4]	C.-T. Tsai and C.-L. Shen, “Interleaved soft-switching coupled-buck converter with active-clamp circuits,” 2009 Int. Conf. Power Electron. Drive Syst., pp. 1113–1118, Nov. 2009.

[5]	K. I. Hwu, W. Z. Jiang, and Y. T. Yau, “Ultra High Step-Down Converter,” IEEE Trans. Power Electron., vol. 8993, no. c, pp. 1–1, 2014.

[6]	L. Yang, E. Chang, C. Lin, and A. V. Gain, “Study of a DC-DC Converter with Large Step- Down Voltage Conversion,” no. 2, pp. 465–468, 2013.

[7]	I.-O. Lee, S.-Y. Cho, and G.-W. Moon, “Interleaved Buck Converter Having Low Switching Losses and Improved Step-Down Conversion Ratio,” IEEE Trans. Power Electron., vol. 27, no. 8, pp. 3664–3675, Aug. 2012.